3GPP TSG-RAN WG4 Meeting #115	R4-2508278

Malta, Malta, 19th - 23rd May, 2025

Agenda item:			7.27.1

Source:	Huawei

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part2

Document for:	Approval

- Topic #1: Adaptation of common signal/channel transmission

Sub-topic 1-1 SSB adaptation – General

Issue 1-1-1: Processing time for SSB adaption

Agreement

RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation.

- FR1: 2ms FR2: 3ms

- FFS L3 transition period measurement

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-2: L1 requirements after transition  - DRX

Agreement:

- For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

Agreement:

- No spec. impact is identified for releasing the measurement results.

- It’s up to UE implementation.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell

Agreement

RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement.

- The cell identification and measurement period requirements apply based on the longer period before or after the transition.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell

Agreement

- Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

Issue 1-3-4: Colliding with MG

- Proposals

  - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

Agreement:

- For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: R18 Scell activation with L3 reporting

  - Option 2: multiple SCells activation scenario with SSB adaptation

- Topic #2: OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Agreement:

- For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell

- Define test case with longer TSI-NR with reasonable value in performance part.

- Reference

R4-2505571	Topic summary for [115][222] Netw_Energy_NR_enh_Part2