# 3GPP TSG-RAN WG4 Meeting #115	R4-2508406

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on requirements of Link Recovery Procedures for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on CR work split, the requirements of LR for RedCap UEs with NTN to be introduced in R19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce RRM requirements of LR for RedCap UEs with NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements of LR for RedCap UEs with NTN in R19 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) 8.5X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 8.5X	Link Recovery Procedures for RedCap UE with Satellite Access

### 8.5X.1	Introduction

The RedCap UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [3] in order to detect beam failure on PCell and the UE is configured with only PCell, which is served by satellite access node (SAN).

The RS resource configurations in the set q0 on PCell can be periodic CSI-RS resources and/or SSBs. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 8.5X.2 and 8.5X.3 if UE does not have set q0.

On each RS resource configuration in the set q0, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR_RedCap for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR_RedCap is defined as the level at which the downlink radio level link of a given resource configuration on set q0 cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5X.2.1-1. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5X.3.1-1.

Upon request the UE shall deliver configuration indexes from the set q1as specified in TS 38.213 [3], to higher layers, and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR_RedCap, which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR_RedCap threshold to the L1-RSRP measurement obtained from an SSB. The UE applies the Qin_LR_RedCap threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set q1 can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources. UE is not required to perform candidate beam detection outside the active DL BWP.

### 8.5X.2	Requirements for SSB based beam failure detection for RedCap UEs with satellite access

#### 8.5X.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set q0 configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5X.2.2.

Table 8.5X.2.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16 for 1 Rx UE; 8 for 2 Rx UE |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Bandwidth (PRBs) | 48 for 1 Rx UE; 24 for 2 Rx UE |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| NOTE 1: SCS=60 kHz is not applicable for FR1 |  |  |

#### 8.5X.2.2	Minimum requirement

The requirements in clause 8.5C.2.2 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 8.5C.2.2 shall apply except:

-	TEvaluate_BFD_SSB_RedCap as defined in table 8.5X.2.2-1 for FR1.

Table 8.5X.2.2-1: Evaluation period TEvaluate_BFD_SSB_RedCap for 1 Rx Redcap

| Configuration | TEvaluate_BFD_SSB_RedCap (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(10 P) TSSB) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(15 P) Max(TDRX,TSSB)) |
| DRX cycle > 320 ms | Ceil(10 P) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. |  |

### 8.5X.3	Requirements for CSI-RS based beam failure detection for RedCap UEs with satellite access

#### 8.5X.3.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set q0 of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set q0for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5X.3.2. UE is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the UE active BWP.

Table 8.5X.3.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16 for 1 Rx UE; 8 for 2 Rx UE |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0 dB |
| Bandwidth (PRBs) | 48 for 1 Rx UE; 24 for 2 Rx UE |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| NOTE 1: SCS=60 kHz is not applicable for FR1 |  |  |

#### 8.5X.3.2	Minimum requirement

The requirements in clause 8.5C.3.2 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 8.5C.3.2 shall apply except:

-	TEvaluate_BFD_SSB_RedCap as defined in table 8.5X.3.2-1 for FR1.

Table 8.5X.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS_RedCap for 1 Rx RedCap

| Configuration | TEvaluate_BFD_CSI-RS_RedCap (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(2MBFD P PBFD) TCSI-RS) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(21.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320 ms | Ceil(2MBFD P PBFD) TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q0. TDRX is the DRX cycle length. |  |

#### 8.5X.3.3	Measurement restrictions for CSI-RS beam failure detection

The requirements in clause 8.5C.3.3 shall apply.

### 8.5X.4	Minimum requirement for L1 indication for RedCap UEs with satellite access

The requirements in clause 8.5C.4 shall apply.

In addition, for HD-FDD UE, the conditions and requirements for TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period.

### 8.5X.5	Requirements for SSB based candidate beam detection for RedCap UEs with satellite access

The requirements in clause 8.5C.5 shall apply when RedCap UE is capable of 2 Rx and 1 Rx RedCap UE.

### 8.5X.6	Requirements for CSI-RS based candidate beam detection for RedCap UEs with satellite access

The requirements in clause 8.5C.6 shall apply when RedCap UE is capable of 2 Rx and 1 Rx RedCap UE.

### 8.5X.7	Scheduling availability of UE during beam failure detection for RedCap UEs with satellite access

The requirements in clause 8.5C.7 shall apply.

In addition, for HD-FDD UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during beam failure detection, except when UE is indicated that UL transmission is prioritized over BFD evaluation in case of collision between CSI-RS resouces to be meaured and UL transmission.

### 8.5X.8	Scheduling availability of UE during candidate beam detection for RedCap UEs with satellite access

The requirements in clause 8.5C.8 shall apply.

In addition, for HD-FDD UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during candidate beam detection, expect when UE is indicated that UL transmission is prioritized over CBD evaluation in case of collision between CSI-RS resouces to be meaured and UL transmission.

### 8.5X.9	Minimum requirement at transitions for beam failure detection for RedCap UEs with satellite access

The requirements in clause 8.5C.9 shall apply.

<End of Change 1>