3GPP TSG-RAN4 Meeting #115	R4-2508407

# Malta, MT, 19th - 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRC Connection Mobility Control for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the work split in WF R4-2504912, RRC Connection Mobility Control for RedCap UEs with NTN is needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM requirements for RRC Connection Mobility Control for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------Start Change #1-------------------------

6.2X	RRC Connection Mobility Control for RedCap UE Satellite Access

6.2X.1	SA: RRC Re-establishment for RedCap UE Satellite Access

6.2X.1.1	Introduction

This clause contains requirements on the RedCap UE regarding RRC connection re-establishment procedure. RRC connection re-establishment is initiated when a UE in RRC_CONNECTED state on the carrier loses RRC connection due to any of failure cases, including radio link failure, handover failure, and RRC connection reconfiguration failure. The RRC connection re-establishment procedure is specified in clause 5.3.7 of TS 38.331 [2].

A neighbour cell for RRC re-establishment procedures for a RedCap UE is defined as an intra-frequency cell if the centre frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the centre frequency and SCS of the reference SSB of the neighbour cell; else it is considered as inter-frequency cell, where:

-	The reference SSB of the target cell is the CD-SSB of the target cell.

-	The reference SSB of the serving cell is the CD-SSB of the serving cell.

The requirements in this clause are applicable for RRC connection re-establishment to NR cell, which is served by satellite access node (SAN).

6.2X.1.2	Requirements

The requirements in clause 6.2C.1.2 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2C.1.2 shall apply except that:

-	Tidentify_intra_NR as specified in table 6.2.1X.2-1.

-	Tidentify_inter_NR, i as specified in table 6.2.1X.2-2.

Table 6.2.1X.2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC) | MAX (800 ms, [11] x TSMTC) |
| < -8 | FR1 | N/A | 800Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 6.2.1X.2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 7 x TSMTC, i) | MAX (800 ms, [14] x TSMTC, i) |
| < -8 | FR1 | N/A | 800Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

6.2X.2	Random access for RedCap UE satellite access

6.2X.2.1	Introduction

This clause contains requirements on the RedCap UE regarding random access procedure. The random access procedure is initiated to establish uplink time synchronization for a UE which either has not acquired or has lost its uplink synchronization, or to convey UE’s request Other SI, or for beam failure recovery. The random access is specified in clause 8 of TS 38.213 [3] and the control of the RACH transmission is specified in clause 5.1 of TS 38.321 [7]. Two types of procedure are defined for the random access, the 4-step RA type, and the 2-step RA type [7]. The decision on which type of procedure to adopt is as described in clause 5.1.1 of TS 38.321 [7].

The requirements in clause 6.2C.2.2 and 6.2C.2.3 shall apply when RedCap UE is capable of 2 Rx.

The 1 Rx RedCap UE for performing the random access procedure applies:

-	rsrp-ThresholdSSB as the signaled value of rsrp-ThresholdSSB [2] + 1 dB.

-	msgA-RSRP-ThresholdSSB as the signaled value of msgA-RSRP-ThresholdSSB [2] + 1 dB.

-	msgA-RSRP-Threshold as the signaled value of msgA-RSRP-Threshold [2] + 1 dB.

-	rsrp-ThresholdMsg3 as the signaled value of rsrp-ThresholdMsg3 [2] - 1 dB.

6.2X.3	SA: RRC Connection Release with Redirection for RedCap UE Satellite Access

6.2X.3.1	Introduction

This clause contains requirements on the RedCap UE regarding RRC connection release with redirection procedure. RRC connection release with redirection is initiated by the RRCRelease message with redirection to NR from NR specified in TS 38.331 [2]. The RRC connection release with redirection procedure is specified in clause 5.3.8 of TS 38.331 [2].

6.2X.3.2	Requirements

6.2X.3.2.1	RRC connection release with redirection to NR

The requirements in clause 6.2C.3.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2C.3.2.1 shall apply except that:

-	Tidentify-NR as specified in table 6.2.3X.2.1-1.

Table 6.2.3X.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, [12] x Trs) |
| NOTE: If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

FFS on: The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

----------------------------------------End Change #1-------------------------