3GPP TSG-RAN WG4 Meeting #115															 R4-2508412

Malta, 19th - 23th May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on timing requirements for NTN Redcap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new timing requirements for NTN Redcap |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the following timing requirements for NTN Redcap - 7.1X UE transmit timing for RedCap UEs with NTN - 7.2X UE timer accuracy for RedCap UEs with NTN - 7.3X Timing advance for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No timing requirements for NTN Redcap |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1X, 7.2X, 7.3X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## 7.1X	UE transmit timing for Redcap for Satellite Access

### 7.1X.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state. The uplink frame transmission takes place NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

### 7.1X.2	Requirements

The requirements in clause 7.1C.2 for FR1-NTN shall apply:

-     when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

-     when it is the transmission for PUSCH on CG resources for SDT in RRC_INACTIVE.

#### 7.1X.2.1	Gradual timing adjustment

The requirements in clause 7.1C.2.1 for FR1-NTN shall apply

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## 7.2X	UE timer accuracy for Redcap for Satellite Access

### 7.2X.1	Introduction

UE timers are used in different protocol entities to control the UE behaviour.

### 7.2X.2	Requirements

The requirements in clause 7.2C.2 for FR1-NTN shall apply

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

## 7.3X	Timing advance for Redcap for Satellite Access

### 7.3X.1	Introduction

The timing advance is initiated by UE configured with only PCell served by SAN, upon initiating a validity timer for NTA,adjcommon  and  NTA,adjUE. The timing advance can be adjusted with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

### 7.3X.2	Requirements

#### 7.3X.2.1	Timing Advance adjustment delay

The requirements in clause 7.3C.2.1 for FR1-NTN shall apply

#### 7.3X.2.2	Timing Advance adjustment accuracy

The requirements in clause 7.2C.2.2 for FR1-NTN shall apply

## <<< END OF CHANGES 3>>>