3GPP TSG-RAN WG4 Meeting #115	R4-2508418

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on 9.5X L1-RSRP measurements for Reporting for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add L1-RSRP measurements for Reporting for RedCap UEs with NTN based on progress in NR_NTN_Ph3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add L1-RSRP measurements for Reporting for RedCap UEs with NTN based on progress in NR_NTN_Ph3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for RedCap UEs with NTN are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NEW 9.5X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change 1>

## 9.5X	L1-RSRP measurements for Reporting for RedCap UEs with satellite access

### 9.5X.1	Introduction

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for PCell, on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

### 9.5X.2	Requirements applicability

The requirements in clause 9.5X apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

For 1 Rx RedCap UE with NTN:

-	L1-RSRP related side conditions given in clauses [10.X.X.X] for FR1, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to [Annex B.2.19.2] for a corresponding band.

For 2 Rx RedCap UE with NTN:

-	L1-RSRP related side conditions given in clauses [10.1.19C.2] for FR1, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to [Annex B.2.19.2] for a corresponding band.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

For 1 Rx RedCap UE with NTN:

-	L1-RSRP related side conditions given in clauses [10.X.X.X] for FR1, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to [Annex B.2.19.2] for a corresponding band.

For 2 Rx RedCap UE with NTN:

-	L1-RSRP related side conditions given in clauses [10.1.19C.2] for FR1, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to [Annex B.2.19.2] for a corresponding band.

A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

### 9.5X.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports only for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1 dB step size according to clause 10.1.19C for FR1 for 2 Rx RedCap UE with NTN, and clause [10.X.X.X] for FR1 for 1 Rx RedCap UE with NTN, if nrofReportedRS is configured to one. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause 10.1.19C for FR1 for 2 Rx RedCap UE with NTN, and clause [10.X.X.X] for FR1 for 1 Rx RedCap UE with NTN. The differential L1-RSRP is quantized to a 4-bit value with 2 dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in clause 10.1.6.

#### 9.5X.3.1	Periodic Reporting

For 1 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clause [10.X.X.X] for FR1.

For 2 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clause [10.1.19C].

The UE shall only send periodic L1-RSRP measurement reports for an active BWP.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.5X.3.2	Semi-Persistent Reporting

For 1 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clause [10.X.X.X]. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

For 2 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clause [10.1.19C]. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.5X.3.3	Aperiodic Reporting

For 1 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clause [10.X.X.X].

For 2 Rx RedCap UE with NTN, reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in claus [10.1.19C].

The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

### 9.5X.4	L1-RSRP measurement requirements

#### 9.5X.4.1	SSB based L1-RSRP Reporting

The requirements in clause 9.5C.4.1 shall apply for both 1Rx and 2Rx RedCap UE.

#### 9.5X.4.2	CSI-RS based L1-RSRP Reporting

When Network indicates [UL overriding DL for collision handling],

- the requirements in clause 9.5C.4.2 shall apply for both 1Rx and 2Rx RedCap UE, provided that no collision between the CSI-RS resources for L1-RSRP measurements and UL transmission;

Otherwise,

- the requirements in clause 9.5C.4.2 shall apply for both 1Rx and 2Rx RedCap UE, as CSI-RS based L1-RSRP measurement is prioritized over UL transmission.

### 9.5X.5	Measurement restriction for L1-RSRP measurement

The requirements in clause 9.5C.5 shall apply for both 1Rx and 2Rx RedCap UE, when the L1-RSRP measurement requirements in 9.5X.4 are applicable.

### 9.5X.6	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions when the UE is performing L1-RSRP measurement are described in the following clauses. For UL, the scheduling restriction applies to UL symbols that fully or partially overlap with the restricted symbols as defined below.

#### 9.5X.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the L1-RSRP measurement period, when the L1-RSRP measurement is prioritized over UL transmission.

#### 9.5X.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14], the requirements in clause 9.5X.6.1 shall apply.

For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

## <End of Change 1>