# 3GPP TSG-RAN WG4 Meeting #115	R4-2508419

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.23.2

Source: 	OPPO, Nokia

Title: 	TP on RAN4 aspects for TR 38.744

Document for:	Approval

- Introduction

In this contribution, we provide the text proposal on the framework of RAN4 part in TR 38.744 to capture the RAN4 agreements.

- Text proposal

< Start of text proposal >

6.2	Interoperability, testability and RRM requirements

6.2.1 RRM requirements for measurement prediction

6.2.1.1 General

The impact on RRM requirements for L3 cell-level RSRP (i.e., Point C as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has been studied for scenarios 2, 3, and 4 as defined in Table 5.2.1.1-1 and the sub-use cases 1, 2, and 3 as defined in section 4.2.

Editor Note: The impact of L3 beam level (Point E as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has not been studied. However, it doesn’t preclude the discussion on L3 beam level measurement prediction in the work item phase.

Editor Note: From RAN4 perspective, it is not precluded that other scenarios in Table 5.2.1.1-1 will be discussed in work item.

Editor Note: For scenario 3, non-co-located cases are not precluded.

6.2.1.2 Potential RRM requirements

The requirement impact corresponding to Point A1 as defined in Figure 9.2.4-1 in TS 38.300 for measurement prediction is not to be studied.

Both absolute and relative accuracy of predicted L3-RSRP are considered as the candidate metrics for RRM measurement prediction use cases for intra-frequency scenario and inter-frequency scenarios.

The absolute accuracy of predicted L3-RSRP is defined as:

- Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

  - cell 1 and cell 2 are on the same frequency

  - the reported RSRP of cell 2 can be measured or predicted.

- Editor Note: The relative RSRP accuracy for beam level measurements may be further discussed during WI phase depending upon RAN2 progress.

The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

  - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

  - the reported RSRP of cell 2 can be measured or predicted.

Editor Note: RAN4 will further discuss whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

As a baseline, the ground truth of L3-RSRP in FR2 is defined as the reported L3 RSRP measurement result under sufficient high SNR.

The ground truth of L3-RSRP in FR1 is based on:

- Alt1: The transmitted or reception power

- Alt2: The reported measurement value under certain conditions.

For intra-frequency temporal domain prediction, the impact on prediction accuracy by UE speed, MRRT, and OW/PW length will be considered. The number of measurements performed in OW could be also considered.

For inter-frequency prediction scenario, the impact on prediction accuracy by following elements can be considered:

- Side condition of frequency prediction (e.g., EPRE difference)

- Cluster approach, e.g.,

  - when measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- RAN4 may further discuss whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

For both intra-frequency and inter-frequency prediction, RAN4 thinks RF and baseband errors on the measurements would impact the prediction accuracy.

- Editor Note: RAN4 may continue discussion on the measurement error model in RAN4#116. Any further agreements will be further captured in this clause

6.2.2 RRM requirements for measurement event prediction

6.2.2.1 General

The impact on RRM requirements for both direct and indirect event prediction use cases as defined in section 5.3 has been studied.

6.2.2.2 Potential RRM requirements

For indirect event prediction, the requirements for the predicted event triggered reporting are considered including [delay and accuracy] requirements and the performance metrics. If the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

Editor Note: For both indirect and direct event prediction, RAN4 may continue discussion on performance metrics. Any further agreements will be further captured in this clause.

6.2.3 Testability for RRM measurement prediction

6.2.3.1 Testing goal

As a baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

6.2.3.2 Prediction consistency in time domain

To ensure prediction consistency in the time domain, it is considered how to model the different time-varying characteristics per cell/site due to moving UE trajectories. It is also considered how to incorporate controlled randomness and the extent of time-domain variation and correlation.

FR1 is prioritized during the discussion.

6.2.3.3 Testing setup

For testing in FR1, conducted testing is considered as a baseline.

For the number of cells configured in the testing, two cells are considered including serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. In the test, the measurement and prediction are performed on the same cell.

Editor Note: RAN4 will further decide if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

6.2.4 Interoperability

RAN4 has not concluded on any interoperability aspect in this study item.

Editor Note: RAN4 may further discuss on this issue in RAN4#116, any further agreements will be captured in this clause.

6.2.5 Generalization

RAN4 has not concluded on any generalization aspect in this study item.

Editor Note: RAN4 may further discuss on this issue in RAN4#116, any further agreements will be captured in this clause.

<End of text proposal>