# 3GPP TSG-RAN WG4 Meeting #115	R4-2508446

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.31.1

Source: 	Qualcomm Incorporated, CATT

Title: 	Ad-hoc minutes for NR_NTN_Ph3

Document for:	Information

# 0 Introduction

This document is the Ad-hoc minutes for NR_NTN_Ph3-Core WI with the following topics covered.

[115][227] NR_NTN_Ph3_Part1:

- Sub-topic 1-2: Issue 1-2-1

- Sub-topic 1-2: Issue 1-2-3 (new issue based on online discussion)

[115][228] NR_NTN_Ph3_Part2:

- Issue 0: LS from RAN2 on multiple SMTCs with different periodicities

[115][227] NR_NTN_Ph3_Part1

    - Sub-topic 1-2 SMTCs and 'in parallel' related features

Issue 1-2-1: The proximity distance between two SMTCs

- Proposals

  - Proposal 1 (Apple, Samsung, CATT, CMCC, HW, Xiaomi): When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

    - the two SMTC occasions are fully or partially overlapping in time domain, or

    - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4ms+ x= Yms.

      - Proposal 1a (Apple, Samsung, CATT, CMCC, HW, Xiaomi): x=1ms

      - Proposal 1b (QC): x=1~3ms

      - Proposal 1c (OPPO): x=0ms

  - Proposal 2 (Nokia): RAN4 to specify for 1Rx and 2Rx RedCap UE with FR1-NTN, that proximity distance between two SMTCs configured on a SAN carrier is defined as:

    - i) 4ms in case UE is configured to measure 1 satellite in each of the SMTC occasions.

    - ii) 5ms in case UE is configured to measure 2 satellites in at least one of the SMTC occasions.

Agreement:

When 1Rx and 2Rx (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

    - the two SMTC occasions are fully or partially overlapping in time domain, or

    - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4ms+ x= Yms.

      - For a UE capable of 25-2 (parallelMeasurementWithoutRestriction-r17):

        - If UE is configured to measure 1 satellite in each of the SMTC occasions, X = 0

        - Otherwise, X = 1

      - For a UE incapable of 25-2 (parallelMeasurementWithoutRestriction-r17):

        - X = 1

Issue 1-2-3: Further clarification on Feature 25-1(parallelSMTC)

- Background

The agreements for Feature 25-1(parallelSMTC) in the last meeting is copied as below:

##### RAN4#114-bis: ##### Issue 1-2-1-1: Feature 25-1(parallelSMTC) Agreement: - For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability. - RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier - ‘Parallel’ means the SMTCs are TDM with proximity distance.

- Regarding whether RedCap UEs with NTN still optional support parallel measurements on 4 SMTC-s for a single frequency carrier,

  - Option 1: Yes.

    - RedCap UEs with NTN still optional support parallel measurements on 4 SMTC-s for a single frequency carrier, and the UE capability of Feature 25-1(parallelSMTC) can be reused.

  - Option 2: No.

    - RedCap UEs with NTN will not optional support parallel measurements on 4 SMTC-s for a single frequency carrier, UE is only mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier.

    - A new UE capability of parallelSMTC is needed.

Agreement:

- RedCap UEs with NTN still optional support parallel measurements on 4 SMTC-s for a single frequency carrier, and the UE capability of Feature 25-1(parallelSMTC) can be reused.

[115][228] NR_NTN_Ph3_Part2

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities

LS on SMTC enhancements (R4-2505313_R2-2503068) For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe. ACTION: RAN2 kindly asks RAN4 to provide feedback on the feasibility to support those agreements in Rel-19 timeframe.

Agreement:

From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

- In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference:

- FFS on the exact RRM requirement change.

- FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment

Check during online session:

From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements.

- RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.

Check during online session:

The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of this WI.