# 3GPP TSG-RAN WG4 Meeting #115	R4-2508625

St Julian’s, Malta, 19 May – 23 May 2025

Agenda item:			7.10.5

Source:	Samsung

Title:	Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod

Document for:	Approval

Introduction

This t-doc provides WF for [115][324] NTN_testing_NGSO_channel_model_and_demod.

WF

Sub-topic 1-1 TE emulated channel model for satellite mobility

Issue 1-1-1 Propagation delay model with considering earth’s rotation during signal flight time

- agreement

  - Downlink propagation delay model at UE reception time tR with considering the Earth's rotation during signal flight time

DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

  - FFS whether and how to consider the propagation delay model for UL delay error for RRM uplink timing error cases

    - Option 1:

ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT#Where

SV (Satellite)		: Receiver

UE 				: Transmitter

∆tUL= tR- tT  : Uplink total propagation delay

tT					: Time when signals are transmitted from UE

tR					: Time when signals are received by satellite

rR 					: SV position vector on ECI Frame at tT.

rT 					: UE position vector on ECI Frame at tT.

VSV				: Satellite velocity vector on ECI Frame

RUL=rR-rT	: Distance between satellite and UE at either tT or tR depending on the reference point.

c = 2.99792458 x 108 [m/s]

    - Other options are not precluded

Issue 1-1-2 Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory

- agreement

  - The mean of absolute error for Doppler should be less than 1 Hz and the mean of absolute error for Delay should be less than 0.05µs based on the following error calculation method

ErrDopplert=11000n=11000Dopplerbaselinet+n1000-Dopplerinterpt+n1000, t=0,1,2,…(sec)

ErrDelayt=11000n=11000Delayrbaselinet+n1000-Delayrinterpt+n1000, t=0,1,2,…(sec)

Note: where Dopplerbaselinen, Dopplerinterp(n), Delayrbaselinen, and Delayinter(n) are calculated once per 1ms, and the average is taken over every 1 second.

Issue 1-1-3 Initial ephemeris information for LEO-600 satellite

- agreement

  - Sets the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654685	, 4387308, 1593097, 14569, -34465, 120190), which corresponds to (-3451.0905 km, 5703.5004 km, 2071.0261 km, 0.87414 km/s, -2.0679 km/s, 7.2114 km/s), related with the initial elevation angle of 30 degrees

Issue 1-1-4 Initial ephemeris information for LEO-1200 satellite

- agreement

  - Reuse TS 38.508 Table 7.6.2.1-3a as the initial ephemeris information for LEO-1200

Table Initial ephemeris information for LEO-1200 stating from elevation angles of 30 degrees (Copy from TS 38.508 Table 7.6.2.1-3a, serving cell 30 degrees elevation angle conditions).

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | -3816.2176 |
| positionY-r17 | km | 6364.098 |
| positionZ-r17 | km | 1544.9954 |
| velocityVX-r17 | km/s | 0.51036 |
| velocityVY-r17 | km/s | -1.39884 |
| velocityVZ-r17 | km/s | 7.09518 |

Sub-topic 1-2 RAN4 requirements

Issue 1-2-3 Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- agreement

  - Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply.

  - Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

    - residual frequency offset and time delay

      - option 1: Fix value

        - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s

        - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s

      - option 2: time-varying propagation delay and Doppler shift.

        - FFS

  - Companies are encouraged to provide the simulation results during RAN4#116 meeting

Issue 2-1-2 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model

- agreement

  - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

Issue 2-1-3 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model

- agreement

  - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1.

Issue 2-1-4 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model

- agreement

  - Only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model.

Issue 2-1-5 Testing time of demodulation test with NGSO LEO-600

- agreement

  - RAN4 introduce the channel model for both NGSO LEO-600 and LEO-1200. If the LEO-600 model is not enough to cover the test time of NR and IoT NTN test cases defined in RAN4, it is up to RAN5 to decide whether LEO-1200 will be used for testing

- Reference

[1] R4-2507856, “Topic summary for [115][324] NTN_testing_NGSO_channel_model_and_demod “, Moderator (Samsung)

- Appendix (for information)

  - A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

A.1.1	Keplerian model based estimation

Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

A.1.2	Equation of Motion based estimation

Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if the satellite position/velocity is updated every 1 second.

Step 2	Solve the equation of motion with the 4th-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

Where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

A.2	Varying Doppler shift and propagation delay generation

A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since it is assumed that UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |

ρtECEF=ρt,xECEIρt,yECEIρt,zECEI=rt,xECEF-UEt,xrt,yECEF-UEt,yrt,zECEF-UEt,z

Range(t)=ρtECEI=ρt,xECEI2+ρt,yECEI2+ρt,zECEI2

RangeRate(t)=ρtECEF⋅vtECEFRnage(t)=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFRange(t)

Doppler(t)=-RangeRate(t)×fcc

vtECI=vt,xECIvt,yECIvt,zECI=vtECEF+00ωE×rtECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Delayt=ρtECEF/c1+1cvtECI⋅ρtECEFρtECEF

A.2.3	Elevation angle and Azimuth angle (for information)

Additionally, the elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

Propagation delay

D.1.1	Propagation delay based on the satellite transmission time (Option 1)

In this option, we modify UEtRECEF-rtTECEF=c ⋅DtTSat with the relation: UEtRECEF=UEtTECEF+vtTUE,ECI⋅DtTSat. That is,

UEtTECEF+vtTUE,ECI ⋅DtTSat-rtTECEF=c ⋅DtTSat

We should emphasize the UE velocity vectors, vtTUE, ECI, shall be inertial reference frame, given by:

vtTUE,ECI=vtTUR,ECEF+00ωE×rtTUR,ECEF=00ωE×rtTUR,ECEF

Note we assume the UE is stationary in ECEF, that is, vtTUR,ECEF=0.

By defining, ρtT ECEF:=UEtTECEF-rtTECEF

ρtT ECEF+vtTUE,ECI⋅DtTSat=c⋅DtTSat

c⋅DtTSat2=ρtT ECEF+vtTUE,ECI⋅DtT2≈ρtT ECEF2+2vtTUE,ECI⋅ρtT ECEF⋅DtTSat

By taking a square root of the equation,

c⋅DtTSat=ρtT ECEF2+2vtTUE,ECI⋅ρtT ECEF⋅DtTSat

=ρtT ECEF1+2vtTUE,ECI⋅ρtT ECEFρtT ECEF2⋅DtTSat

≈ρtT ECEF1+vtTUE,ECI⋅ρtT ECEFρtT ECEF2⋅DtTSat=ρtT ECEF+vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat

Finally, we get the equation below.

c⋅DtTSat≈ρtT ECEF+vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat

Solve the above equation by DtT,

c-vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat=ρtT ECEF

DtTSat=ρtT ECEFc-vtTUE,ECI⋅ρtT ECEFρtT ECEF=ρtT ECEF/c1-1cvtTUE,ECI⋅ρtT ECEFρtT ECEF

D.1.2	Propagation delay based on the UE receiption time (Options 2/4)

In this option, we modify UEtRECEF-rtTECEF=c ⋅DtRUE with the relation: rtRECEF=rtTECEF+vtRECI⋅DtRUE. That is,

UEtRECEF-rtRECEF-vtRECI⋅DtRUE=c⋅DtRUE

We should emphasize the satellite velocity vectors, vtRECI, shall be inertial reference frame, given by:

vtRECI=vtRECEF+00ωE×rtRECEF

By defining, ρtR ECEF:=rtRECEF-UEtRECEF

c⋅DtRUE=UEtRECEF-rtRECEF+vtRECI⋅DtRUE

c2DtRUE2=UEtRECEF-rtRECEF+vtRECI⋅DtRUE2

c⋅DtRUE2=-ρtRECEF+vtRECI⋅DtRUE2=ρtRECEF-vtRECI⋅DtRUE2

≈ρtRECEF2-2vtRECI⋅ρtRECEF⋅DtRUE

By taking a square root of the equation,

c⋅DtR=ρtRECEF2-2vtRECI⋅ρtRECEF⋅DtRUE

=ρtRECEF1-2vtRECI⋅ρtRECEFρtRECEF2⋅DtRUE

≈ρtRECEF1-vtRECI⋅ρtRECEFρtRECEF2⋅DtRUE=ρtRECEF-vtRECI⋅ρtRECEFρtRECEF⋅DtRUE

Finally, we get the equation below.

c⋅DtRUE≈ρtRECEF-vtRECI⋅ρtRECEFρtRECEF⋅DtRUE

Solve the above equation by Δt,

c+vtRECI⋅ρtRECEFρtRECEF⋅DtRUE=ρtRECEF

DtRUE=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF