# 3GPP TSG-RAN WG4 Meeting #115                                                                R4-2508647

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | - | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS36.108 Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1.Add 10 MHz channel bandwidth for SAN ACLR limit for GEO class to support New band for 5G Broadcast Utilizing Geosynchronous Satellite - 2.Add 10 MHz channel bandwidth for EVM window length to support New band for 5G Broadcast Utilizing Geosynchronous Satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New band for 5G Broadcast Utilizing Geosynchronous Satellite requirements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2, B.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.181 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

6.6.3.2	Minimum requirement for SAN type 1-H

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

The ACLR shall be higher than the value specified in Table 6.6.3.2-1 and Table 6.6.3.2-2.

Table 6.6.3.2-1: SAN ACLR limit for GEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 1.4, 10 | BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
| NOTE: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. |  |  |  |  |

Table 6.6.3.2-2: SAN ACLR limit for LEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 1.4 | BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 24 |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | [24] |
| NOTE: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. |  |  |  |  |

For SAN supporting standalone NB-IoT operation in paired spectrum, the ACLR shall be higher than the value specified in Table 6.6.3.2-3 and Table 6.6.3.2-4.

Table 6.6.3.2-3: ACLR limit of SAN supporting standalone NB-IoT operation for GEO class

| Channel bandwidth of NB-IoT lowest/highest carrier transmitted BWChannel (kHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 200 | [200 kHz] | Standalone NB-IoT | Square (180 kHz) | 14 |
|  | [400 kHz] | Standalone NB-IoT | Square (180 kHz) | 14 |

Table 6.6.3.2-4: ACLR limit of SAN supporting standalone NB-IoT operation for LEO class

| Channel bandwidth of NB-IoT lowest/highest carrier transmitted BWChannel (kHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 200 | [200 kHz] | Standalone NB-IoT | Square (180 kHz) | 24 |
|  | [400 kHz] | Standalone NB-IoT | Square (180 kHz) | 24 |

# << Next change >>

B.5.1	Window length

Table B.5.1-1 and Table B.5.1-1a below specify EVM window length (W) for normal CP, the cyclic prefix length  is 10 for symbols 0 and 9 for symbols 1-6.

Table B.5.1-2 specify the EVM window length (W) for extended CP for 15 kHz, subcarrier spacing.

Table B5.1-1: EVM window length for normal CP for E-UTRA

| Channel Bandwidth (MHz) | FFT size |  | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (NOTE) (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 128 |  | 10 | 9 | 5 | 55.6 |
| Note: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |  |

Table B.5.1-2: EVM window length for normal CP for NB-IoT

| FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (NOTE) (%) |
| --- | --- | --- | --- | --- |
| 128 | 10 | 9 | 3 | 33.3 |
| Note: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table B.5.1-3: EVM window length for extended CP for 15 kHz subcarrier spacing

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (NOTE) (%) |
| --- | --- | --- | --- | --- |
| 1.4 | 128 | 32 | 28 | 87.5 |
| 10 | 1024 | 256 | 250 | 97.7 |
| Note : These percentages are informative. |  |  |  |  |

# << End of change >>