3GPP TSG-RAN4 Meeting #115	R4-2508671

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for FRCs for ICI requirements with 8RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce reference channels for requirements introduced for inter-cell interference with 8RX |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced FRCs for FDD and TDD for MCS 17 with 2 layers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test definiton will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, Table A.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

Table A.3.2.1.1-25: PDSCH Reference Channel for FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-25.1 FDD | R.PDSCH.1-25.2 FDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 25 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 17 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 17 |  |  |  |
| Modulation |  | 16QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.43 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 6272 | 34816 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 | 5 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 12600 | 78624 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13200 | 82368 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 5.958 | 33.075 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Change 2

Table A.3.2.2.2-27: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  |  | R.PDSCH.2-27.1 TDD |  |  | R.PDSCH.2-27.2 TDD |  |  | R.PDSCH.2-27.3 TDD |  |  | R.PDSCH.2-27.4 TDD |  |  |
| Channel bandwidth |  |  | MHz |  |  | 20 |  |  | 20 |  |  | 20 |  |  | 40 |  |  |
| Subcarrier spacing |  |  | kHz |  |  | 30 |  |  | 30 |  |  | 30 |  |  | 30 |  |  |
| Allocated resource blocks |  |  | PRBs |  |  | 51 |  |  | 12 |  |  | 6 |  |  | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  |  |  |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  |  |  |  | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  |  |  |  |  | 12 |  |  | 12 |  |  | 12 |  |  | 12 |  |  |
| Allocated slots per 2 frames |  |  |  |  |  | 31 |  |  | 31 |  |  | 31 |  |  | 31 |  |  |
| MCS table |  |  |  |  |  | 64QAM |  |  | 64QAM |  |  | 64QAM |  |  | 64QAM |  |  |
| MCS index |  |  |  |  |  | 19 |  |  | 19 |  |  | 19 |  |  | 17 |  |  |
| Modulation |  |  |  |  |  | 64QAM |  |  | 64QAM |  |  | 64QAM |  |  | 64QAM |  |  |
| Target Coding Rate |  |  |  |  |  | 0.51 |  |  | 0.51 |  |  | 0.51 |  |  | 0.51 |  |  |
| Number of MIMO layers |  |  |  |  |  | 2 |  |  | 1 |  |  | 2 |  |  | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  |  | 6 |  |  | 6 |  |  | 6 |  |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  |  |  | 12 |  |  | 12 |  |  | 12 |  |  | 12 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits |  |  | 13064 |  |  | 1544 |  |  | 1544 |  |  | 23040 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits |  |  | 40976 |  |  | 4736 |  |  | 4736 |  |  | 71688 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits |  |  | 24 |  |  | 16 |  |  | 16 |  |  | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits |  |  | 24 |  |  | 24 |  |  | 24 |  |  | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs |  |  | 2 |  |  | 1 |  |  | 1 |  |  | 3 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs |  |  | 5 |  |  | 1 |  |  | 1 |  |  | 9 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |  |
| For Slots i = 20, 21 |  | Bits |  |  | 77112 |  |  | 9072 |  |  | 9072 |  |  | 160272 |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits |  |  | 25704 |  |  | 3024 |  |  | 3024 |  |  | 53424 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | Bits |  |  | 80784 |  |  | 9504 |  |  | 9504 |  |  | 167904 |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps |  |  | 57.930 |  |  | 6.702 |  |  | 6.702 |  |  | 101.386 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |