3GPP TSG RAN WG4 Meeting #115	R4-2508698

# Malta, Malta, 19 May 2025 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.922 | CR | 0010 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (FS_NR_IMT_4400_7125_14800MHz) CR to TR 38.922 on BS spurious emissions for 4400 to 4800 MHz frequency range and spectral mask for 14800 to 15350 MHz frequency range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_4400_7125_14800MHz |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In some BS spurious emissions and spectrum mask requirement tables, “basic limit” is used. However, the required values are not for the basic limit defined as emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update on AAS BS spectral masks for WA BS of 14800 to 15350 MHz - Addition of AAS BS spectral masks for Micro cell and small cell indoor of 14800 to 15350 MHz - Clarificaiton on AAS BS spurious emissions for Micro cell and small cell indoor of 14800 to 15350 MHz - “Basic limit” is replaced by “limit” in the tables for BS spurious emissions for 4400 to 4800 MHz frequency range and spectral mask for 14800 to 15350 MHz frequency range |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are too stringent. TR will not contain the latest agreements for AAS BS spectral masks and spurious emissions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.4, 6.3.1.2, 6.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Corrections are on the endorsed draft CR R4-2504266 that updates the AAS BS spectral masks for WA BS for 14800 to 15350 MHz frequency range and agreed on new infromation on AAS BS spectral masks and spurious emissions for Micro cell and small cell indoor, in response to 5D/TEMP/273 from ITU-R WP 5D. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

#### 4.2.1.4	Spurious emissions

The spurious emission limits applicable for band n79 are re-used. Related extracts from TS 38.104 [9] are listed in Table 4.2.1.4-1 and Table 4.2.1.4-2. for conducted requirements.

Table 4.2.1.4-1: General BS transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329, s4.1. NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. |  |  |  |

Table 4.2.1.4-2: General BS transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329, s4.1. NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. |  |  |  |

Additional spurious emissions requirements relevant for band n79 can be found in TS 38.104 [9], clause 6.6.5.2.3 and clause 6.6.5.2.4. Refer to TS 38.104 [9] Clause 9.7.5 for radiated requirements applicable for this range.

--------------Next change-------------

#### 6.3.1.2	Spectral mask

Category A limits for Wide Area BS for AAS BS are listed in Table 6.3.1.2-1.

Table 6.3.1.2-1: Wide Area BS operating band unwanted emission limits for AAS BS (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | -72dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -5 dBm | 100 kHz |
| 100 MHz  f  fmax | -4 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Note: AAS BS Category B limits for Wide Area BS are not yet specified by regional regulatory bodies.

Medium Range BS and Local Area BS OBUE limits are listed in Table 6.3.1.2-2, 6.3.1.2-3 and 6.3.1.2-4.

Table 6.3.1.2-2: Medium Range BS operating band unwanted emission limits for AAS BS, 40 < Prated,c,TRP  47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | Prated,c,TRP-53dB-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | Prated,c,TRP - 60dB | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP - 60dB, -16dBm) | 100 kHz |
| NOTE 1: Prated,c,TRP is rated carrier total radiated power declared per radiated interface boundary. NOTE 2: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

Table 6.3.1.2-3: Medium Range BS operating band unwanted emission limits for AAS BS, Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -13dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -20 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -20 dBm | 100 kHz |
| NOTE 1: Prated,c,TRP is rated carrier total radiated power declared per radiated interface boundary. NOTE 2: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

Table 6.3.1.2-4: Local Area BS operating band unwanted emission limits for AAS BS

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -21dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -28 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -28 dBm | 100 kHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

--------------Next change-------------

#### 6.3.1.4	Spurious emissions

The Category A spurious emission limits applicable are extracted from TS 38.104 [9], as listed in Table 6.3.1.4-1.

Table 6.3.1.4-1: General BS transmitter spurious emission limits for 14800 - 15350 MHz operation (Category A)

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

Note: AAS BS Category B limits are not yet specified by regional regulatory bodies.

The AAS BS transmitter spurious emission limits in Table 6.3.1.4-1 are also applicable to Medium Range and Local Area BS.

--------------End of change-------------