3GPP TSG-RAN WG4 Meeting #115                                                               R4-2508707

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.25.1

Source:	Samsung

Title:	Way Forward for [115][306] NR_duplex_evo_General

# Topic #1: General system parameters for SBFD operation

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

    - X value will be selected in next meeting.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

##### Issue 1-2-4: Restriction on DL subband size

Agreement:

- If the SSB symbol is overlapped with SBFD symbol,

  - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration

  - FFS the test configuration during performance part discussion.

### Sub-topic 1-3: Requirement applicability and BS RF specification structure for SBFD

##### Issue 1-3-1: Missed clause for EEIRP introduced in Rel-19

Agreement:

- Use the following updated clause structure and requirement set applicability for SBFD-capable BS.

12	Requirements for SBFD-capable BS

12.1	General

12.2	Conducted transmitter characteristics for SBFD-capable BS

12.2.1	General

12.2.2	Base station output power for SBFD

12.2.3	Output power dynamics for SBFD

12.2.4	Transmit ON/OFF power for SBFD

12.2.5	Transmitted signal quality for SBFD

12.2.6	Unwanted emissions for SBFD

12.2.7	Transmitter intermodulation for SBFD

12.2.8	Transient period between SBFD and non-SBFD

12.2.9	In-channel adjacent subband leakage for SBFD

12.3	Conducted receiver characteristics for SBFD-capable BS

12.3.1	General

12.3.2	Reference sensitivity level for SBFD

12.3.3	Dynamic range for SBFD

12.3.4	In-band selectivity and blocking for SBFD

12.3.5	Out-of-band blocking for SBFD

12.3.6	Receiver spurious emissions for SBFD

12.3.7	Receiver intermodulation for SBFD

12.3.8	In-channel selectivity for SBFD

12.3.9	In-channel adjacent subband blocking for SBFD

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS

12.5.1	General

12.5.2	Radiated transmit power for SBFD

12.5.3	OTA base station output power for SBFD

12.5.4	OTA output power dynamics for SBFD

12.5.5	OTA transmit ON/OFF power for SBFD

12.5.5.1	General

12.5.5.2	OTA transmitter OFF power for SBFD

12.5.5.3	OTA transient period for SBFD

12.5.6	OTA transmitted signal quality for SBFD

12.5.7	OTA unwanted emissions for SBFD

12.5.8	OTA transmitter intermodulation for SBFD

12.5.9	OTA spatial emission for SBFD

12.5.10	OTA transient period between SBFD and non-SBFD

12.5.11	OTA in-channel adjacent subband leakage for SBFD

12.6	Radiated receiver characteristics for SBFD-capable BS

12.6.1	General

12.6.2	OTA sensitivity for SBFD

12.6.3	OTA reference sensitivity level for SBFD

12.6.4	OTA dynamic range for SBFD

12.6.5	OTA in-band selectivity and blocking for SBFD

12.6.6	OTA out-of-band blocking for SBFD

12.6.7	OTA receiver spurious emissions for SBFD

12.6.8	OTA receiver intermodulation for SBFD

12.6.9	OTA in-channel selectivity for SBFD

12.6.10	OTA in-channel adjacent subband blocking for SBFD

12.7	   Radiated performance requirements for SBFD-capable BS

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA operating band unwanted emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 |  | 12.5.9 | 12.5.9 |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.11 | 12.5.11 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

Agreement:

- The work splitting is further updated accordingly:

| Work splitting packages | Relevant impacted clauses in TS38.104 | Responsible company |
| --- | --- | --- |
| Package-1 | Chapter 2 References Chapter 3 Definitions, symbols and abbreviations Chapter 5 Operating bands and channel arrangement | Samsung |
| Package-2 | 12.2 Conducted transmitter characteristics for SBFD-capable BS 12.2.1 General 12.2.2 Base station output power for SBFD 12.2.3 Output power dynamics for SBFD 12.5 Radiated transmitter characteristics for SBFD-capable BS 12.5.1 General 12.5.2 Radiated transmit power for SBFD 12.5.3 OTA base station output power for SBFD 12.5.4 OTA output power dynamics for SBFD | Ericsson |
| Package-3 | 12.2.4 Transmit ON/OFF power for SBFD 12.2.5 Transmitted signal quality for SBFD 12.5.5 OTA transmit ON/OFF power for SBFD 12.5.6 OTA transmitted signal quality for SBFD | Nokia |
| Package-4 | 12.2.6 Unwanted emissions for SBFD 12.5.7 OTA unwanted emissions for SBFD | CMCC |
| Package-5 | 12.2.7 Transmitter intermodulation for SBFD 12.5.8 OTA transmitter intermodulation for SBFD | Nokia |
| Package-14 | 12.5.9 OTA spatial emission | Ericsson |
| Package-6 | 12.2.8 Transient period between SBFD and non-SBFD 12.5.10 OTA transient period between SBFD and non-SBFD | Samsung |
| Package-7 | 12.2.9 In-channel adjacent subband leakage for SBFD 12.5.11 OTA in-channel adjacent subband leakage for SBFD | ZTE |
| Package-8 | 12.3 Conducted receiver characteristics for SBFD-capable BS 12.3.1 General 12.3.2 Reference sensitivity level for SBFD 12.3.3 Dynamic range for SBFD 12.6 Radiated receiver characteristics for SBFD-capable BS 12.6.1 General 12.6.2 OTA sensitivity for SBFD 12.6.3 OTA reference sensitivity level for SBFD 12.6.4 OTA dynamic range for SBFD | Huawei |
| Package-9 | 12.3.4 In-band selectivity and blocking for SBFD 12.6.5 OTA in-band selectivity and blocking for SBFD | Huawei |
| Package-10 | 12.3.5 Out-of-band blocking for SBFD 12.6.6 OTA out-of-band blocking for SBFD | CATT |
| Package-11 | 12.3.6 Receiver spurious emissions for SBFD 12.3.7 Receiver intermodulation for SBFD 12.6.7 OTA receiver spurious emissions for SBFD 12.6.8 OTA receiver intermodulation for SBFD | Qualcomm |
| Package-12 | 12.3.8 In-channel selectivity for SBFD 12.3.9 In-channel adjacent subband blocking for SBFD 12.6.9 OTA in-channel selectivity for SBFD 12.6.10 OTA in-channel adjacent subband blocking for SBFD | Ericsson |
| Package-13 | B.7 Averaged EVM | ZTE |

# Topic #2: SBFD-aware UE related issues

### Sub-topic 2-2: Update for HPUE duty cycle evaluation

##### Issue 2-2-1: The necessity to update for HPUE duty cycle evaluation

- No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

# Topic #4: New BS RF requirements for SBFD

### Sub-topic 4-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 4-1-2: Text proposal related to transient period between SBFD and non-SBFD

Agreement:

- Text proposal related to transient period requirement between SBFD and non-SBFD

  - Terminology “non-SBFD” need to be introduced.

  - Use below figure for the illustration of transient period between SBFD and non-SBFD in TS 38.104.

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

### Sub-topic 4-2: In-channel adjacent subband leakage

##### Issue 4-2-1: Assumption for in-channel adjacent subband leakage requirement

- No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

##### Issue 4-2-2: Absolute limit for in-channel adjacent subband leakage

Agreement

- For absolute limit for in-channel adjacent subband leakage:

  - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed.

  - For FR1 BS,

    - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed.

    - For FR1 WA and MR SBFD-capable BS,

      - Option 1: [3]dB relaxation is provided.

      - Option 2: no relaxation from corresponding existing requirement

### Sub-topic 4-3: In-channel adjacent subband blocking

##### Issue 4-3-1: In-channel adjacent subband blocking requirement

Agreement

- Wanted signal mean power, i.e., degradation level for Refsens level

  - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens

  - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON.

- On the level of in-channel adjacent subband blocking requirement:

  - For FR1 WA BS:

    - [-31+3 = -28] dBm

    - FFS isolation between sectors can captured in specification

  - For FR1 MR BS:

    - [-32] dBm.

    - FFS isolation between sectors can captured in specification

  - For FR1 LA BS:

    - [-34] dBm.

# Topic #5: BS RF conformance requirements for SBFD

### Sub-topic 5-2: General observations/proposals for BS RF conformance testing

##### Issue 5-2-1: General observations/proposals for BS RF conformance testing

Way forward:

- For BS RF conformance requirement, the following issues are identified can be further discussed in next meetings:

  - Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS.

    - Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

    - FFS other declaration is required

  - Issue-2: RF channels to be tested for SBFD operation

    - FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band.

  - Issue-3: Additional test models are needed for SBFD-capable BS.

    - FFS new parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

    - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband.

    - FFS the test model for SBFD DL subbband needs to be scalable

    - FFS signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed.

    - FFS how to accommodate SBFD time domain pattern with the test model.

  - Issue-4: UL FRC

    - The existing UL FRC can be reused for SBFD testing.

    - FFS any other impact existence on UL FRC.

  - Issue-5: Interference signal

    - FFS the interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement