3GPP TSG-RAN WG4 Meeting #115						R4-2508748

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.16.3

Source:	Apple, Qualcomm

Title:	Draft TP to TR on SCM study - 5.1 CDL approach

Release:	Rel-19

Document for:	Endorsement

# 1. Introduction

This draft TP captures the CDL approach for spatial channel modelling methodologies.

# 2. Draft TP

## 5.1	CDL Approach

The Clustered Delay Line (CDL) channel model is a widely used framework for simulating realistic wireless propagation environments. CDL models capture the spatial, temporal, and frequency-dependent characteristics of radio channels by representing multipath propagation through discrete clusters of scatterers, each contributing multiple delay paths. The details of the CDL based approach employed in the study are provided in this section with the relevant parameter configurations. The model employed in the study is applicable for 3.5 GHz frequency in urban macro scenario.

### 5.1.1	Antenna Panel Configuration

The antenna panel is one of the primary components for CDL modeling methodology. The antenna panel configuration employed in this study for BS and UE are presented in Table 5.1.1.-1.

Table 5.1.1-1: BS and UE Panel parameters

| Parameter | Value |
| --- | --- |
| LCS UE | α = 180°, β=0°, γ = 0° |
| LCS gNodeB | α = 0°, β=10°, γ = 0° |
| GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
| GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
| Antenna Panel Placement | YZ Plane |

### 5.1.2	Antenna Array Virtualization

The antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one antenna port through a virtualizer (set of complex values).

The antenna panel configuration is defined as (M,N,P,Ms,Ns), according to the description below:

M is the number of antenna elements per panel with the same polarization in each column (vertical size)

N is the number of antenna elements per panel with the same polarization in each row (horizontal size)

P is the number of polarizations

Ms is the number of antenna elements per subarray with the same polarization in each column (vertical size)

Ns is the number of antenna elements per subarray with the same polarization in each row (horizontal size)

Different AAV configurations for base station are employed in this study.

BS AAV Configurations:

- Sub-array Antenna Array Virtualization (AAV) configuration

(M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

(M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- Pass though AAV configuration:

(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

UE AAV is a pass-through AAVc configuration

(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX

### 5.1.3	Antenna Element Configuration

<This section provides parameters for antenna element configuration>

Table 5.1.3-1: BS and UE Antenna element parameters

| Parameter | Value |
| --- | --- |
| BS Antenna Polarization | (+45, -45) |
| BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |
| UE Antenna Polarization | (0, 90) |
| UE Antenna Radiation Pattern | Omnidirectional |

### 5.1.4	Channel Models

The CDL-C model from 38.901 in Table 7.7.1-3 is taken as a starting point. The parameters for angle scaling, delay spread are derived based on urban macro (UMa) scenario for 3.5 GHz carrier. The randomness introduced by the channel generation based on 38.901 is alleviated by randomness reduction framework adopted from 38.827.

#### 5.1.4.1	Parameters for Angle Scaling

The angles need to be scaled for the scenario to achieve the desired angular spread and direction. The parameters for scaling the angles of arrival and departure are provided here. The CDL tables provided in 5.1.4.3 include the delay scaling for RMS delay of 365ns and desired angle scaling.

φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model)

φn,model	is the tabulated CDL ray angle from 38.901

ASmodel	is the rms angular spread of the tabulated CDL from 38.901 including the offset ray angles provided below

μφ,model	is the mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901

ASdesired	is the desired rms angular spread provided below

φn,scaled	is the resulting scaled ray angle.

y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180

The power weighted mean angle is given by

μφ=arg{n=1Nm=1Mexp(jφn,m)Pn,m}

where Pn,m is the power for the mth ray of the nth cluster and φn,m is the ray angle (either AOA, AOD, ZOA, ZOD) given in radians.

Table 5.1.4.1-1: Angle scaling parameters

| AS | ASmodel / ASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| Model (827) | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| Model (901) | 37.4036 | 71.4535 | 4.0665 | 10.4182 |
| Desired | 25.7620 | 74.1138 | 4.8978 | 18.2050 |

#### 5.1.4.2	Parameters for Randomness Reduction

The randomness reduction framework employs fixed coupling pattern for ray angles and fixed initial phases for polarization matrices in the channel coefficient generation.

Within a cluster the AOD angles are coupled to AOD angles using the fixed coupling pattern specified in Table 5.1.4.2-1.  The same fixed coupling pattern is applied for all clusters n.

Table 5.1.4.2-1: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

The initial phase {Φn,mθθ,Φn,mθφ,Φn,mφθ,Φn,mφφ} for each ray m of each cluster n and for four different polarization combinations (θθ, θϕ, ϕθ, ϕϕ) are fixed as in the Table 5.1.4.2-2

Table 5.1.4.2-2: Fixed initial phases for 2x2 polarization matrices

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

#### 5.1.4.2	CDL Truncation

The CDL model is truncated to 12 taps based on the procedure provided in Annex.

#### 5.1.4.3	Channel Models

UMa-CDLC1: Derived based on CDL-C from 38.901 with the desired delay and angle spread applied, and truncated to 12 significant clusters.

Table 5.1.4.3-1: Channel model parameters for UMa-CDL-C1

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |