3GPP TSG-RAN WG4 Meeting #115	R4-2508754

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	BT

Title:	TP for TR 38.753 Section 6.2 MU-MIMO

Agenda item:	7.16.2

Document for:	Endorsement

Introduction

In RAN4#113 work split for TR 38.753 was agreed [1]. In this contribution we provide draft for TP on Section 6.2 MU-MIMO.

Text proposal

--------------------------------------------- Start of text proposal -----------------------------------------------

## Annex D: MU-MIMO (Informative)

The study of SCM for MU MIMO scenarios was part of the initial WID and interest in this aspect was voiced by a number of contributors, including operators. MU MIMO has not been part of the alignment or comparison discussions, however comments and proposals have been made, and discussions have been observed throughout the study.

The discussions highlighted two types of MU scenarios. Firstly, the legacy version with a single transmitter and a single receiver, hence necessitating creation of a single SU CDL channel and addition of MU interference via simultaneous transmission of two transport blocks using two precoders. Secondly, a version with a single transmitter and two receivers, hence necessitating creation of two single SU CDL channels (potentially with spatial consistency constraints on the both).

It was not concluded whether the "two receiver" version is useful and/or necessary for MU performance requirement derivation.

No agreement on SCM, implementation, alignment, usage in MU MIMO has been reached in this phase of SCM study.

Spatial channel models for MU-MIMO cases were evaluated based on the table Section 3 in [1].

| Parameter |  | MU-MIMO PDSCH (FR1) |  |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |
| UE receiver type |  | IRC, E-IRC |  |
| Number of Tx Ports |  | 4 |  |
| Number of Rx Ports/Antennas |  | 4 |  |
| Number of layers |  | 2+2 |  |
| PMI |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |  |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP |  |
|  | Codebook configuration | (N1,N2,O1,O2) = (2,1,4,1) |  |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | TBD |  |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |
| Channel Models |  | The purpose of the study item. |  |
| Testing Metric |  | Full Throughput Curves |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | (+45, -45) |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |
|  | UE Antenna Polarisation | (0, 90) |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

--------------------------------------------- End of text proposal -----------------------------------------------

- References

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.