3GPP TSG-RAN WG4 Meeting #115	R4-2508757

Malta, MT, 19th – 23rd May, 2025

Title: 	WF on demodulation and CSI requirements for 7MHz

Agenda Item:	7.8.1

Source: 	Huawei, HiSilicon

Document for:	Approval

1: UE demodulation and CSI requirements

1-1 PDSCH requirements

Whether to define PDSCH single carrier requirements for 7MHz bandwidth

Agreements:

- Don’t define 7MHz for PDSCH requirements for single carrier

Whether to define PDSCH CA requirments for 7MHz bandwidth

Agreements:

- Define normal PDSCH type A CA requirements for 7MHz

- Not to consider HST-SFN and HST-DPS scenario.

- Interested companies are welcome to bring results for 7MHz.

- FFS how to define requirements:

  - Option 1: Define new requirements for 7MHz CC based on the simulations

  - Option 2: Reuse existing 5MHz/10MHz requirements

Simulation assumptions for 7MHz CC for CA requirements

Agreements:

- Duplex mode: Only FDD

- Bandwidth/SCS: 7MHz/15kHz

- Number of RB for CORESET:30

- Number of RB: 35PRBs

- MCS: 13

- Rank: Rank2

- MCS: 16QAM,0.48

- Propagation conditions: TDLA30-10

- Antenna configuration: 2T2R, 2T4R ULA Low

- Other parameters: Reuse Table 5.2.1 in TS 38.101-4

SDR requirements

Agreements:

- Define SDR requirements for 7MHz bandwidth

  - For Table 5.5A-1 in TS 38.101-4, add the following in TS38.101-4 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”.

  - For Table 5.5A-4 in TS 38.101-4, set the number of CORESET PRBs to ‘30’.

1-2 CSI requirements

Whether to define CSI requirements for 7MHz for single carrier

Agreements:

- Don’t define CQI requirements for 7MHz for single carrier

Whether to define CSI requirements for 7MHz for CA

Agreements:

- Define CQI requirements for 7MHz for CA

Updates to current CA CQI test setup to cover 7MHz bandwidth (If agreed)

Agreements:

- Set CQI subbandSize to ‘8’ for 7MHz CBW in TS 38.101-4 Table 6.2A.3.1.1-2.

1-3 PDCCH and PBCH requirements

Whether to define PDCCH and PBCH requirments for 7MHz bandwidth

Agreements:

- Don’t define PDCCH and PBCH requirements for 7MHz

1-4 Release independence

Release independence for 7MHz bandwidth requirements

- Proposals

  - Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

Topic #2: BS demodulation requirements

Agreements:

- Don’t introduce BS demodulation requirements for 7MHz

- Don’t update applicability rules for BS supporting different bandwidth.

Topic #3: CR split

Companies are welcome to bring the CRs according to following CR splitting table next meeting.

| CR/simulation summary | Content | Company |
| --- | --- | --- |
| PDSCH requirements with CA for 7MHz | Capture the requirements table for 7MHz for both 2Rx and 4Rx. Capture the number of RB for CORESET in common parameter table | Nokia |
| FRC for 7MHz | Capture the 7MHz FRC | Apple |
| CQI and SDR requirements for 7MHz | Update the test setup for CQI and SDR requirements for 7MHz | CTC |
| Simulation summary for 7MHz | Spread sheet for simulation summary | ZTE |
| Big CR on introducing demodulation and CSI requirements for 7MHz | Big CR | Huawei |