3GPP TSG-RAN WG4 Meeting #116										 					R4-2509071

Bengaluru, 25th - 29th August, 2025

Agenda item:			7.28.1

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [116][228] IoT_NTN_Ph3

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) phase 3, including the following topics:

- Topic#1: RRM core requirements (AI 7.28.4)

- Topic#2: RRM performance requirements (AI 7.28.5)

Recommended issues for online discussion:

- Issue 1-1, 2-1

# Topic #1: RRM core requirements (AI 7.28.4)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2511575 | Nokia | - Confirm to RAN2 that there are no concerns to the optionality of TAC in the CB-MSG3 procedure. |
| R4-2510775 | Ericsson | Proposal 1: Agree draft LS text, as per appendix. As the remaining timing error is less than 97*Ts (TS 36.133 sections 7.20A and 7.24A) and it can be in negative or positive direction, to bring the error to an error less or equal to Rel-18 NTN in connected mode, it is sufficient to be able to signal TA commands in the range [-96:16:96] *Ts (or [-112:16:128] *Ts using 4 bits or reusing the 6 bit TAC in connected mode [-496:16:512]*Ts). Based on the analysis above RAN4 has no concern with regards to the agreements regarding TAC in MSG4 (i.e., CB-msg3 response). |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1: LS reply

#### Issue 1-1: LS reply for TAC in CB-Msg3-EDT procedure

Background:

- RAN2 LS in R2-2504776, LS on CB-msg3-EDT, for TAC in CB-Msg3-EDT procedure

Proposals:

- Proposal 1 (Nokia): Confirm to RAN2 that there are no concerns to the optionality of TAC in the CB-MSG3 procedure.

- Proposal 2 (Ericsson): Agree draft LS text in R4-2510775, as per appendix.

  - As the remaining timing error is less than 97*Ts (TS 36.133 sections 7.20A and 7.24A) and it can be in negative or positive direction, to bring the error to an error less or equal to Rel-18 NTN in connected mode, it is sufficient to be able to signal TA commands in the range [-96:16:96] *Ts (or [-112:16:128] *Ts using 4 bits or reusing the 6 bit TAC in connected mode [-496:16:512]*Ts).

  - Based on the analysis above RAN4 has no concern with regards to the agreements regarding TAC in MSG4 (i.e., CB-msg3 response).

Recommended WF:

- RAN4 has no concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4

- A separate email thread will be triggered to further discuss the details of the LS reply.

#### Issue 1-2: Core CR draft

| T-doc number | Company | Comment |
| --- | --- | --- |
| R4-2511284 | Nokia | NB RA |
| R4-2510038 | CMCC | M1 RA |
| R4-2511169 | ZTE Corporation, Sanechips | Add the HD-FDD band group for category M1 for satellite access. |

Recommended WF:

- A separate email thread will be triggered during the meeting to collect comments.

# Topic #2: RRM performance requirements (AI 7.28.5)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2510379 | Ericsson | Proposal 1: RAN4 to define: - A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT and - A.13.3.2.X2 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage for CB-msg3-EDT. |
| R4-2509225 | MediaTek inc. | Proposal 1: introduce NB-IoT test cases for CB-msg3-EDT for both normal and enhanced coverage. |
| R4-2510039 | CMCC | Proposal 1: Define CB-msg3 EDT test cases for all there cases above. Proposal 2: Set up 2 CE level in the test, add CB-Msg3 EDT configuration as defined in CB-Msg3-ConfigSIB-NB-r19 Proposal 3: In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN shall be met at first as defined in TS 36.331 5.3.3.1x. Proposal 4: In order to test correct UE behavior of replica transmission and re-attempt a CB-Msg3 transmission, the System Simulator shall transmit a CB-Msg4 over CB-RNTI after 2 CB-Msg3 transmission attempts |
| R4-2511279 | Nokia | Proposal 1: Include one test case for UEs in standalone mode in normal coverage for CB-MSG3-EDT. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 2: Test case for CB-Msg3-EDT procedure

#### Issue 2-1: Test cases list

Background: WF R4-2508289

Agreement:

- Define test cases for random access for CB-msg3-EDT, and the correct behaviour and transmit timing can be verified in the same test case.

- Further discuss whether to down-select the test cases:

| Random Access (A.13.3.2) | A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT | TC1 |
| --- | --- | --- |
|  | A.13.3.2.X2 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage for CB-msg3-EDT | TC2 |
|  | A.13.3.2.X3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage for CB-msg3-EDT | TC3 |

Proposals:

- Proposal 1: Define only TC1. (Nokia)

- Proposal 2: Define TC1 and TC2. (Ericsson, MTK)

- Proposal 3: Define TC1, TC2, and TC3. (CMCC)

Moderator’s comment: Since earlier RACH tests (without skipping Msg1/2) already verify the UE’s selection of enhanced vs. normal coverage and timing accuracy, including on the non‑anchor carrier, and this test target is to verify CB-Msg3-EDT procedure, e.g. replica transmissions and reattempts, we may not need to include all test cases.

Recommended WF:

- discuss whether Proposal 1 is acceptable.

#### Issue 2-2: Test procedure

Proposals:

- Proposal 1 (CMCC): Set up 2 CE level in the test, add CB-Msg3 EDT configuration as defined in CB-Msg3-ConfigSIB-NB-r19

- Proposal 2 (CMCC):  In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN shall be met at first as defined in TS 36.331 5.3.3.1x.

- Proposal 3 (CMCC):  In order to test correct UE behavior of replica transmission and re-attempt a CB-Msg3 transmission, the System Simulator shall transmit a CB-Msg4 over CB-RNTI after 2 CB-Msg3 transmission attempts.

Recommended WF:

- discuss whether proposals are acceptable.