3GPP TSG-RAN WG4 Meeting #116					 R4-2509199

Bengaluru, India, 25th - 19th August, 2025

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	7.1.2.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#115 there was a WF [1] to further study power domain enhancements for NR single carrier. In this paper we present additional views on this topic.

# 2. 	Discussion

In RAN4#115 there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier.

In the last meeting the following equations for the new inner RB region in extended CBW was taken as a starting point [1]:

#### Issue 1-1-3: Derivation of new inner RB region in extended CBW

- WF

- The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well

  - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

- Other approach is not precluded

- The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

Based on further analysis we determined that the equations for defining the inner region needs to be modified as follows:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

If the RBStart,Low_new and RBStart,High_new are substituted into the RBStart inequality then the following in obtained

max(0, (max(1, floor((LCRB)/2))- RBShiftlLow)) <= RBstart <= min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB), and LCRB  <= ceil((NRB+RBShift_low+RBShift_high )/2)

Based on the above equations the following proposal is made:

Proposal 1: Adopt the following equations for defining the inner region when extended CBW is allowed:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

After making substitutions into the above inequality the following is obtained for the inner region:

max(0, (max(1, floor((LCRB)/2))- RBShiftlLow)) <= RBstart <= min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB), and LCRB  <= ceil((NRB+RBShift_low+RBShift_high )/2)

When BW extenstion is allowed on one or both sides of the original CBW  the inner region becomes larger as indicated in figure 1 where the left figure illustrates the inner and outer regions corresponding to the original non-extended CBW and the right shows the inner and outer regions when the original CBW is extended on both sides. A comparision of the 2 plots shows that the inner region has grown by the red shaded region illustrated in the extended CBW figure.

Figure 1: Increase in inner region with BW extension

In release 18 power boosting was defined for the inner region for Pi/2 BPSK and QPSK signals. Based on the capabilites powerBoosting-pi2BPSK-QPSK-r18 and powerBoosting-pi2BPSK-QPSK-Modified-r18 powerboost is possible in both the enhanced inner region as defined in [2] and the region that encompasses the inner region but is outside the enhanced inner region.

In our opinion the power boost feature defined in release 18 can be naturally extended to the release 19 MPR reduction feature where the inner region is expanded by BW expansion. Using the Rel-18 power boost capability in the expanded inner region benefits overall system performance by enabling higher transmit powers for inner RB allocations.

Our simulations indicate that both with and without FDSS a power boost of at least 1 dB is possible in the inner and extended inner regions with and without bandwidth expansion. Figure 2 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS in the absence of bandwidth expansion. This represents the rel-18 baseline power boost scenario.

Figure 2 – Rel-18 baseline power boost scenario

Figure 3 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion on the lower side of the spectrum.

Figure 3 –  Power boost with and without FDSS with BW expansion on lower side of spectrum

Figure 4 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion on the upper side of the spectrum.

Figure 4 –  Power boost with and without FDSS with BW expansion on upper side of spectrum

Figure 5 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion on both sides of the spectrum.

Figure 5 –  Power boost with and without FDSS with BW expansion on both sides of spectrum

Comparing figure2 with figures 3, 4 and 5 we see that the 1dB power boost that was agreed in rel-18 for the inner and enhanced inner regions is also possible for the expanded inner and expanded enhanced inner regions created by bandwidth expansion.

Proposal 2: For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced  inner regions as in the original inner and enhanced inner regions.

We propose that if a UE supports MPR reduction through bandwidth expansion then the enhanced inner allocation region within the inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

Proposal 3: If a UE supports MPR reduction through bandwidth expansion then the expanded enhanced power inner allocation region within the expanded inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

In RAN4#115 companies could not agree on whether IBE should be applied in the guardband of the extended CBW. Currently, even in the non-extended CBW case the IBE is not applied in the original guardband so we see no reason to apply it in the guardband of the extended CBW.

Proposal 4: No need to apply IBE in the guardband of the extended CBW

In the WID: UE RF enhancements for NR FR1/FR2 and EN-DC, phase 4 [3] power boost and MPR reduction was identified as one of the main areas of study in the justification section and one of the main study areas captured in the objectives section as shown below:

3	Justification

This work item targets at the UE RF requirement enhancements for the deployment using both NR frequency range 1 (FR1) and frequency range 2 (FR2). The following working areas are included.

- High power UE (HPUE) for CA in terrestrial network (TN) (including 3Tx)

- Power boosting and/or MPR reduction

- 6 Rx for handheld and FWA UE

Power boosting and/or MPR reduction

This area is made up of two parts: power boosting or MPR reduction for single carrier for PC2 and power class 3 (PC3), and MPR reduction for intra-band UL CA configurations.

Firstly, in the Rel-18 coverage enhancement WI, the power boosting and/or MPR reduction for PC2 and PC3 with QPSK were specified, which is mainly for the inner region of a single UL carrier. The further power enhancement is restricted by out-of-band emission requirements, e.g., ACLR requirement. It is observed that emission requirements could be relaxed under the conditions where no co-existence issue is caused especially for the deployment of two adjacent frequency blocks. Thus there is additional room to enhance the Tx power further.

4.1	Objective

Power boosting and/or MPR reduction

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA

  - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier

    - Include the following scenarios:

      - when there is no adjacent in-band/out-of-band co-existence issue

      - when a UE uses a narrower channel bandwidth within a wider BS bandwidth

    - Include both (e)RedCap UE (only PC3) and non-RedCap UE

    - Limited to QSPK and 16QAM

  - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration

    - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1

    - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2

    - MPR requirement is not applicable until the SCell is activated

  - Necessary signaling to support the above objectives

As stated in the justification section of the above WID further enhancement of the Tx power feature introduced for inner regions in Rel-18 was seen as a major motivation for this work. Tx power enhancement can be either an increase in Tx power and/or an enlargement of the MPR region in which the enhancement can be applied. In this study MPR reduction took on two forms. The first being for intra-band CA where single CC MPR values are adopted when a single cell is activated. The second is by BW expansion which allowed the spectrum adjacent to the UE UL to have relaxed emission requirements thereby reducing the MPR requirements.

In RAN4#112-bis the following agreement for intra-band contiguous CA with single activated cell as detailed below was made [4] which states that if the UE supports capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 Rel-18 then power boost can be supported for intra-band contiguous CA with single activated cell. This enables the single activated cell to transmit at higher powers in the inner MPR region.

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell

- WF

  - For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

    - The Rel-18 power boosting feature can be supported depending on UE capability

      - Feature group description shall clarify the interaction between Rel-18 power boosting capabilities and Rel-19 MPR applicability enhancement capability.

  - FFS PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet

Similarly, we see that if a UE supports MPR reduction by means of BW expansion and it also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 then the boosting capabilities apply to the entire expanded inner MPR region which has been enlarged due to BW expansion.

Based on the above we propose the following clarifications to the NR feature list as detailed below. Only a subset of the feature list has been shown below so as to fit the table into this document. The complete table can be found in R4-2510117.

;

| Features | Index | Feature group | Components | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applicable to the activated CC | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per FS | FR1 only | Optional with capability signaling |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | [Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 [{1/4, …}].] If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |

Proposal 5:   If a UE supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 then to enable these features for intra-band contiguous CA with a single activated cell the following changes need to be made to the NR feature list.

| Features | Index | Feature group | Components | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applicable to the activated CC | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per FS | FR1 only | Optional with capability signaling |

Proposal 6:   If a UE supports MPR reduction by means of BW expansion and supports capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified then to enable these capabilities to be used in the entire expanded inner and expanded enhanced inner MPR regions the following changes needs to be made to the NR feature list.

| Features | Index | Feature group | Components | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | [Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 [{1/4, …}].] If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: Adopt the following equations for defining the inner region when extended CBW is allowed:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

After making substitutions into the above inequality the following is obtained for the inner region:

max(0, (max(1, floor((LCRB)/2))- RBShiftlLow)) <= RBstart <= min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB), and LCRB  <= ceil((NRB+RBShift_low+RBShift_high )/2)

Proposal 2: For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced  inner regions as in the original inner and enhanced inner regions.

Proposal 3: If a UE supports MPR reduction through bandwidth expansion then the expanded enhanced power inner allocation region within the expanded inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

Proposal 4: No need to apply IBE in the guardband of the extended CBW

Proposal 5:   If a UE supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 then to enable these features for intra-band contiguous CA with a single activated cell the following changes need to be made to the NR feature list.

| Features | Index | Feature group | Components | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applicable to the activated CC | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per FS | FR1 only | Optional with capability signaling |

Proposal 6:   If a UE supports MPR reduction by means of BW expansion and supports capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified then to enable these capabilities to be used in the entire expanded inner and expanded enhanced inner MPR regions the following changes needs to be made to the NR feature list.

| Features | Index | Feature group | Components | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | [Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 [{1/4, …}].] If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | FR1 only | Optional with capability signaling |

# References

[1] R4-2507930, WF on NR_ENDC_RF_Ph4_part1, RAN4#115, May 19th – May 23rd, 2025

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0

[3] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, March 8-21, 2024

[4] R4-2417204, WF on power domain enhancements, RAN4#112-bis, October 14th – October 18th, 2024