3GPP TSG-RAN WG4 Meeting #116	R4-2509429

Bengaluru, India, August 25th – 29th, 2025

Source:	Apple

Title:	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN#107, the revised SID of the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [1]. In this revised SID, the following scope was agreed for RAN4:

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In this contribution, we share our views on the following topics:

- Relation to legacy requirements

- LCM-related aspects for two-sided model case, including relation to LCM of UE-sided models and performance monitoring

Discussion

Relation to legacy requirements

The following agreement was made in RAN4#112bis regarding the relation to legacy requirements for CSI prediction and beam management use cases.

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

As 3GPP RAN4 progresses the specification of performance requirements for AI/ML-based functionalities in NR air interface (e.g., CSI prediction, CSI compression, and beam management), a key challenge is determining how these new AI/ML-based approaches relate to existing legacy (non-AI/ML) performance requirements.

#### 1. Baseline Expectation: No Worse Than Legacy

An agreement was made in RAN4#112bis that AI/ML-based performance should not be worse than legacy implementations where a relevant equivalent legacy feature exists. However, this immediately raises several follow-up issues:

- Which legacy requirements (e.g., defined in TS 38.101-4) are applicable as a reference baseline for each AI/ML use case?

- Should AI/ML performance targets only match legacy performance, or should they exceed it to justify the additional complexity and possible gain from reduced measurement overhead?

- What is the appropriate target performance metric for beam management under AI/ML?

#### 2. Use of γ Ratio as a Legacy Metric

For CSI prediction, the γ ratio, originally defined for PMI reporting in TS 38.101-4, has been selected as a candidate reference metric. This ratio compares the throughput achieved using the UE-reported precoder to that obtained using random precoding under the same SNR:

- t_ue_AI/ML = throughput using UE-reported AI/ML-based precoder

- t_rnd = throughput using Rel-15 single-panel random precoder

- γ = t_ue_AI/ML / t_rnd

This definition gives a normalized indication of how much gain is achieved relative to random precoding, and aligns with well-understood legacy testing principles.

During RAN4#115 it was agreed that the performance of AI based CSI prediction should not be worse than R16 etypeII codebook.

In evaluating AI/ML-based functionalities such as CSI prediction, beam management, or CSI compression, it is essential that performance metrics account not only for absolute accuracy (e.g., throughput gain or error reduction), but also for the overall system efficiency, particularly the ability of AI/ML to reduce signaling, measurement burden, or latency.

A core benefit of AI/ML approaches lies in their ability to infer or compress information, potentially allowing the system to operate with:

- Fewer downlink CSI-RS resources

- Reduced uplink reporting

- Less frequent measurement triggering or active probing

However, this reduction in overhead may come at the cost of a slight degradation in prediction accuracy or throughput, particularly in challenging or rapidly changing radio environments.(generalization related)

If strict legacy-level performance (e.g., same gamma ratio as PMI) is mandated without considering the associated savings in measurement or signaling overhead, the AI/ML feature may appear to underperform, even when it brings system-level efficiency improvements.

It can be argued that it is insufficient to evaluate AI/ML model performance in isolation. The tradeoff between performance and measurement overhead is intrinsic to the AI/ML value proposition. RAN4 requirements should be designed to recognize this balance, particularly in use cases like beam prediction  where legacy procedures are known to be measurement intensive.

RAN4 is invited to discuss a composite performance metric for AI/ML-based functionalities that jointly captures both prediction accuracy (e.g., throughput gain or beam selection accuracy) and system efficiency (e.g., reduction in measurement or reporting overhead). Such a composite metric would better reflect the true benefits of AI/ML approaches by acknowledging trade-offs between slight degradations in accuracy and significant gains in control resource savings

Life-Cycle Management related aspects for two-sided models

Relation to LCM of UE-sided models and performance monitoring

While many aspects of UE-sided LCM, such as activation, deactivation, and fallback timing are already being discussed in RAN4 for one-sided models, two-sided deployments bring the additional complexity of distributed inference roles, where inference may be jointly performed by UE and network. This raises the question of:

- Can the existing LCM framework for UE-sided models be reused for the UE part of two-sided models without modification?

- How should synchronization and coordination of model state (active/inactive, valid/invalid) between UE and gNB be managed, especially when model updates or failures occur independently at one side?

If not carefully defined, discrepancies in model state between the UE and gNB could lead to misaligned decoding, degraded CSI reconstruction, or invalid assumptions about inference quality.

According to TR 38.843, performance monitoring in CSI compression using two-sided model use case can be NW-side or UE-side.

- NW-side performance monitoring: NW monitors the performance and make decisions of model/functionality activation/ deactivation/updating/switching. Impact to enable performance monitoring using an existing CSI feedback scheme as the reference, including the association between AI/ML scheme and existing CSI feedback scheme for monitoring, are considered. Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model/functionality activation/deactivation/updating/switching. Impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE, are not precluded.

Below the agreements related to performance monitoring in CSI compression from several RAN1 meetings are shown:

RAN1#117 Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. RAN1#120bis Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. RAN1#120bis Conclusion UE-side monitoring is feasible. RAN1#120bis Observation - Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

Two-sided CSI compression introduces two performance monitoring domains:

- NW-side monitoring: The network monitors CSI quality based on UE feedback, possibly using legacy eType-II codebooks or SRS-based approaches.

- UE-side monitoring: The UE evaluates the quality of its own CSI encoder using internal outputs (e.g., intermediate KPIs like SGCS, reconstructed channel estimates, or proxy models).

Each approach has its distinct advantages and constraints:

- NW-side monitoring leverages existing legacy CSI feedback formats but may lack insight into internal model confidence or intermediate encoder behaviors.

- UE-side monitoring can offer faster and localized detection of anomalies but may require new signaling formats and delay budgets for reporting.

Moreover, hybrid proposals suggest combining both, but this introduces new interoperability issues. Hybrid performance monitoring schemes, where both the UE and gNB independently assess the performance of two-sided AI/ML models introduce new interoperability challenges that do not arise in legacy, one-sided procedures. A primary concern is how to handle conflicting monitoring results. For instance, the UE may report acceptable performance based on local metrics (e.g., intermediate KPIs like SGCS or reconstruction quality), while the network, relying on decoded CSI reports or SRS-based observations, may detect a performance degradation. Without a coordinated resolution mechanism, this discrepancy could lead to inconsistent behavior: either unnecessary model fallback triggered by the UE or delayed corrective action by the network. Therefore, RAN4 must consider whether to define precedence rules (e.g., gNB’s assessment overrides in certain conditions) or signaling exchanges to reconcile and synchronize decisions between the two ends.

These unresolved issues necessitate:

- Clear definition of performance metrics for each monitoring mode (e.g., intermediate KPI, SGCS, reconstruction error).

- Case-by-case delay requirement analysis, depending on whether monitoring is periodic, aperiodic, or event-triggered.

In NW-side monitoring scenarios, the UE sends legacy-style CSI reports (e.g., eType-II), and thus, existing delay requirements may apply. However, for UE-side monitoring, where the UE sends AI-specific metrics or intermediate KPIs, there is no direct legacy equivalent, and new delay requirements must be defined by RAN4 to:

- Ensure timely fallback or reconfiguration actions

- Avoid unnecessary model switching or deactivation due to stale monitoring results

Without such definitions, performance monitoring for AI/ML-based two-sided CSI compression may lack consistency and fail to provide the reliability needed for robust deployment.

RAN4 to study the applicability of the existing LCM framework defined for UE-sided AI/ML models to the UE component of two-sided AI/ML models. This includes evaluating whether the activation, deactivation, switching, and fallback procedures defined for single-ended models can be reused without modification, or whether additional signalling and synchronization are needed between UE and gNB to maintain consistent inference behaviour.

RAN4 to define performance monitoring requirements for two-sided CSI compression use cases, separately for UE-side monitoring and NW-side monitoring. For NW-side monitoring, legacy delay requirements may be reused, as the UE reports CSI via legacy codebooks. For UE-side monitoring, new delay requirements should be studied, reflecting the non-legacy nature of the reported metrics (e.g., intermediate KPIs such as SGCS or reconstruction error). The study should also identify applicable reporting modes (periodic, semi-persistent, or aperiodic) and ensure that the specified delay budgets support timely activation or fallback decisions based on monitoring results.

Conclusion

The following proposals were made in this contribution:

- RAN4 is invited to discuss a composite performance metric for AI/ML-based functionalities that jointly captures both prediction accuracy (e.g., throughput gain or beam selection accuracy) and system efficiency (e.g., reduction in measurement or reporting overhead). Such a composite metric would better reflect the true benefits of AI/ML approaches by acknowledging trade-offs between slight degradations in accuracy and significant gains in control resource savings

- RAN4 to study the applicability of the existing LCM framework defined for UE-sided AI/ML models to the UE component of two-sided AI/ML models. This includes evaluating whether the activation, deactivation, switching, and fallback procedures defined for single-ended models can be reused without modification, or whether additional signalling and synchronization are needed between UE and gNB to maintain consistent inference behaviour

- RAN4 to define performance monitoring requirements for two-sided CSI compression use cases, separately for UE-side monitoring and NW-side monitoring. For NW-side monitoring, legacy delay requirements may be reused, as the UE reports CSI via legacy codebooks. For UE-side monitoring, new delay requirements should be studied, reflecting the non-legacy nature of the reported metrics (e.g., intermediate KPIs such as SGCS or reconstruction error). The study should also identify applicable reporting modes (periodic, semi-persistent, or aperiodic) and ensure that the specified delay budgets support timely activation or fallback decisions based on monitoring results

- References

- RP-250308, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.