3GPP TSG-RAN4 Meeting  #116	R4-2509437

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.755 | CR | 0002 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TR maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_FR1_DL_Frag_Carrier |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in R4-2509436, several aspects in Clause 8.2 requires further clarificaiton or wording improvement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General wording improvements - Pointing out that Scenario 3a may not be allowed in current specification and thus would imply specification impact in future releases to support it - Furhter clarifying Network-initiated and UE-initiated transition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity in the TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

## 8.2	CA fallback and architecture switching

When either the in-gap interference or the self-interference is too high, the receiver is hard to maintain the “one Rx RF chain” mode, and the “fallback” behaviour needs to be studied. In order to make the application of this function more future-oriented, some higher-order CA combinations are also considered.

In total, two aspects should be involved when discussing fallback behaviour: “CA fallback” and “architecture switching”.

“CA fallback” means releasing at least one SCell in one band of the band combination, while the “architecture switching” means the switching from “One Rx RF chain (aka. fully shared Rx RF chain)” to legacy CA architecture  [44], where legacy CA architecture includes fully separated Rx chain and partially shared Rx chain.When there is only one component carrier in one band, it is natural that UE RF hardware is also configured to legacy architecture for single carrier in the band.

When categorizing “CA fallback”, the following scenarios are involved:

a.) Legacy CA combination falls back to lower order CA

-	Legacy CA combination belongs to mix of inter-band CA and intra-band CA with more than 2CCs, etc.

b.) Legacy CA combination falls back to “non-CA” mode

-	Legacy CA combination belongs to intra-band CA combination with 2CCs.

When categorizing “architecture switching”, the following scenarios are involved:

a.) Architecture switches between “partially shared Rx RF chain” mode and “one Rx RF chain” mode

b.) Architecture switches between “fully separate Rx RF chain” mode and “one Rx RF chain” mode

Based on the analysis above, the fallback behaviour should be divided into the following three scenarios:

a.) Scenario 1: CA fallback without architecture switching

b.) Scenario 2: CA fallback with architecture switching

c.) Scenario 3: architecture switching without CA fallback

Below Figure 8.2-1 is an example to illustrate how the UE’s work mode transfers from “one Rx RF chain” to scenarios 1~3. The figure is generated based on Figure 3 in [45] and Figure 1 in [44].

Figure 8.2-1: Fall-back behaviour, scenarios 1~3

For scenario 1, it does not require additional UE capability or network configuration, assuming normal CA reconfiguration from the network to enable the fallback is still required. As a default, any UE supporting “one Rx RF chain” supports Scenario 1 as the legacy CA fallback configuration. From network point of view, if the performance on CC2 is not at satisfactory level, the network could just release the CC2 using existing CA operation process. Based on the evaluation result in section 7, the performance loss of “one Rx RF chain” is comparable with that of the other CA configurations due to interference such as UL harmonic, intermodulation, limited cross band isolation, etc. in the RAN4 specification. It is therefore feasible to handle the CA configuration with “one Rx RF chain” as a normal CA configuration. It is also beneficial for early deployment of the fragmented carrier feature without requiring upgrading the network. On the other hand, as [46] pointed out, in some cases Scenario 1 might not be the best choice from network’s point of view if CC2 has larger bandwidth than CC3, or CC2 could achieve better channel quality using legacy CA architecture than CC3. Scenario 2 is thus considered.

For Scenario 2, it keeps CC2 and releases CC3 if the network decides CC2 could provide what the network expects with better performance than CC3. This scenario requires architecture switching, which means the RF chains previously used by CC3 need to be retuned to support CC2 in the case where all the Rx chains are used up. However, whether the RF chains for different bands could be flexibly switched across the bands depends on UE implementation and is band combination specific, which means such info as UE capability may need to be indicated to the network. Whether and how such information is indicated to network is for further study. Scenario 2 would be useful especially when the performance of CC2(CC1) could be improved a lot when switching back to legacy CA architecture, such as when there is serious Tx leakage, or the in-gap interference is quite high considering the non-collocated scenario. Additional UE measurement might be needed to assist the network to compare the performance of CC2 and CC3.

For Scenario 3, it does not reduce the number of CCs, but requires architecture switching with reduced Rxs or DL MIMO layers (Scenario 3a) or the same number of Rx/DL MIMO layers (Scenario 3b). It is worth noting that Scenario 3a may not be allowed in current specification and thus would imply specification impact in future releases to support it. Similar to Scenario 2, whether the switching is feasible depends on UE implementation and is band combination specific, and whether and how such information is indicated to network is for further study. Scenario 3b assumes there are enough spare Rx chains. [47] mentions the benefit of adopting “one Rx RF Chain” is not very clear from the RF performance aspect if there are spare Rx chains. On the other hand, the UE might get the benefit of power saving from “one Rx RF Chain” if the relevant conditions could ensure a satisfactory Rx performance.

It needs to be considered whether the transition from “one Rx RF Chain” to Scenario 1~3 is “network initiated” or “UE initiated”. “Network initiated” means the network sends a command to the UE to trigger the transition, with or without assistant indication or measurement report from the UE. “UE initiated” means the UE could execute the transition without receiving the network’s command. UE may inform the network after the transition happened. Network-initiated transition may or may not have standards impact, and UE-initiated transition is expected to have standards impact. Note that current CA framework is mainly based on the “UE-assisted, network-initiated” model. Furthermore, UE-initiated transition may lead to some additional benefits. For example, with the current “UE-assisted, network-initiated” model, any SCell deactivation/release from the network may take tens of milliseconds or longer, resulting in prolonged link quality degradation and packet losses. In this case, UE-initiated SCell deactivation/release allows the UE in one Rx RF chain mode to quickly deal with link quality degradation.

-	Scenario 1, 2 and 3a involve the change of number of CCs or number of DL MIMO layers, which has to be configured by the network, i.e. ‘network initiated’.

-	For Scenario 2, the network configuration might explicitly indicate the architecture switching, or the switching could be done by the UE autonomously.

-	Scenario 3b could be transparent to the network if the UE has spare Rx chains, although some interruption is needed. The network could also intentionally configure Scenario 3b. Therefore both ‘network initiated’ or ‘UE initiated’ could apply to Scenario 3b.

More details on how the UE’s work mode switching from separate mode to “one RX RF chain” and falling back to scenarios 1~3 are achieved are as follows:

# <End of Change #1>