# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509451

# Bangalore, India, 25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR of interruptions for OD-SSB transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-07-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 agreed to introduce interruption requirements for OD-SSB operation. Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.. |
| Summary of change: |  | - Introduce interruption requirements for OD-SSB operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements for OD-SSB operation would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.2.1.2.x, (new) 8.2.2.2.x, (new) 8.2.3.2.x, (new) 8.2.4.2.x, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

##### 8.2.1.2.x	Interruptions at OD-SSB activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When OD-SSB on one deactivated SCell in SCG is activated or deactivated, the UE is allowed an interruption on any active serving cell in SCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slots, if the active serving cells are contiguous to any of the SCells on which the OD-SSB is being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

-	of up to Y2 slots if the active serving cells are in the same FR2 band as any of the SCells on which the OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

Where X2 and Y2 are specified in table 8.2.1.2.x-1.

Table 8.2.1.2.x-1: Interruption length X2 and Y2 at OD-SSB activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

# <End of Change 1>

# <Start of Change 2>

##### 8.2.2.2.x	Interruptions at OD-SSB activation/deactivation

When OD-SSB is activated or deactivated on a deactivated SCell, the UE is allowed an interruption on any active serving cell, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slots, if the active serving cell is non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

Where X2 is specified in table 8.2.2.2.x-1.

or

-	of up to the duration shown in table 8.2.2.2.x-2, if the active serving cells are contiguous to any of the SCells on which the OD-SSB is being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.x-2, if the active serving cells are in the same FR2 band as any of the SCells on which the OD-SSB is being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

Table 8.2.2.2.x-1: Interruption length X2 for OD-SSB activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 8.2.2.2.x-2: Interruption duration for OD-SSB activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 4 |
| 5 | 0.03125 | 16 |
| 6 | 0.015625 | 32 |

# <End of Change 2>

# <Start of Change 3>

##### 8.2.3.2.x	Interruptions at OD-SSB activation/deactivation

The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell.

When OD-SSB on one deactivated SCell in MCG is activated or deactivated the UE is allowed an interruption on any serving cell in MCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, or

-	of up to Y2 slots + TSMTC_duration if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot.

Where X2 and Y2 are specified in table 8.2.3.2.x-1.

Table 8.2.3.2.x-1: Interruption length X2 and Y2 at OD-SSB activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

# <End of Change 3>

# <Start of Change 4>

##### 8.2.4.2.x	Interruptions at OD-SSB activation/deactivation

When OD-SSB is activated or deactivated on a deactivated SCell, the UE is allowed an interruption on any active serving cell, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to the duration shown in table 8.2.4.2.x-1, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, where the requirements for Sync apply for synchronous NR-DC, and for asynchronous NR-DC if the active serving cell is in the same CG as all the SCells on which OD-SSB is being activated, and the requirements for Async apply for asynchronous NR-DC if the active serving cell is not in the same CG as any of the SCells on which OD-SSB is being activated, or

-	of up to the duration shown in table 8.2.4.2.x-2, if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot.

Table 8.2.4.2.x-1: Interruption duration for OD-SSB activation/deactivation for inter-band DC/CA

|  | NR Slot length | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 1 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 5 |
|  |  | Aggressor cell is on FR1 | 5 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 | 33 |

Table 8.2.4.2.x-2: Interruption duration for OD-SSB activation/deactivation for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 4 |
| 5 | 0.03125 | 16 |
| 6 | 0.015625 | 32 |

# <End of Change 4>