# 3GPP TSG-RAN WG4 Meeting #116	R4-2509501

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.27.5.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#115, and the open issues have been summarized in the topic summary for [115][227] NR_NTN_Ph3_Part1 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Sub-topic 1-2 SMTCs and 'in parallel' related features

Issue 1-2-2: SMTC adjustment in idle/inactive

<Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

In current spec, to reflect the SMTC shifting at UE, it was specified that “The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.” RAN4 could resue the same requirement applicability definition for the (e)RedCap NTN case for simplicity.

Proposal 1: Keep the same applicability definition used in NR NTN for RedCap UE: The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

Sub-topic 1-3 The impact of HD-FDD

Issue 1-3-3: The impact of HD-FDD on scheduling availability of UE related to CSI-RS and PRS

online agreement < Agreement >: - When NW indicates UL overriding DL for collision handling, - RAN4 to clarify that the measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission. - Otherwise, - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission. - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

In last meeting, RAN1 agreed the CR(R1-2504975) for HD-FDD RedCap UE in NTN, and the principle for collision handling is as following:

A HD-UE that operates on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate CollisionHandlingOfHDFDDOperation, does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS in a set of symbols based on an indication by a first DCI format, and is scheduled to transmit a PUSCH, PUCCH, SRS, or PRACH that overlap with any symbol from the set of symbols based on an indication by a second DCI format, determines - to either receive the PDSCH or the CSI-RS or transmit the PRACH based on the HD-UE implementation - to either receive the PDSCH or transmit the PUSCH, PUCCH, or SRS based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set - to receive the PDSCH or the CSI-RS if the first DCI format is provided by a PDCCH the HD-UE received according to a search space set other than a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntnRedCapCollisionCase4UlPriority, and timeline conditions for cancellation of the UL transmission as defined in clause 17.2 are satisfied - to transmit the PUSCH, PUCCH, or SRS, if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and - the HD-UE is provided ntnRedCapCollisionCase4UlPriority, or - the HD-UE is not provided ntnRedCapCollisionCase4UlPriority and timeline conditions for cancellation of the UL transmission as defined in clause 17.2 are not satisfied

It seems RAN4 agreement in “otherwise” is conflicting with RAN1 agreement in the CR, and therefore we propose to add more RAN4 clarification to comply with RAN1 CR. When UE indicates CollisionHandlingOfHDFDDOperation, to either receive the PDSCH or the CSI-RS or transmit the PRACH is based on the HD-UE implementation, and therefore the requirement for CSI-RS, scheduling restriction of PDSCH, and PRACH shall only apply when no collision happens between CSI-RS/PDSCH and PRACH.

Proposal 2: When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH.

Sub-topic 1-4 UE capability

Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN

<Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

In our view, separated capability of RedCap and NTN has already specified in the spec, and UE can indicate both of them or one of them to let network understand whether or not RedCap UE can work in NTN system. Option 1 is the best way to express the UE capability. However, we don’t think R17 RedCap UE can support NTN operations by default, e.g., the newly introduced collision handling for HD-FDD UE in NTN is a blend new design which shall not be any release independent feature.

Proposal 3: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: Keep the same applicability definition used in NR NTN for RedCap UE: The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

Proposal 2: When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH.

Proposal 3: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

# References

[1] R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, RAN4#115

[2] R4-2505576, Topic summary for [115][227] NR_NTN_Ph3_Part1, CATT, RAN4#115