3GPP TSG-RAN4 Meeting #116	R4-2509617

# Bengaluru, India, 25-29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Introducing L1 CLI measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft CR introduces L1 CLI measurement requirements in TS 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the measurement requirements for L1-SRS-RSRP and L1-CLI-RSSI, it contains the changes endorsed in R4-2508459 and introduces new changes such as removal of brackets and alignment of terms with the latest RAN2 specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements for SBFD are missing. There are no requirements for L1 CLI measurements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.X new clause, with title: L1 Cross Link Interference measurements |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a resubmission of R4-2508295 with some additional change |  |  |  |  |  |  |  |  |

<Start of Change >

## 9.x	L1 Cross Link Interference measurements

### 9.x.1	Introduction

The UE capable of performing L1 CLI measurements shall be able to measure L1-SRS-RSRP and L1-CLI-RSSI which are defined in TS 38.215 [4] within the active DL BWP. The measurements requirements in this clause apply for TDD mode only.

The L1 CLI measurement requirements specified in this clause are applicable only for RRC_CONNECTED intra-frequency:

-	when L1-SRS-RSRP measurement resource is configured within active DL BWP

-	when L1-CLI-RSSI measurement resource is configured within active DL BWP

The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SRS-RSRP and L1-CLI-RSSI measurements within the active BWP.

For performing CLI measurements in FR2, UE can optionally be configured with CLI measurement resources containing an explicit indication of a TCI state providing a TypeD QCL source [26, TS 38.214]. If the TCI state is not configured, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

### 9.x.2	L1-SRS-RSRP measurements

#### 9.x.2.1	Introduction

When configured by the network, the UE shall be able to perform L1-SRS-RSRP measurements on the SRS-RSRP-MeasurementResourceSet configured in a CSI-ResourceConfig.

When the UE measures L1-SRS-RSRP, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

#### 9.x.2.2	Requirements applicability

The requirements in clause 9.x.2 apply, provided:

-	SRS resources configured for L1-SRS-RSRP measurements are measurable.

An SRS resource configured for L1-SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

-	L1-SRS-RSRP related side conditions given in clauses [10.1.22.1] for FR1 and FR2 for a corresponding band,

-	SRS_RP and SRS Ês/Iot according to Annex [B.2.7] for a corresponding band.

#### 9.x.2.3	Measurement Reporting Requirements

The UE shall send L1-SRS-RSRP reports only for report configurations configured for the active DL BWP,

The UE shall report the L1-SRS-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1 dB step size according to clause 10.1.X for FR1 and 10.1.Y for FR2, if nrofReportedCLImeasureResource is configured to one. If  nrofReportedCLImeasureResource is configured to be larger than one, the UE shall use differential L1-SRS-RSRP based reporting as defined in clause 10.1.X for FR1 and 10.1.Y for FR2. The differential L1-SRS-RSRP is quantized to a 4-bit value with 2 dB step size. The mapping between the reported L1-SRS-RSRP value and the measured quantity is described in clause 10.1.X.

##### 9.x.2.3.1	Aperiodic Reporting

Reported L1-SRS-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-SRS-RSRP reports shall meet the requirements in clauses [10.1.X] for FR1 and [10.1.Y] for FR2, respectively.

The UE shall only send aperiodic L1-SRS-RSRP measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-SRS-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

#### 9.x.2.4	Measurement capability

The UE shall be capable of performing L1-SRS-RSRP measurements on the SRS resources configured for measurement, provided that no more than 8 SRS resources are configured in a slot and that the total number of SRSs to be monitored by the UE does not exceed 32.

<End of Change >