# 3GPP TSG-RAN WG4 Meeting #116	R4-2509716			R4-250xxxx

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.5

Source:	CMCC

Title:	Discussion on OTA test method for A-IoT device

Document for:	Approval

1	Introduction

In RAN#106 meeting, a new WI Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. following list the objective of this OTA test part.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

- In this paper, we will focus on the following issues for OTA testing.

2	Discussion

- Last meeting agreements for test setup are listed as below, main remaining issues focus on how to ensure the isolation between CW node and measurement antenna.

- Topic 4-2: Test setup Issue 4-2-2: Isolation between CW node and measurement antenna Agreement in RAN4#115: - Power difference between received backscatter signal and the CW signal leakage is at least [15dB] - FFS on reference point of power difference

- Isolation between CW node and measurement antenna

- It’s suggested to find out the motivation of the isolation. The common understanding is that the CW phase noise is one of the key factor that will impact final reader REFSENSE performance. For that reason, we don’t suggest to use signal generator to generate CW signal, instead, the real CW nodes are suggested for testing. For the REFSENSE testing, the CW power level falling into measurement antenna should be the side condition and be defined in core requirements.

- So our suggestion is that above isolation is used to avoid impact from CW signal to device’s transmit requirements in testing. Device Tx requirements include output power, unwanted emission and spurious requirements. For the output power it is within the D2R bandwidth where the CW wanted signal and phase noise works. For the uwnanted emission and spurious requirements it is outside the D2R bandwidth, basicly the CW leakage works. For CW wanted signal, it is much easier to be cancelled, e.g. using filter to filter out. Besides, the CW signal will not mixed with device signal in frequency domain, which means the CW signal will not impact device Tx requirement testing. The key factor that impact device’s Tx requirements are CW phase noise, CW unwanted emission performance. For these two factors, they will be mixed with device’s wanted signal in frequency domain, and are hard to be cancelled.

- To avoide CW impact, following suggestions are suggested.

- Proposal 1: Power difference between received backscatter signal and the CW signal leakage is at least 15dB to avoid the impact of CW signal to device Tx requirements during testing. For device Rx requirement testing, CW input level should be the side condition defined together with reader’s REFSENSE requirements.

- Test setup

- As for the relative positions between testing antenna and CW antenna, last meeting TE vendors suggest to reuse current UE test setup to avoide design new test setup for device. For such case, CW and measurement antenna are suggested to be co-located and using cross polarization respectively for each other. For this case the isolation is not enough just using spatial isolation, instead, isolation material is needed, for which case it’s suggested to not rotate the CW/measurement antenna, instead, fixed CW/measurement antenna by rotating device only.

- Proposal 2: it’s suggested to use figure 7.2-1 as the baseline with fixed co-located testing and CW antenna while rotating device for 3D testing.

Figure 7.2-1: Example of a FR1 TRP TRS OTA test system with combined axis

- Another issue that is not discussed before is whether to test the isolation between CW and testing antenna before formal testing. This is necessary to avoid CW impact this could be done after calibration procedure. Therefore we updae the testing procedure as below.

- Test procedure

Test procedure for Tx RF requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Step 3: Place DUT based on UE positioning guidelines.

- Step 4: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 5: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

- Step 6: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 7: Move to next measurement point and repeat Step 4-6 until complete all the testing point in the measurement grid.

- Step 8: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

- Step 9: FFS on performance metric. FFS on how to specify the performance metric.

Test procedure for receiver RF requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Step 3: Place DUT based on UE positioning guidelines.

- Step 4: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 5: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 6: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 7: Move to next measurement point and repeat Step 4-6 until complete all the testing point in the measurement grid.

- Step 8: FFS on performance metric. FFS on how to specify the performance metric.

- Proposal 3: add the testing step to ensure target isolation from CW node to testing antenna as below:

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Test simplification for Rx orientation

- The common understanding is to define orientation requirements for Rx requirements, i.e. EIS spherical coverage related requirements. For testing, it’s suggested to simplify related testing, one suggestion is to assume device using dipole antenna and translate the CDF of spherical coverage into the department degree from peak antenna direction. For example, EIS spherical coverage requirement of 50% CDF point with extra 3dBm desense could be translated into 78degree department from peak antenna direction. If so we can only test 78 degree cross section instead of testing the total spherical. Another suggestion is that we only test hemi-spherical which align with device antenna design which may only point at one side while the other side is attached on the goods.

- Observation 1: following list two options to simplify OTA testing requirements.

- Option 1: only test semi-spherical

- Option 2: only test the cross section with X degree.

- It’s hard to assume all devices only using dipole antenna. So above option 1 is suggested.

- Proposal 4: it’s suggested to only test the cross section with Y degree.

- MU

- In 38.870, MU of measurement distance source is different for three modes, i.e. hand only (browsing mode), Beside Head and Hand (Talk mode) and Wrist-Worn device measurement. For device, we should make it clear to refer to which modes.

| - Uncertainty value | - hand only (browsing mode) | - Beside Head and Hand (Talk mode) | - Wrist-Worn device |
| --- | --- | --- | --- |
| - Measurement distance | - 0,0 | - 0.27,0.27 | - 0,0 |

- From out view, the measurement distance uncertainty budget should be zero, i.e. the same as hand only could be reused.

- Proposal 5: the MU of measurement distance source is the same as hand only mode.

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: Power difference between received backscatter signal and the CW signal leakage is at least 15dB to avoid the impact of CW signal to device Tx requirements during testing. For device Rx requirement testing, CW input level should be the side condition defined together with reader’s REFSENSE requirements.

- Proposal 2: it’s suggested to use figure 7.2-1 as the baseline with fixed co-located testing and CW antenna while rotating device for 3D testing.

- Figure 7.2-1: Example of a FR1 TRP TRS OTA test system with combined axis.

- Proposal 3: add the testing step to ensure target isolation from CW node to testing antenna as below:

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Observation 1: following list two options to simplify OTA testing requirements.

- Option 1: only test semi-spherical

- Option 2: only test the cross section with X degree.

- Proposal 4: it’s suggested to only test the cross section with Y degree.

- Proposal 5: the MU of measurement distance source is the same as hand only mode.

References

- RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR