3GPP TSG RAN WG4 Meeting #116				R4-2509725

Bengaluru, India, 25th – 29th August 2025

Source:	MediaTek Inc.

Title:	UE RF centric evolution Work scope for 5G-Advanced in Rel-20

Agenda Item:	11

Document for:	Information

# Introduction

In last Prague RAN #108 meeting, the WF on RAN4 R20 package was endorsed [1]. However, RAN did not decide detail work scope for UE centeric evolution in Rel-20. RAN only decided a few broad objectives such as High power enhancements in FR1, 6MHz irregular CBW and relaxed Tx EVM scope in 5G-A as shown in section 2.1. In this paper, we share our views on the detail UE centric RF work scopes to consider the balance with 5G-A features and 6G Study features in Rel-20 timeline.

# FR1 High Power UE Work scope for 5G-A Rel-20

## Agreements at last RAN Plenary for UE RF related topics [1]

## Agreements:

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109):

- FR1 HPUE enhancement

  - Further consider/discuss until RAN#109 on the detailed scope

  - This work is to focus on the TN application

- 6MHz channel bandwidth

  - It can be prioritized to FDD band <1GHz

- Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM

  - BS demod scope is included.

  - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact

    - The details for study is to be discussed until RAN#109

  - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G.

- No RAN1/2 scope is expected

- Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

The detail FR1 HPUE work scope is not finalize until now, hence we share our preferred work scope in FR1 HPUE in Rel-20 in this paper.

## Rel-20 FR1 HPUE enhancement work scopes for 5G-A Rel-20

Firstly, we can suggest to have a study phase for 1Tx (1x29dBm PA) PC1.5 UE in TDD band to verify the feasibility of commercial PA whether to support PC1.5 UE with front-End loss with 4dB in TDD band. Although a 29dBm archivable PA is developed to support PC1.5 with 1PA configuration, this is not directly match to apply to small form-factor handheld device, i.e. smartphone device.

Hence, RAN4 shall consider a feasibility study phase for 1Tx PC1.5  for TDD operating band. Based on above observation, the Following UE RF centric evolution work scope are proposed in 5G-A feature.

Proposal 1: For Release 20 UE centric evolution, only single band HPUE work scope would be considered. A study phase for 1Tx (1x29dBm PA) PC1.5 UE in TDD band would be considered to verify the feasibility of commercial PA/Components whether to support PC1.5 UE with front-End loss with 4dB in TDD band.

- Work Item Objectives for HP UE RF evolution

- FR1 HPUE enhancement (PC1.5/PC1) for single carrier

  - Single band and single CC:

  - PC1.5 (29dBm) for NR bands for both HHUE and FWA

    - 1Tx (1x29dBm) for NR TDD band (Need study phase to verify the feasibility)

      - Taking n41, n77 and n79 as the example bands.

    - 2Tx (2x26dBm) for NR FDD band

      - Take an example band from Operator request

    - 3Tx (26+23x2 or 26x2+23) for NR bands

      - Take an example band from Operator request

  - PC1 (31dBm) for NR bands for FWA only

    - 4Tx (4x26dBm) for NR bands

    - 2Tx (2x29dBm) for NR bands on top of the results of feasibility study on 1Tx (1x29dBm) for TDD band

    - Specify TN ACLR level for PC1 FWA Device (Not to refer the ACLR requirements of the public safety PC1 UE)

## Rel-20 Irregular Channel work scopes for 5G-A Rel-20

Based on the RAN Plenary discussion, we propose our preferred Objectives for 6MHz irregular CBW

- Work Item Objectives for irregular CBW

- 6MHz channel bandwidth

  - Specify RAN4 RF requirements and system parameters

  - NR Example band: n5, n12, n29 and n85

Note: Limited 15kHz SCS for example bands to minimize RAN4 workload

## TX EVM enhancements for 5G-A Rel-20

Even though the TX EVM relaxation was included in 5G-A, the Tx EVM scope would be studied from Q1 2026, furthermore Tx EVM relaxation will be impacted to re-define the existing MPR/A-MPR requirements in UE aspect. Also, RAN4 can have some similar work scopes are studied included in 6G study timeline. Then the UE RF workload can be duplicated to 5G-A MPR/A-MPR optimization and 6G MPR/A-MPR evaluation with unnecessary regulation limits update. Therefore, we would like to move the Tx EVM scope of 5G-A into 6G Study work scope at this stage. It would be fine to follow the 6G conclusion and decision of the corresponding RF requirements would be applied to 5G-A specification.

Based on the above observation, we recommend as follow

Proposal 2: For Tx EVM enhancement feature, we recommend to move the scope in the 6G study since the evaluation workload will be duplicated between 5G-A MPR/A-MPR optimization by Tx EVM and 6G MPR/A-MPR evaluation by ACLR, IBB, OOBE, occupied CBW and others.

# Conclusion

In this RAN4 #116 meeting, we provided our preferred objectives on High power UE related work scope in Rel-20 based on the endorsed WF [1] at last RAN #108 Meeting.

To balance with 5G-A workload and 6G study workload, we propose detail UE centric evolution work objectives in 5G-A as follow:

Proposal 1: For Release 20 UE centric evolution, only single band HPUE work scope would be considered. A study phase for 1Tx (1x29dBm PA) PC1.5 UE in TDD band would be considered to verify the feasibility of commercial PA/Components whether to support PC1.5 UE with front-End loss with 4dB in TDD band.

Proposal 2: For Tx EVM enhancement feature, we recommend to move the scope in the 6G study since the evaluation workload will be duplicated between 5G-A MPR/A-MPR optimization by Tx EVM and 6G MPR/A-MPR evaluation by ACLR, IBB, OOBE, occupied CBW and others.

UE RF centric evolution

- FR1 HPUE enhancement (PC1.5/PC1) for single carrier

  - Single band and single CC:

  - PC1.5 (29dBm) for NR bands for both HHUE and FWA

    - 1Tx (1x29dBm) for NR TDD band (Need study phase to verify the feasibility)

      - Taking n41, n77 and n79 as the example bands.

    - 2Tx (2x26dBm) for NR FDD band

      - Take an example band from Operator request.

    - 3Tx (26+23x2 or 26x2+23) for NR bands

      - Take an example band from Operator request.

  - PC1 (31dBm) for NR bands for FWA only

    - 4Tx (4x26dBm) for NR bands

    - 2Tx (2x29dBm) for NR bands on top of the results of feasibility study on 1Tx (1x29dBm) for TDD band

    - Specify TN ACLR level for PC1 FWA Device (Not to refer the ACLR requirements of the public safety PC1 UE)

- 6MHz channel bandwidth

  - Specify RAN4 RF requirements and system parameters

  - NR Example band: n5, n12, n29 and n85

* Note: Limited 15kHz SCS for example bands to minimize RAN4 workload

# References

- RP-251879, “Way Forward on RAN4 R20 packages” RAN4 Chair (Apple), 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.

- RP-251752, “Motivation for irregular Channel Bandwidth Work Item in Rel-20”, T-Mobile USA, China Telecom, China Unicom, Bell Mobility, Boost Mobile, UScellular, Murata, Iridium, 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.

- RP-251305, “Rel-20 RAN4 UE RF/OTA”, MediaTek Inc., 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.