3GPP TSG-RAN WG4 Meeting #116								    R4-259728

Bengaluru, India, 25th – 29th Aug, 2025

Agenda Item: 5.30.1

Source: MediaTek Inc.

Title: Discussion on remaining issue of Tx switching for 3Tx UEs in Rel-19 TEI

Document for: Discussion

# 1	Introduction

In RAN4#114b meeting, a WF [1] was agreed on further enhancing Tx switching in TEI-19, and an LS [2] was sent to RAN1. In RAN4#115 meeting, a WF [3] was agreed on further details for scenario 1. In our understanding, the only remaining issue is the UE feature, which needs to be provided to RAN2 to define the corresponding UE capabilities. Therefore, in this paper, we focus on UE feature of scenario 1.

# 2	Open issues

Regarding the UE feature list, we provided a draft version which was revised from Rel-17 feature list [4] based on the agreed WF [3]. Since RAN2 can finish the Rel-17 UE capability based on RAN4’s Rel-17 feature table, we do not foresee any problem to create Rel-19 feature list based on Rel-17 Tx switching. In our understanding, this is the approach which can minimize the required discussions in both RAN4 and RAN2, because the Rel-17 conclusions can be re-used as much as possible.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59-1 | Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Indicate the supported switching period for dynamic UL Tx switching between two bands for 3Tx UE with up to two transmit antenna connectors in each band | Rel-16RAN4 feature group 7-1 (Dynamic Tx switching between two uplink carriers) | Yes | N/A | UE does not support dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | UE signals supported switching period per pair of UL bands per UL band combination | No need | FR1 only | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} UE is mandatory to support dualUL when supporting this feature | Optional with capability signalling |
| 59-2 | Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate that for the band where DL interruption is needed due to dynamic UL Tx switching between 2 band for 3Tx UE with up to 2Tx per band | 59-1 | Yes | N/A | UE not reporting this capability means DL interruption is not required | UE capability is defined as per band per band combination for each band pair supporting UL Tx switching | No need | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 59-3 | UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate whether UL-MIMO coherence is supported when dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band is conducted. | 59-1 | Yes | N/A | Rel-15 per band capability pusch-TransCoherence is applicable | Per band per BC | No need | FR1 only | Support mixture of FDD/TDD | Candidate values set: {nonCoherent, fullCoherent } | Optional with capability signalling |

Proposal 1: For scenario 1, RAN4 to introduce the UE features 59-1, 59-2 and 59-3 for the switching period, DL interruption and UL-MIMO coherence for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band, respectively.

# 3	Summary

In this paper, we discussed the open issues based on the previous agreed WF for scenario 1 for further Tx switching enhancement in Rel-19 TEI. We have the following proposal.

Proposal 1: For scenario 1, RAN4 to introduce the UE features 59-1, 59-2 and 59-3 for the switching period, DL interruption and UL-MIMO coherence for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band, respectively.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59-1 | Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Indicate the supported switching period for dynamic UL Tx switching between two bands for 3Tx UE with up to two transmit antenna connectors in each band |  | Yes | N/A | UE does not support dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | UE signals supported switching period per pair of UL bands per UL band combination | No need | FR1 only | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} UE is mandatory to support dualUL when supporting this feature | Optional with capability signalling |
| 59-2 | Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate that for the band where DL interruption is needed due to dynamic UL Tx switching between 2 band for 3Tx UE with up to 2Tx per band | 59-1 | Yes | N/A | UE not reporting this capability means DL interruption is not required | UE capability is defined as per band per band combination for each band pair supporting UL Tx switching | No need | FR1 only | Support mixture of FDD/TDD | The same capability for Rel-16 DL interruption due to Tx switching is reused. | Optional with capability signalling |
| 59-3 | UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate whether UL-MIMO coherence is supported when dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band is conducted. | 59-1 | Yes | N/A | Rel-15 per band capability pusch-TransCoherence is applicable | Per band per BC | No need | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

# 4	References

- [1] R4-2505220, WF on Tx switching for TEI19, MediaTek inc.

- [2] R4-2505221, LS on UL Tx switching for TEI19, RAN4

- [3] R4-2508037, WF on 3Tx switching, MediaTek inc.

- [4] R4-2211189, Rel-17 RAN4 UE feature list for NR, CMCC