3GPP TSG-RAN WG4 Meeting #116		R4-2509911
Bangalore, India, 25th – 29th Aug 2025

Title:	R19 NTN overheating and SAR

Source:	OPPO

Agenda item:	7.6.3.3.1

Document for:	Approval

- Introduction

In previous meetings, the overheating for NTN HPUE was raised and how to accommodate it needs further discussion. This paper shares the view on this issue.

- Discussion

2.1 Power reduction due to overheating

It is understood that UE may face the overheating when keep transmitting at MOP, and unfortunately this is the typical situation for NTN. It was pointed out in previous meeting that this overheating may constraint UE RF components like duplexer design. Meanwhile, for FDD HPUE in TN, the maxUplinkdutycycle capability and default 50% value seem also apply to FDD HPUE in 38.101-1. Whether and how to apply this capability for NTN HPUE was also discussed.

In our view, the maxUplinkdutycycle capability and default 50% value were introduced in Rel-15 for TDD HPUE due to SAR issue at that time, and then in Rel-17 when FDD HPUE introduced this capability was reused. And in NTN, now the problem raised is overheating issue which is different from SAR issue. Mixing these two issues with one capability will make the power reduction complex and unclear in spec.

Meanwhile, it was also commented that whether UE will do power reduction when the scheduled duty cycle exceeds the maxUplinkdutycycle capability is undetermined, i.e., allow but not mandate to do power backoff. This is also different from how the maxUplinkdutycycle capability works in SAR issue.

Therefore, if similar duty cycle scheme is to be applied to overheating issue, then a separate duty cycle capability should be used.

Observation 1:   For NTN, UE will transmit at its MOP in most of the time which may cause the overheating issue. And limiting its transmitting time would be one of the solutions.

Observation 2:   The UE behavior (power backoff) would be different for NTN overheating issue comparing to TN SAR issue. And mixing these two issues together by using same maxUplinkdutycycle will complex the specification and break the already defined SAR scheme.

Proposal 1:         If similar duty cycle scheme for TN SAR issue is to be applied to NTN overheating issue, then a separate duty cycle capability should be used. In this way, UE could be allowed but not mandate to do power backoff when the scheduled duty cycle exceeds the capability.

2.2 NTN SAR

About SAR issue, duty-cycle based SAR scheme has also been discussed a long time. And there are various duty cycle based solutions as listed in last meeting WF[1]:

- Options for Selection: - Option 1: Keep the same status as TN for respective base RAT - NR-NTN: Reuse NR TN, - Including ΔPPowerClass in configured transmitted power clause - eMTC based Iot-NTN: Reuse eMTC - Not precluding ΔPPowerClass in configurated transmitted power clause - NB-Iot based Iot-NTN: Reusing NB-Iot - No duty cycle related scheme defined - Option 2: Introduce further revision / new method - NR-NTN sub-options: - A. Introduce clarification for FDD band support; (Nokia, Qualcomm) - B. changing the ΔPPowerClass from mandatory to optional;(Qualcomm) - C. UE voluntarily dropping uplink transmissions by self-assessment to control the SAR (vivo) - D. differentiate UE working status in terms of whether SAR is required in a time duration. (OPPO) - E. To have the network to notify/signal its upper bound of uplink ratio for a certain time duration (Samsung) - F. Energy headroom reporting (Qualcomm) - G. ΔPPowerClass and P-MPR reporting (Ericsson, SONY, Nokia) - H. Send information LS to RAN2 support PC1 NTN HPUE for FDD use cases. (Nokia) - I. inform the RAN that it is close to the SAR limit (ViaSet, Inmarsat) - Iot-NTN (including eMTC & NB-Iot based): - Introduce NR like scheme - Introduce other revision / new method as proposed for NR-NTN - Option3: Do not introduce for NTN - NR NTN : Explicitly preclude ΔPPowerClass and no other duty cycle related reference - eMTC-based Iot-NTN: Explicitly preclude ΔPPowerClass and no other duty cycle related reference - NB-Iot based Iot-NTN: No duty cycle or other new scheme defined

Firsly, we think the overheating issue should be considered separatley from SAR issue; Secondly, duty cycle based SAR scheme should differentiate UE working status to make the duty cycle scheme can be applied better, and we gave the detail solution in [2].

Considering this is the last meeting of Rel-19 WI, and still so many different solutions on the table, it seems impossible to reach consensus. If no consensus reached in the end, we suggest to stop the discussion and rely on P-MPR.

Proposal 2:         Don’t introduce duty cycle based SAR scheme in R19 for all NTN HPUE, and no reporting of  deltaPPowerClass in NTN.

- Conclusion

In this contribution, we discussed the overheating issue and SAR issue for NTN HPUE ,and abserved:

Observation 1:   For NTN, UE will transmit at its MOP in most of the time which may cause the overheating issue. And limiting its transmitting time would be one of the solutions.

Observation 2:   The UE behavior (power backoff) would be different for NTN overheating issue comparing to TN SAR issue. And mixing these two issues together by using same maxUplinkdutycycle will complex the specification and break the already defined SAR scheme.

Proposed:

Proposal 1:         If similar duty cycle scheme for TN SAR issue is to be applied to NTN overheating issue, then a separate duty cycle capability should be used. In this way, UE could be allowed but not mandate to do power backoff when the scheduled duty cycle exceeds the capability.

Proposal 2:         Don’t introduce duty cycle based SAR scheme in R19 for all NTN HPUE, and no reporting of  deltaPPowerClass in NTN.

Reference

[1] R4-2508122, WF on IoT-NTN HPUE, vivo, RAN4 #115

[2] R4-256920, R19 on NTN SAR, OPPO, RAN4 #115