3GPP TSG-RAN WG4 Meeting #116	R4-2509928

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on Conditional Intra-CU LTM

Agenda item:	7.25.3.3

Document for:	Discussion

Introduction

In this contribution, we continue the discussion about RRM core requirements for intra-CU CLTM and share our views on the remaining open issues related mainly to CLTM cell switch delay.

Discussion

CLTM cell switch delay

Although several agreements regarding the components of the CLTM cell switch delay were made during the RAN4#115 meeting, few issues regarding some delay components, like Tmeasure and Tfirst-RS/TRS-proc, remained open.

Tmeasure

Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

For CSI-RS-based L1-RSRP measurement:

Similar to the RAN4#115 agreement on Tmeasure definition for SSB-based L1-RSRP measurement, we believe that the definition of Tmeasure for CSI-RS-based L1-RSRP measurement can also follow the definition of the measurement delay in the event-triggered L1 reporting requirements discussed under the corresponding agenda item.

RAN4 to adopt the following definition of Tmeasure when CSI-RS-based L1-RSRP measurement is used in the event:

- 1> If CSI-RS-based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of CSI-RS-based L1-RSRP measurement periods of the serving cell and target cell.

- 2> serving cell measurement period TL1-RSRP_Measurement_Period_CSI-RS is defined in clause 9.5.4.2 assuming TReport = 0, and

- 2> if the target cell is a known cell:

- 3> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra defined in clause 9.14a.5 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled, or

- 3> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra + TCSI-RS_SFN_intra, otherwise. TL1-RSRP_Measurement_Period_CSI-RS_intra is defined in clause 9.14a.5 assuming TReport = 0, and TCSI-RS_SFN_intra is defined in clause 9.10.2.5.

Tfirst-RS/TRS-proc

Issue 1-2-4: General on Tfirst-RS and TRS-proc < Agreement > - [Tfirst-RS and TRS-proc] are included in Tinterrupt.

As we have pointed out in our previous contributions, we think it is reasonable to assume that Tfirst-RS and TRS-proc do not need to be considered as part of the interruption. When the fine DL synchronization is done before the cell switch in LTM, the procedures (i.e., early TCI state activation) do not cause an interruption. We proposed in Rel-18 that these procedures would not cause an interruption during the cell switch either, but at that time it was argued by some companies that since the network sends the cell switch command, the UE will not be scheduled after receiving the cell switch command anymore. This is not the case in CLTM, where the cell switch decision happens at the UE side. Hence, we think that DL synchronization components, if included in the cell switch delay, should not be part of the interruption time.

Tfirst-RS and TRS-proc should not be included in the CLTM cell switch interruption time.

Since there has been strong opposition from some companies towards Proposal 2, it may be difficult to agree to introduce this as mandatory UE requirement. However, since there has been support for this enhancement from at least one UE vendor, another option that RAN4 can consider is to define a UE capability that allows the network to know if the UE is able to perform fine DL synchronization with the target cell without interruption.

RAN4 to consider defining a UE capability to indicate that UE can perform fine DL synchronization with the target cell before interruption.

Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 < Agreement > ad hoc - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. Note: the limitation of 160/480 ms from Rel-18 still applies.

In the last meeting, RAN4 agreed on one new condition for Tfirst-RS = 0 and TRS-proc = 0 based on the target TCI state. In the following part, we propose another condition for Tfirst-RS = 0 and TRS-proc = 0, that is needed in the CLTM cell switch delay definition.

When the network sends PDCCH order and the UE sends the preamble to a candidate cell, similar to L3-based Rel-18 LTM, the DL sync should be considered applicable based on a successful preamble transmission. For UE to be able to use the TA it received in TA MAC-CE for RACH-less cell switch, UE will need to have DL sync. Hence, while the TA timer is running, it is beneficial that the UE keeps track of the DL sync and hence Tfirst-RS and TRS-proc should be zero. This proposal was briefly discussed in the last meeting, but companies wanted to check the timeline more carefully. As the DL sync based on preamble transmission was discussed widely in Rel-18, we think this discussion does not need to be repeated. UE configured with CLTM can expect to receive a TA MAC-CE following the PDCCH order, so the DL sync can apply when the UE has received the TA MAC-CE until the TA timer expires.

If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running.

L1-based condition evaluation for cells without active TCI states in FR2

In Rel-18 L1 measurement requirements, after TCI state activation, the UE is allowed to prioritize measurements on candidate cells with active TCI states and hence there are no requirements for L1 measurements on candidate cells without active TCI states in FR2. Therefore, if the UE is configured with L1-based CLTM condition for a candidate cell without an active TCI state in FR2, the related measurement requirements are undefined, and it is unclear whether the UE will continue condition evaluation for this cell.

In RAN4#114bis, we proposed to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2 (i.e., whether RAN4 needs to define measurement requirements for these cells so that the evaluation delay (Tmeasure) can be defined and tested). As a result of this discussion, RAN4 made the following agreement.

< Agreement > - L1 measurement behaviour, which was introduced in R18, is not changed. CLTM requirements do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2.

To the best of our knowledge, there was no dedicated behavior defined in RAN2 for CLTM L1-based condition evaluation for candidate cells without active TCI states in FR2 (e.g., that UE would stop condition evaluation for candidate cells without active TCI states).

Taking these details into account, if UE triggers a conditional cell switch to a candidate cell without active TCI state based on an L1 condition in FR2, CLTM cell switch delay and interruption time requirements are undefined and cannot be tested. While it remains unclear whether the UEs will continue condition evaluation, we think at least the interruption requirement should be followed if the UE continues condition evaluation and triggers a cell switch. To make sure that at least interruption time requirements apply for this case, we propose to revisit the RAN4#114bis agreement as follows.

RAN4 to revisit the agreement made in RAN4#114bis regarding CLTM requirements for candidate cells without active TCI states in FR2, following this update:

  - L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.

Conclusion

The following proposals were made in this contribution:

Proposal 1: RAN4 to adopt the following definition of Tmeasure when CSI-RS-based L1-RSRP measurement is used in the event:

1> If CSI-RS-based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of CSI-RS-based L1-RSRP measurement periods of the serving cell and target cell.

2> serving cell measurement period TL1-RSRP_Measurement_Period_CSI-RS is defined in clause 9.5.4.2 assuming TReport = 0, and

2> if the target cell is a known cell:

3> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra defined in clause 9.14a.5 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled, or

3> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra + TCSI-RS_SFN_intra, otherwise. TL1-RSRP_Measurement_Period_CSI-RS_intra is defined in clause 9.14a.5 assuming TReport = 0, and TCSI-RS_SFN_intra is defined in clause 9.10.2.5.

Proposal 2: Tfirst-RS and TRS-proc should not be included in the CLTM cell switch interruption time.

Proposal 3: RAN4 to consider defining a UE capability to indicate that UE can perform fine DL synchronization with the target cell before interruption.

Proposal 4: If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running.

Proposal 5: RAN4 to revisit the agreement made in RAN4#114bis regarding CLTM requirements for candidate cells without active TCI states in FR2, following this update:

	L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.