3GPP TSG-RAN WG4 Meeting #116	R4-2509941

Bengaluru 25th – 29th Aug 2025

Source: 	Nokia

Title: 					n1 PHS A-MPR proposals

Agenda Item: 			4.2.1

Document for:	Approval

# 1	Introduction

We propose adjustments to the A-MPR tables of NS_05 worked on in previous meetings [1], [2].

# 2	Discussion and proposals

Table 1: A-MPR regions for PC3 NS_05 from [1] and [2].

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |  |  |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs | ≤ 2.16MHz/12/scs | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 1.8MHz/12/scs | ≤ 5.76MHz/12/scs | > 1.8MHz/12/scs ≤ 4.68MHz/12/scs |

Table 2: A-MPR regions for PC2 NS_05 from [1].

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 5 | < 1925 | ≥ 3.24(18RB) | ≤ 0.18 | N/A | N/A | N/A | N/A |
| 10 | ≤ 1925 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6 | ≥ 0 |
| 10 | 1925 < fc < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs | ≤ 3.6MHz/12/scs | ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs | ≥ 0 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs | ≤ 1.8MHz/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | N/A | N/A |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs | ≥ 0 | ≤ 6.48MHz/12/scs; < RBstart*1.0 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | N/A | N/A |

According to our review of A-MPR triangles provided by the companies, the region highlighted with red color in table 1 doesn’t need A-MPR.

We think the A-MPR values due to allocation-image IMD3 hitting the same mask segment could be harmonized. In tables 1 and 2, the maximum A-MPR values are assigned to -25 dBm@1MHz mask segment in blue cells and -30 dBm@1MHz segment in green cells.

Proposal 1: RAN4 shall remove the A-MPR for Region A3 of table 1 for 15 MHz CBW at 1932.5 MHz ≤ fc < 1938.2 MHz (cell highlighted with red color).

Proposal 2: RAN4 shall harmonize the A-MPR values when they come from allocation-image IMD3 hitting the same mask segment. The corresponding A-MPR regions are highlighted with blue and green colors in tables 1 and 2.

Proposal 3: RAN4 shall not specify A-MPR smaller than MPR.

# 3	Conclusions

We have made the following proposals considering n1 NS_05 A-MPR.

Proposal 1: RAN4 shall remove the A-MPR for Region A3 of table 1 for 15 MHz CBW at 1932.5 MHz ≤ fc < 1938.2 MHz (cell highlighted with red color).

Proposal 2: RAN4 shall harmonize the A-MPR values when they come from allocation-image IMD3 hitting the same mask segment. The corresponding A-MPR regions are highlighted with blue and green colors in tables 1 and 2.

Proposal 3: RAN4 shall not specify A-MPR smaller than MPR.

# 4	References

[1] R4-2508023, WF on PHS protection update, KDDI, OPPO, Ericsson

[2] R4-2505222, WF on PHS protection update, KDDI