3GPP TSG-RAN WG4 Meeting #116										R4-2510213

Bengaluru, India, 25thth to 29th August 2025

Agenda Item:	7.9.3.3

Source:	Ericsson

Title:	On Transmitter Spurious Emission Co-existence discussion

Document for:	Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except for a few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

In this contribution, we share our simulation evaluations and findings to trigger further discussions on open issues.

# 2	Discussion

In this contribution, we outline our approach for conducting system-level evaluations to assess the requirements for spurious emissions considering technical aspects related to AAS base stations. Section 2.1 addresses the modeling of antenna gain for out-of-band unwanted emissions. Section 2.2 describes the coupling loss methodology employed in system-level analysis. Finally, Section 2.3 presents pertinent simulation results that support our findings.

## 2.1	Modelling of antenna gain for unwanted emissions

The model equation for analyzing decorrelation effects in advanced antenna systems (AAS) was first introduced in 3GPP Rel-11 as part of the AAS Study Item documented in TR 37.840. This equation provided a mathematical framework to capture the relationship between spatial separation, correlation, and system performance. However, at that time, RAN4 could not reach agreement on the most appropriate roll-off profile, which describes how correlation decreases as spatial separation increases. The absence of a consensus meant that the model remained incomplete in this respect, limiting its practical applicability for performance evaluations and simulations.

In Rel-19, a profile was finally introduced in TR 38.922 providing a reference curve for modeling decorrelation in certain deployment scenarios. This topic has also been discussed in other standardization bodies, including ITU-R and ECC, though no firm conclusions have been reached so far. Ultimately, RAN4 remains the expert group responsible for deciding on the most appropriate model for this purpose.

The equation describes the composite far field antenna gain for an AAS in the presence of partial spatial correlation – particularly relevant for unwanted emissions.

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

As can be seen in the equation, the composite antenna pattern is the sum of the subarray element gain and an array gain component which depends on the array geometry, beamforming vector and element correlation. This model accounts for spatial decorrelation due to spectral separation from the carrier, which affects the coherent summation in the array factor. The key addition in this model is the ρ(f) where f=offset from carrier center, which models the roll-off of coherent gain as a function of offset from the carrier center frequency.

In the TR 38.922 Clause 7.3.2.1.2, a parameterized piece-wise linear roll-off model is suggested to capture fundamental aspects of the array antenna gain drop outside the wanted signal.

This function rolls off from:

- ρ(f)=1 when fc is [−0.5B, 0.5B] resulting in full coherent gain

- ρ(f)=0 when f = ±fo resulting in only subarray gain (no beamforming)

Many companies have raised concerns that assuming the correlation factor ρ(f) to be either 0 or 1 outside the desired carrier band is too simplistic and unrealistic for unwanted emissions from an Active Antenna System (AAS).

The unwanted emissions consist of a mix of two components:

- Correlated intermodulation distortion (IMD) products, which are partially coherent across antenna elements because they originate from the same nonlinear processes.

- Uncorrelated wideband transmitter noise, which is random and independent at each antenna element.

Because of this mixture, the correlation factor ρ(f) does not simply jump from 1 (fully correlated) inside the carrier to 0 (fully uncorrelated) immediately outside it. Instead, ρ(f) gradually decreases with frequency offset from the carrier edge as noise becomes more dominant and coherence diminishes. Assuming ρ(f)=1 outside the carrier tends to overestimate the coherent array gain, potentially underestimating interference levels, while assuming ρ(f)=0 can be too conservative by ignoring any partial correlation that exists.

Some companies raised concerns that with assumptions of ρ(f)= 0 or ρ(f)= 1 outside the wanted carrier is an oversimplified modelling of unwanted emissions from an AAS perspective. A more realistic approach uses a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside.

Some companies raised concerns that with assumptions of ρ(f)= 0 or ρ(f)= 1 outside the wanted carrier is an oversimplified modelling of OOB emissions from an AAS perspective.

A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside.

Selecting ρ therefore balances between underestimating array gain by assuming too little correlation and overestimating it by assuming too much, making it crucial to choose a value that reflects the actual mixture of noise and distortion in the system.

The full array doesn’t make sense for integer multiples of the fundamental frequency as the antenna array and beamforming are primarily optimized for the fundamental frequency. Thus, the antenna efficiency drops reducing the effective radiated power at integer multiples regardless of the array size. We can also not ignore the fact that the physical size of the antenna elements is fixed but the wavelength λ changes with frequency which further drastically changes the radiation pattern, impedance input and efficiency. These combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions.

As a tradeoff due to above reasons, we can look at two choices of correlation factor  ρ – 0.1 (almost incoherent) and 0.5 (partial coherent), as a way to model the partial coherence between array elements.

RAN4 to discuss the modelling of antenna gain for unwanted emissions assuming correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations.

## 2.2	Coupling loss approach with AAS vs Non-AAS

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive sector antenna with a fixed main beam with peak gain of 17 dBi.

Figure 2.2-1 illustrates the Rel-99 BS-to-UE and BS-to-BS coupling loss approach with non-AAS BS where there are two Networks 1 and 2 within the same geographical area. The aggressor AAS BS has interfering links towards the wanted UE in the same network and victim non-AAS BS of the other network. This impacts the spatial gain due to the fixed radiation pattern and limited nulling ability.

In Release-99 approach to evaluate the Minimum Coupling Loss which refers to 100% on the CDF curve, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas.

Figure 2.2-1 Rel-99 BS-to-UE and BS-to-BS coupling loss approach with non AAS BS

Figure 2.2-2 illustrates the BS-to-UE and BS-to-BS Coupling Loss approach with AAS BS where there are two Networks 1 and 2 within the same geographical area. In this scenario, the aggressor AAS BS from Network 1 maintains:

- A serving link to its wanted UE in the same network, which is the intended link through beamforming.

- Interfering links towards –

  - a victim UE affecting the DL performance

  - a victim AAS BS, which can suffer through BS-to-BS interference affecting the UL transmissions.

Figure 2.2-2 BS-to-UE and BS-to-BS Coupling Loss approach with AAS

With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the served UE generates a spatial separation towards victim UEs or BSs of the other network, in addition, larger antenna gain, sub-array antenna structures and more relevant propagation models that are available today should also be considered in the coupling loss evaluation.

To derive the coupling loss, we should derive the relevant CDF of the coupling loss between the aggressor BS and the victim UE or BS considering the impact of adaptive beamforming, adjusting the beam patterns to maximize the reception and transmission of signal. The beam could be steered to a different location in each scenario based on the channel and UE locations.

Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal.

In addition, in Rel-99 the minimum coupling loss which refers to 100% on the CDF curve was calculated towards the wanted UE or a neighboring base station. This value, however, is at the queue of the CDF, which is very sensitive to the simulation and can fluctuate across different simulation runs because the beams are dynamically adjusting, which might lead to variability in observed MCL values.

Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99.99th percentile of the coupling loss CDF, instead of the minimum value.

## 2.3	Simulation Results

In this section, we discuss the results based on correlation factor – 0.1 and 0.5. The results are obtained deriving the coupling loss between BS-to-UE and BS-to-BS from a two-network deployment where the aggressor network is at 3.5 GHz and the victim network is at 6.4 GHz.

### 2.3.1	BS-to-BS coupling with AAS Macro at 3.5 GHz and AAS Macro at 6.4 GHz

Figures 2.3.1.1-1 and 2.3.2.2-1 depicts the coupling loss CDF between the aggressor AAS Macro BS operating at 3.5 GHz, and the victim AAS Macro BS receiver operating at 6.4 GHz.

Proposed detailed assumptions can be found in Annex A along with the simulation methodology. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 3.5 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – based on correlation factor for 0, 0.1, 0.5 and 1.

  - Propagation model – TR 38.901

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 6.4 GHz

  - Antenna model – AAS with 8x16 array and 3x1 subarray antenna configuration

  - Gain at Rx – full array gain

  - Propagation model – TR 38.901

  - Antenna height – 25m

  - BS NF is 6 dB (based on n104)

We evaluate the coupling loss CDF from the aggressor BS and to a victim UE/ BS, assuming grid shift 100% and 50%. Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective. Considering grid shifts less than 50% would resemble the situation with two base stations co-located at the same site in the same geographical location. Co-location related work in terms of coupling loss is handled in separate discussions in RAN4.

Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective.

Further looking at the Clause 10.2.1 TR 25.942 from Rel-99, the BS-to-BS coupling loss was calculated assuming intermediate case scenario of 288m which reflects 50% grid shift.

RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation.

#### 2.3.1.1	Network grid shift is 100%

Figure 2.3.2.1-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 100%

In the Figure 2.3.2.1-1 the coupling loss from an aggressor AAS Macro BS in Network 1 towards the victim AAS Macro BS in Network 2 within the same geographical area is calculated assuming grid shift of 100% (black curve).Looking at correlation factor 0.5 as a trade-off to be not overly simplistic in modelling the out of band gains, the coupling loss at 99.99th percentile indicates a potential correction of ~7 dB compared to Rel-99 values in the BS-to-BS coupling loss assumption

The BS-to-BS coupling loss with 100% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~7 dB compared to Rel-99 values. However, as stated before Network grid shift of 50% better reflects the reuse of existing deployment infrastructure.

#### 2.3.1.2	Network grid shift is 50%

Figure 2.3.2.2-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 50%

In the Figure 2.3.2.2-1 the coupling loss from an aggressor AAS Macro BS in Network 1 towards the victim AAS Macro BS in Network 2 within the same geographical area is calculated assuming grid shift of 50% Looking at correlation factor 0.5 as a trade-off to be not overly simplistic in modelling the out of band gains, the coupling loss at 99.99th percentile indicates a potential correction of ~5 dB compared to Rel-99 values in the BS-to-BS coupling loss assumption.

The BS-to-BS coupling loss with 100% grid shift is higher due to greater spatial separation and increased path loss.

The BS-to-BS coupling loss with 50% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~5 dB compared to Rel-99 values.

### 2.3.2	BS-to-UE coupling with AAS Macro at 3.5 GHz and Isotropic UE at 6.4 GHz

Figure 2.3.2-1 depicts the CDF of the coupling loss between the aggressor BS deployed at 3.5 GHz and the victim UE associated to a 6.4 GHz network deployment.

Proposed detailed assumptions can be found in Annex A along with the simulation methodology. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 3.5 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – based on correlation factor for 0, 0.1, 0.5 and 1.

  - Propagation model – TR 38.901

  - Minimum BS - UE distance (2D) – 35 m

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 6.4 GHz

  - Isotropic UE with 0 dBi gain and without beamforming

  - Propagation model – TR 38.901

  - UE NF is 9 dB

Figure 2.3.2-1 Coupling loss CDF from aggressor BS Macro towards victim UE

If we compare the BS-to-BS and BS-to-UE coupling loss based on the CDF plots, the former depicts a more dominant case with the use of AAS BSs – high gain directional antennas and higher transmit powers resulting in stronger coupling.

BS-to-BS coupling is more dominant than the BS-to-UE coupling, as BS receivers are more sensitive and the use of AAS BSs with high gain directional antennas and higher transmit powers, results in stronger coupling.

With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~5 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Some companies raised concerns that with assumptions of ρ(f)= 0 or ρ(f)= 1 outside the wanted carrier is an oversimplified modelling of OOB emissions from an AAS perspective.

Observation 2	A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside.

Observation 3	The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

Observation 4	RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions.

Observation 5	In Release-99 approach to evaluate the Minimum Coupling Loss which refers to 100% on the CDF curve, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas.

Observation 6	Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal.

Observation 7	Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective.

Observation 8	The BS-to-BS coupling loss with 100% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~7 dB compared to Rel-99 values. However, as stated before Network grid shift of 50% better reflects the reuse of existing deployment infrastructure.

Observation 9	The BS-to-BS coupling loss with 100% grid shift is higher due to greater spatial separation and increased path loss.

Observation 10	BS-to-BS coupling is more dominant than the BS-to-UE coupling, as BS receivers are more sensitive and the use of AAS BSs with high gain directional antennas and higher transmit powers, results in stronger coupling.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to discuss the modelling of antenna gain for unwanted emissions assuming correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations.

Proposal 2	Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99.99th percentile of the coupling loss CDF, instead of the minimum value.

Proposal 3	RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation.

Proposal 4	The BS-to-BS coupling loss with 50% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~5 dB compared to Rel-99 values.

Proposal 5	With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~5 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	  R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

# Annex A: Simulation assumptions

## A.1	Co-existence simulation scenarios

Table A.1-1 summarizes the initial proposed simulation scenarios.

Table A.1-1: Summary of simulation scenarios

| No. | Aggressor Simulating frequency | Victim Simulating frequency | Direction | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 GHz/ 3.5 GHz | 6.4 GHz | BS DL to UE UL | Urban macro | First priority |
| 2 | 2 GHz/ 3.5 GHz | 6.4 GHz | BS DL to BS UL | Urban macro | First priority |
| 3 | 2 GHz/ 3.5 GHz | 6.4 GHz | BS DL to UE UL | Micro | Second priority |
| 4 | 2 GHz/ 3.5 GHz | 6.4 GHz | BS DL to BS UL | Micro | Second priority |
| Note: RAN4 to prioritize Macro deployments. |  |  |  |  |  |

## A.2	Network layout model

### A.2.1	Urban macro

The details on urban macro network layout model are listed in Table A.2.1-1 below.

Table A.2.1-1: Network assumptions for Urban Macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | For BS-to-BS operation, network grid shift is 50% &100% |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | 100% Outdoor |  |
|  | Indoor UE ratio | 0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1.5 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Figure A.2.1-1: Network layout with Grid shift

## A.3	Propagation Model

### A.3.1	Pathloss

The pathloss models are summarized in Table 7.4.1-1 from TR 38.901.

### A.3.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1. from TR 38.901.

### A.3.3	O-to-I penetration loss

The pathloss incorporating O2I building penetration loss is described in Clause 7.4.3.1 from TR 38.901.

## A.4	Antenna configuration

### A.4.1	Macro AAS BS antenna

- For the aggressor frequency 2 GHz and 3.5 GHz, the antenna parameters are re-used from 4400 to 4800 MHz frequency range found in Table 4.4.1.2-1 TR 38.922.

- For the victim frequency 6.4 GHz, the antenna parameters are re-used from 7125 to 8400 MHz frequency range found in Table 6.5.1.2-1 TR 38.922.

### A.4.2	Macro Non-AAS BS antenna

For the non-AAS antenna at the aggressor frequency 2 GHz and 3.5 GHz, the Table 5.4.4.2.1-1 in TR 37.840 states parameters for typical passive antenna types shared below -

| Antenna type | A1 | A5 | A10 | A15 | B5 | B10 | B15 | D5 | D10 | D15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No of radiation elements per column | 1 | 5 | 10 | 15 | 5 | 10 | 15 | 5 | 10 | 15 |
| No of columns | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 4 | 4 | 4 |
| Max array gain for a single column / dBi | 8.7 | 15 | 18 | 19.5 | 14.5 | 17 | 18.5 | 14.5 | 17 | 18.5 |
| Max antenna gain / dBi | 8.7 | 15 | 18 | 19.5 | 17 | 19.5 | 21 | 20 | 22.5 | 24 |
| Vertical radiating element spacing d/ | - | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| Horizontal radiating element spacing d/ | - | - | - | - | 0.6 | 0.6 | 0.6 | 0.5 | 0.5 | 0.5 |
| Vertical 3dB beamwidth of single element / deg | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 |
| Horizontal 3dB beamwidth of single element / deg | 65 | 65 | 65 | 65 | 80 | 80 | 80 | 80 | 80 | 80 |
| Losses of cable network / dB | 0.5 | 0.8 | 1.0 | 1.2 | 0.8 | 1.0 | 1.2 | 0.8 | 1.0 | 1.2 |

However, it was not discussed which antenna type to follow for the simulation assumptions. The antenna type A10 with 10 element single column antenna model could be used as a starting point for the simulation work. It is expected to produce results similar to a real passive antenna (with nulls and side-lobes). Further, it should be noted that A10 has a narrower horizontal beamwidth and 0.9 lambda vertical spacing as compared to AAS antenna assumptions.

Table A.4.2-1: Macro Non-AAS BS antenna

|  | Non- AAS Urban Macro |  |
| --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) |
| Parameter |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 10x1 | 10x1 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1x1 | 1x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | N/A |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 38 | 38 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 |

## A.5	Simulation Methodology

The simulation methodology is described below –

Step 1 – Generate Aggressor and Victim networks based on the deployment parameters, defined for the Network layout. The aggressor network operates at 2/3.5 GHz, while the victim network operates at 6.4 GHz. The propagation of the spurious emissions happens at 6.4 GHz.

Step 2 – UEs are randomly generated inside the two networks deployment.

Step 3 – The BF weights are adjusted to point to the LOS direction between the serving BSs and served UE, in the two networks.

Step 4 – Apply BF weights to the subarray ports.

Step 5 – Calculate the Coupling Loss between the aggressor BS antenna, dimensioned to operate at 2/3.5 GHz, and the victim UE or BS antenna, which are operating at 6.4 GHz.