3GPP TSG-WG RAN4 Meeting #116	R4-2510235

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.5

Source: 	Qualcomm Incorporated

Title: 	Discussion on Ambient IoT Device Testability

Document for:	Approval

# 1. Introduction

The agreements for A-IoT device OTA test method and open issues for the last two RAN4 meetings were captured in [1-2]. In this paper, we provide further views on the OTA test methodology for A-IoT devices.

# 2  OTA test methodology for A-IoT device

## 2.1 Test setup for A-IoT device testing

Two examples of FR1 TRP and TRS OTA test system are presented in TS 38.161 [3] as below figures.

Figure 1: Example of a FR1 TRP TRS OTA test system with combined axis [3]

Figure 2: Example of a FR1 TRP TRS OTA test system with distributed axis [3]

For test setup for AIoT testing, the target is to reuse the legacy OTA test setup for TRP and TRS as much as possible to reduce the test cost.

For the test system with distributed axis, one implementation is to use multiple test antennas to support θ range. For this implementation, it is easy to extend to support multiple candidate CW nodes. For example, expect the test antenna, the other test antennas can be used as CW node. While if only one test antenna is equipped in the test systems as shown in Figure 1 and Figure 2, it will increase the complexity of test system very much to support more than one CW node positions.

Observation 1: RAN4 should consider reusing the legacy TRP and TRS test setup for AIoT device testing. It is suggested that one probe test setup is reused for AIoT testing.

With additional CW node, another aspect is that whether the test setup should ensure the isolation between CW node and reader/measurement antenna. In [2], the following agreement was made:

Issue 4-2-2: Isolation between CW node and measurement antenna Agreement in RAN4#115: - Power difference between received backscatter signal and the CW signal leakage is at least [15dB] - FFS on reference point of power difference

The motivation of defining a min. isolation between CW node and measurement antenna is to avoid the degradation at the receiver due to the leakage from CW node.

Since the 2SB transmission BW is assumed for D2R bandwidth transmission, then an example of D2R transmission bandwidth is provided in Figure 3.

Figure 3: D2R transmission bandwidth in frequency domain.

Based on the feedback from TE vendors during the RAN4#115 offline discussions, the receiver of TE such as spectrum analyser can identify CW signal and D2R signal due to the frequency offset between D2R and CW. Therefore, it seems it is not necessary to define the power difference between received backscatter signal and the CW signal leakage, especially, if R=2 or 4 since CW leakage could more easily be filtered out. The only potential issue from CW leakage is that we should ensure it would not lead to blocking at the receiver.

Observation 2: The receiver of TE such as spectrum analyser can identify CW signal and D2R signal due to the frequency offset between D2R and CW.

Proposal 1: There is no need to specify power difference between received backscatter signal and the CW signal leakage. The min. isolation is defined to avoid the blocking issue at the receiver if separate CW antenna and measurement antenna is used.

## 2.2 Test methodology for Tx RF requirements of A-IoT device

### 2.2.1 Backscatter power/loss

The following agreement was reached in [2] regarding the backscatter loss. It is stated that the incoming CW signal and the backscatter signal are aligned with peak gain direction.

Issue 3-1-1: Transmit output power Agreement in RAN4#115: - Performance metric: EIRP - FFS on how to determine device’s peak antenna gain direction - FFS on whether to measure multiple directions - Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna. - FFS for BPSK - The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

Therefore, for backscatter power/gain testing, the separation between measurement antenna and CW antenna is 0 degree. That says we can use the same probe as measurement antenna and CW antenna. The peak direction could be declared by device. Regarding the polarization of CW antenna, since the backscatter power/gain is verified at the peak direction. It seems there is no need to send CW signal in two polarized sequentially. CW polarization could be declared by device.

Issue 4-2-3: Polarization of CW antenna Agreement in RAN4#115: - The CW signal shall be sent in two polarized sequentially, FFS on how to combine two polarization test results.

Proposal 2: For backscatter power/gain testing, the separation between measurement antenna and CW antenna is 0 degree, i.e., using the same probe as measurement antenna and CW antenna. The peak direction and CW polarization could be declared by device.

When the backscatter power/gain is tested, the input power level of CW signal is also needed to be defined. It could be one value or a range of CW Tx power depending on the conclusion from RF core requirements. On top of input power level of CW signal, the power level of R2D signal also needs to be specified as one of test conditions. The power level of R2D at device receiver should be set as sensitivity+[X]dB to avoid device failing the test due to not successfully decoding R2D signal.

Proposal 3: Input power level of CW signal and the power level of R2D should be decided together with core requirements. The level of R2D could be set as sensitivity+[X]dB. FFS on X.

### 2.2.2 Output RF spectrum emissions

For output RF spectrum emission (Out of band emission and SEM), the test procedure is quite similar as backscatter power/gain testing. The test direction for emission is the same as the test direction of backscatter power/gain testing, i.e., peak direction declared by device. The input power level of CW signal should be decided together with core requirements discussion. For CW polarizations, two CW polarizations should be sent sequentially to make sure maximum emission is verified.

Proposal 4: For AIoT device emission testing, the test direction is the same as the test direction of backscatter power/gain testing, i.e., peak direction declared by device. The input power level of CW signal should be decided together with core requirements. To verify the emission, two CW polarizations should be sent sequentially.

## 2.3 Test methodology for Rx RF requirements of A-IoT device

### 2.3.1 Reference sensitivity

It was agreed that reference sensitivity shall be specified for A-IoT device. Reference sensitivity is to verify the received performance of R2D link. While different from legacy NR device testing, it is not feasible to measure the reference sensitivity for R2D link via throughput metric, e.g., >95% of maximum throughput. Then the following agreements were reached in [2] regarding the test performance metric and test procedure for REFSENS:

Issue 3-4-1: Reference sensitivity Agreement in RAN4#115: - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

To properly test REFSENSE of AIoT device, it is proposed to use known PagingID and CFRA in the REFSENS test procedure. An example of REFSENS test procedure with CFRA is described in Figure 3.

Figure 4: Example of REFSENS test procedure with CFRA

Proposal 5: It is proposed to use known PagingID and CFRA in the REFSENS test procedure.

For the test directions for reference sensitivity, it is relying on the decision from core requirements discussion. For the peak direction of min EIS, it is recommended that the peak direction is declared by device to save the testing efforts.

Proposal 6: The testing direction for EIS at peak direction should be declared by device.

To ensure that the REFSENS performance of R2D is not impacted by poor D2R transmission quality—potentially causing incorrect decoding of the pagingID—we should assume a relatively high CW transmission signal during testing. For example, the CW input level should be at least equal to the maximum input level of R2D. Additionally, to simplify the testing process, the device can declare the polarization of the CW signal.

Proposal 7: The CW input level should be at least equal to the maximum input level of R2D for REFSENS testing.

Proposal 8: To simplify the testing process, the device can declare the polarization of the CW signal for REFSENS testing.

2.3.2 Maximum Input level

Similar as reference sensitivity testing, the maximum input level should be testing in the peak direction. And to ensure that the REFSENS performance of R2D is not impacted by poor D2R transmission quality, we should assume a relatively high CW transmission signal during testing. For example, the CW input level should be at least equal to the maximum input level of R2D. Additionally, to simplify the testing process, the device can declare the polarization of the CW signal.

Proposal 9: The testing direction for maximum input level should be declared by device.

Proposal 10: The CW input level should be at least equal to the maximum input level of R2D for maximum input level testing.

Proposal 11: To simplify the testing process, the device can declare the polarization of the CW signal for maximum input level testing.

## 2.4 Energy source and energy harvesting

In Step 3 of test procedure define in [1] for both Tx and Rx RF requirements, it is mentioned that device should have enough energy to complete the test according to device declaration on the required energy conditions.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

While it is not very clear about what would the required energy conditions declared by device. As captured in TR 38.769, the ON-OFF pattern for the device should be known during the testing as there is no specified device behavior regarding when device wakes up after fully charging and how long the device can be in ON status after starting transmission.

From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

Proposal 12: From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

# 3  Conclusion

In this paper, we provide our view on the remaining issues for A-IoT testability, and the following observations and proposals are made:

Observation 1: RAN4 should consider reusing the legacy TRP and TRS test setup for AIoT device testing. It is suggested that one probe test setup is reused for AIoT testing.

Observation 2: The receiver of TE such as spectrum analyser can identify CW signal and D2R signal due to the frequency offset between D2R and CW.

Proposal 1: There is no need to specify power difference between received backscatter signal and the CW signal leakage. The min. isolation is defined to avoid the blocking issue at the receiver if separate CW antenna and measurement antenna is used.

Proposal 2: For backscatter power/gain testing, the separation between measurement antenna and CW antenna is 0 degree, i.e., using the same probe as measurement antenna and CW antenna. The peak direction and CW polarization could be declared by device.

Proposal 3: Input power level of CW signal and the power level of R2D should be decided together with core requirements. The level of R2D could be set as sensitivity+[X]dB. FFS on X.

Proposal 4: For AIoT device emission testing, the test direction is the same as the test direction of backscatter power/gain testing, i.e., peak direction declared by device. The input power level of CW signal should be decided together with core requirements. To verify the emission, two CW polarizations should be sent sequentially.

Proposal 5: It is proposed to use known PagingID and CFRA in the REFSENS test procedure.

Proposal 6: The testing direction for EIS at peak direction should be declared by device.

Proposal 7: The CW input level should be at least equal to the maximum input level of R2D for REFSENS testing.

Proposal 8: To simplify the testing process, the device can declare the polarization of the CW signal for REFSENS testing.

Proposal 9: The testing direction for maximum input level should be declared by device.

Proposal 10: The CW input level should be at least equal to the maximum input level of R2D for maximum input level testing.

Proposal 11: To simplify the testing process, the device can declare the polarization of the CW signal for maximum input level testing.

Proposal 12: From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

# 4. References

- R4-2505230, “WF on A-IOT device requirements”, CMCC, April, 2025

- R4-2508116, “WF on A-IOT device requirements”, CMCC, May, 2025

- TS 38.161