3GPP TSG RAN WG4 Meeting #116	R4-2510328

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.25.3.2

Source:	vivo

Title:	On RRM requirements for LTM CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

In the RAN4#115 meeting, RAN4 discussed RRM requirements for LTM CSI-RS based L1 measurement. Following agreements were made and captured in the WF [1].

#### Issue 3-1: definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. #### Issue 3-4: Scaling factor for Rx beam sweeping Agreement: - For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8. #### Issue 3-5: UE measurement behaviour if CSI-RS configuration exceeds UE measurement capability Agreement: - RAN4 requirements apply assuming network will not configure CSI-RS based L1 RSRP measurement for LTM candidate cell exceeding UE measurement capability. - FFS on whether to capture this agreement in RAN4 spec. #### Issue 3-8: Requirement applicability Agreement: - For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, reuse legacy measurement period, measurement restriction and scheduling restriction requirements as baseline. - For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

In this contribution, we further provide our views on RRM requirements for LTM CSI-RS based L1 measurement.

# Discussion

#### Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

In the last RAN4 meeting it was agreed that for neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8. This would at least be applicable to LTM neighbor cell. However, for the serving cell as a LTM candidate cell, there were different views on whether SSB based measurement can be skipped.

If L1-RSRP measurement of the LTM candidate cell of serving cell follows serving cell measurement, rather than neighbor cell measurement, the legacy UE behavior of measuring L1-RSRP of serving cell should be unchanged. It may be complicated if different CSI-RS in the serving cell is measured with different behavior.

Proposal 1: If a FR2 serving cell is configured as LTM candidate cell, the L1-RSRP measurement of the LTM candidate cell follows legacy serving cell measurement, i.e., step 2 is not skipped and CSI-RS based measurement is not scaled by 8.

#### Issue 3-3: Sharing factor for resource collision between serving and neighbour cell Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

RAN4 specified requirements for different L1 measurements, e.g., RLM, BFD/CBD, L1-RSRP, L1-SINR, in an independent manner. If measurement resources for different L1 measurements is collided, measurement restriction requirements will apply. If is up to UE implementation how the measurements are performed. For ICBM, a scaling factor was introduced for L1-RSRP measurements on serving cell and neighbor cell with different PCI. However, when the RS for ICBM is further overlapping RSs for other L1 measurements, measurement restriction still applies. It seems not a complete solution.

For this CSI-RS based L1 measurement for LTM, measurement restriction approach could be used. If sharing factor is used, the requirements for SSB-based and CSI-RS L1-RSRP measurement for serving cell need to be updated accordingly. Furthermore, there is no sharing factor for SSB-based measurement for LTM if the SSB is overlapping with serving cell reference resource for L1-RSRP measurement.

Proposal 2: Measurement restriction requirements are defined for FR2 CSI-RS L1-RSRP measurements on neighbour cell.

#### Issue 3-6: Measurement restriction Candidate options: - Option 1: Apple, ZTE, Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms. - Option 2: vivo - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

It is similar to the restriction of CSI-RS configuration for L3 CSI-RS based mobility. Since the CSI-RS measurement will cause interruptions, it would be better to constrain the interruption to some fixed location. Option 1 is fine in general. In addition, it is better to further clarify the separation of the two windows if configured. Otherwise, if the two windows are too close, then it is similar to a 10ms length window.

Proposal 3: When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

Early CSI acquisition on a candidate cell was introduced for CSI-RS based LTM. There was discussion on whether and how to define requirements for this procedure.

#### Issue 3-9: early CSI acquisition Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

The details of early CSI acquisition are as follows based on RAN1 agreements.

For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available Option 1-1-1: Codepoint with the lowest CQI index is reported

LTM cell switch delay is defined as:

DLTM = Tcmd + TLTM-interrupt

TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

CSI measurement can be performed after UE obtains downlink synchronization. In legacy LTM cell switch, UE will wait for first RACH occasion for RACH-based cell switch or wait for first uplink transmission occasion for RACH-less cell switch. UE can perform CSI measurement during and after this uncertain waiting time depending on the CSI-RS resource availability.

Since it is different from legacy CSI acquisition after LTM switch, RRM requirements for early CSI acquisition need to be defined. A component of TCSI_reporting can be added to the existing LTM cell switch delay additionally, e.g.,

DLTM = Tcmd + TLTM-interrupt + TCSI_reporting

Where, TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

For periodic CSI-RS, it should be enough with adding the TCSI_reporting to the cell switch delay. For semi-persistent CSI-RS, UE may need to wait for MAC-CE for activation of the semi-persistent CSI-RS if it is not sent to UE together with cell switch command. To simplify the requirements, it can be assumed two MAC-CE are sent in the same MAC PDU.

For option 2, it would be very challenging to define scheduling/measurement restriction requirements for optional early CSI acquisition since CSI measurement behavior was not actually discussed in RAN4. It may take serval meeting cycles to converge.

Proposal 4: RAN4 to define RRM requirements for target cell early CSI-acquisition.

Proposal 5: A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

Proposal 6: TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

# Summary

In this contribution, we further provided our views on RRM requirements for LTM CSI-RS based L1 measurement. Following proposals and observations are present.

Proposal 1: If a FR2 serving cell is configured as LTM candidate cell, the L1-RSRP measurement of the LTM candidate cell follows legacy serving cell measurement, i.e., step 2 is not skipped and CSI-RS based measurement is not scaled by 8.

Proposal 2: Measurement restriction requirements are defined for FR2 CSI-RS L1-RSRP measurements on neighbour cell.

Proposal 3: When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

Proposal 4: RAN4 to define RRM requirements for target cell early CSI-acquisition.

Proposal 5: A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

Proposal 6: TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

- References

- R4-2508282	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple