3GPP TSG RAN WG4 Meeting #116	R4-2510341

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.18.2

Source:	vivo

Title:	Discussion on general aspects for AI/ML for NR air interface Phase 2

Document for:	Discussion

- Introduction

On the general aspects for AI/ML, in the last meeting, RAN4 further discussed the issues on LCM discussion. In this contribution, we further provide our views for the issue on RRM requirements for LCM. The latest way forward was captured in [1].

- Discussion

  - Activation delay requirements for BM

Agreements: Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

In the last meeting, we discussed the starting point of activation delay for each of the three CSI reporting types for Option A. Below, we will conduct further analysis on the activation delay for these three types.

- Periodic CSI reporting

Agreements

- (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

- (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).  FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

- (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

For periodic CSI reporting under Option A, we need to await further feedback from RAN2 regarding LS[2]. For Option B, as reference, we have listed RAN2's previous relevant conclusions above. The key difference from Option A is that when the UE submits an applicability report, the network only transmits sets of inference-related parameters. These parameters can be carried either via: RRCReconfigurationComplete or UAI messages. Subsequently, upon receiving the inference configuration in Step 5, the UE sends RRCReconfigurationComplete. For Option B, there are several candidate definitions for the activation starting point:

- Option 1: Functionalities are considered already activated before reporting applicable functionalities via RRCReconfigurationComplete in Step 4 (aligned with Interpretation 1 in the referenced LS)

- Option 2: Functionalities start to be activated when the UE reports applicable functionalities via RRCReconfigurationComplete in Step 4 (aligned with Interpretation 2 in the referenced LS)

- Option 3: Functionalities start to be activated after the UE receives CSI-ReportConfig for inference configuration in the RRCReconfiguration message of Step 5

Regarding Option 2, our understanding is: After the network transmits inference-related parameters, the UE can initiate pre-processing (e.g., loading partial inference-related information). The UE completes remaining processing only after receiving the inference configuration in Step 5's RRCReconfiguration message

The determination of the activation starting point for Option B's delay relates directly to RAN2’s reply to the LS about periodic CSI reporting for Option A. Therefore, the final definition of the starting point remains pending RAN2's response to the LS.

LS on AI/ML functionality activation [2] Companies in RAN4 have different interpretations about the following RAN2 agreement: “the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4”. - Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4 - Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 RAN4 kindly requests RAN2 to clarify which interpretation is aligned with RAN2 agreements.

Observation 1: For Option B, the determination of whether UE functionality should be considered as: beginning activation, or already activated after UE receives RRCReconfiguration in Step 5 (or UE reports applicable functionalities via RRCReconfigurationComplete in Step 4) is related to RAN2’s reply to the LS about periodic CSI reporting for Option A.

To facilitate the definition of activation delay for periodic CSI reporting, we propose the following working assumption:

UE functionality activation begins only after UE has conducted applicability reporting, and Network has sent the full inference configuration (For Option A: After Step 4, For Option B: After Step 5)

Under this assumption, the functionality activation timeline for both options would be structured as follows:

Upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (for Option A) or upon the reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5 (for Option B) in slot n，the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the activation delay of the functionality in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Proposal 1：For the periodic CSI reporting, the activation delay requirements created is defined as:

Upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (for Option A) or upon the reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5 (for Option B) in slot n, the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the activation delay of the functionality in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Note: The definition of the above delay is based on the working assumption that "activation begins only after the UE has reported applicability reporting and the network has sent the complete inference configuration."

- Semi-persistent CSI reporting and aperiodic CSI reporting

For semi-persistent CSI reporting and aperiodic CSI reporting, the starting point of the corresponding activation delay was already agreed in the last meeting. For DCI-triggered activation, the corresponding delay component is similar to the one mentioned above and to the periodic CSI reporting scenario. For MAC-CE-triggered activation, an additional THARQ must be considered. The specific delay definitions are as follows:

For MAC-CE based semi-persistent CSI reporting and aperiodic CSI reporting,

Upon reiceiving MAC-CE triggering activation command in slot n，the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+THARQ+3ms+ Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the functionality activation delay in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

For DCI-based semi-persistent CSI reporting, except for the starting point, the activation delay definition is similar to that of periodic CSI reporting.

Proposal 2: For MAC-CE based semi-persistent CSI reporting and aperiodic CSI reporting,, the functionality activation delay is defined as below:

Upon reiceiving MAC-CE triggering activation command in slot n, the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+THARQ+3ms+Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the functionality activation delay in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Proposal 3: For DCI-based semi-persistent CSI reporting, except for the starting point, the activation delay definition is similar to that of periodic CSI reporting

  - Activation/Deactivation requirements for CSI prediction

Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction Way forward (not agreements) Deactivation delay for CSI prediction: - The following questions were noted for further discussion - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting? - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation.

For CSI prediction, similar to BM LCM operations, the CSI framework will be utilized. Specifically, for activation, the mechanisms introduced by existing RAN1/RAN2 should be common to both BM and CSI prediction. Therefore, the activation delay for CSI prediction also needs to be defined. The relevant delay components can reference the activation delay requirements defined for BM.

Proposal 4: For CSI prediction, activation delay requirements should be introduced for periodic, semi-persistent, and aperiodic CSI reporting.

Regarding deactivation delay for CSI prediction, according to the current RAN2 running CR description, for AI/ML-based reporting, the corresponding reported quantities are newly introduced in R19, as highlighted below. From this perspective, the network can distinguish between predicted CSI reporting and measured CSI reporting based on this IE.

nrofReportedRS-v19xy ENUMERATED {n6, n8} OPTIONAL, -- Need R predictionConfiguration-r19 CHOICE { configurationForChannelPrediction-r19 SEQUENCE { predictionType-r19 ENUMERATED {beamPrediction, csi-InferencePrediction}, reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R

Regarding the necessity of introducing deactivation delay, we believe it is not necessary. The reason is as follows:

Existing RAN4 deactivation mechanisms (e.g., SCell deactivation) primarily address interruptions during deactivation. However, for AI-based functionality, it may coexist in parallel with non-AI operations without impacting them. Deactivating an AI functionality implies that the network no longer intends to use it for inference, which does not necessitate a strict time-bound completion requirement. RAN2-defined deactivation is implemented via network-initiated cell reconfiguration to release the existing CSI configuration, rather than through dedicated deactivation signaling.

Concerning the second bullet point ("In case of switch between AI"), the related procedures have neither been thoroughly discussed in other working groups nor reached any conclusions. Therefore, RAN4 does not need to address or define them.

Proposal 5: For CSI prediction and beam management, RAN4 should not define deactivation delay requirements.

  - Work load analysis for BM-Case 2

For the beam management use case, RAN4 has mainly focused on studying and analyzing the RRM impact and requirements of BM-Case 1 in the WI. However, for BM-Case 2, RAN4 has not conducted in-depth discussions or reached any agreements. Based on our preliminary analysis and summary, the RRM requirements related to time-domain prediction mainly include the following aspects:

- Aspect 1: Prediction delay requirement for BM-Case 2

- Aspect 2: TCI state-related (including TCI state known condition and MAC-based TCI state switch delay)

- Aspect 3: Metrics/KPIs for BM-Case 2 and corresponding performance evaluation

- Aspect 4: Performance monitoring-related (including delay requirement and accuracy requirement)

For these four aspects, we have provided analyses on the potential RRM impact for BM-Case 2 in the corresponding companion papers. Among them, aspects such as performance monitoring-related and TCI state-related requirements for BM-Case 2 have never been discussed before. Additionally, for Metrics/KPIs evaluation, our current simulation performance evaluaion only covers BM-Case 1. If we consider defining indicators for BM-Case 2, simulation evaluations will also need to be initiated.

According to the work plan, the requirements for the core part (mainly Aspects 1, 3, and 4) need to be finalized in this meeting. Completing the definition of these requirements for BM-Case 2 within a single meeting will be a highly challenging task. Therefore, RAN4 may need to reach a consensus on the plan for defining BM-Case 2 requirements.

Regarding monitoring reporting delay and accuracy, RAN4 reached the following highlighted conclusion in the last meeting: If the definition for BM-Case 2 is not completed in Rel-19, there would not be a monitoring delay requirement. For the other aspects, considering the limited discussion time, RAN4 needs to discuss whether to handle them in the same manner or selectively define only the necessary requirements.

Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

Observation 2: For BM-Case 2, the potential RRM requirement impacts mainly include the following aspects. Regarding these requirements, RAN4 has mainly focused on discussions for BM-Case 1 in the WI, while BM-Case 2 has not been thoroughly studied or discussed. Completing the definition of these requirements for BM-Case 2 within a single meeting will be a highly challenging task.

- Aspect 1: Prediction delay requirement for BM-Case 2

- Aspect 2: TCI state-related (including TCI state known condition and MAC-based TCI state switch delay)

- Aspect 3: Metrics/KPIs for BM-Case 2 and corresponding performance evaluation

- Aspect 4: Performance monitoring-related (including delay requirement and accuracy requirement)

Proposal 6: RAN4 needs to discuss whether and how to define the RRM requirements for BM-Case 2.

- Alt 1: If the definition of all core part-related requirements for BM-Case 2 cannot be completed in the RAN4#116 meeting, then no requirements related to BM-Case 2 (including core part and performance part) will be introduced in Rel-19.

- Alt 2: RAN4 selectively defines only some necessary requirements in RAN4#116 meeting.

- Summary

Observation 1: For Option B, the determination of whether UE functionality should be considered as: beginning activation, or already activated after UE receives RRCReconfiguration in Step 5 (or UE reports applicable functionalities via RRCReconfigurationComplete in Step 4) is related to RAN2’s reply to the LS about periodic CSI reporting for Option A.

Proposal 1：For the periodic CSI reporting, the activation delay requirements created is defined as:

Upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (for Option A) or upon the reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5 (for Option B) in slot n, the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the activation delay of the functionality in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Note: The definition of the above delay is based on the working assumption that "activation begins only after the UE has reported applicability reporting and the network has sent the complete inference configuration."

activation delay is defined as below:

Upon reiceiving MAC-CE triggering activation command in slot n, the UE shall be capable to transmit valid CSI inference report and apply actions for the functionality being activated no later than in slot n+THARQ+3ms+Tactivation_time+TCSI_ReportingNR slot length , where:

Tactivation_time is the functionality activation delay in milliseconds, including preprocessing for inference (e.g., loading required information for inference).

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Proposal 3: For DCI-based semi-persistent CSI reporting, except for the starting point, the activation delay definition is similar to that of periodic CSI reporting

Proposal 4: For CSI prediction, activation delay requirements should be introduced for periodic, semi-persistent, and aperiodic CSI reporting.

Proposal 5: For CSI prediction and beam management, RAN4 should not define deactivation delay requirements.

Observation 2: For BM-Case 2, the potential RRM requirement impacts mainly include the following aspects. Regarding these requirements, RAN4 has mainly focused on discussions for BM-Case 1 in the WI, while BM-Case 2 has not been thoroughly studied or discussed. Completing the definition of these requirements for BM-Case 2 within a single meeting will be a highly challenging task.

- Aspect 1: Prediction delay requirement for BM-Case 2

- Aspect 2: TCI state-related (including TCI state known condition and MAC-based TCI state switch delay)

- Aspect 3: Metrics/KPIs for BM-Case 2 and corresponding performance evaluation

- Aspect 4: Performance monitoring-related (including delay requirement and accuracy requirement)

Proposal 6: RAN4 needs to discuss whether and how to define the RRM requirements for BM-Case 2.

- Alt 1: If the definition of all core part-related requirements for BM-Case 2 cannot be completed in the RAN4#116 meeting, then no requirements related to BM-Case 2 (including core part and performance part) will be introduced in Rel-19.

- Alt 2: RAN4 selectively defines only some necessary requirements in RAN4#116 meeting.

- References

- R4-2508051, WF on NR_AIML_air_part2, Ericsson

- R4-2508085, LS on AI/ML functionality activation, CMCC