3GPP TSG-RAN WG4 Meeting #116	R4-2510353

# Bengaluru, India, August 25th – 29th, 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	7.1.2.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Approaches to enable MPR reduction

Extension values/ratios:

- Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case.

According to the WF in the last meeting [2], we have reached a consensus on extension values. The remaining issue is that the extension ratios are based on UE max transmission bandwidth configuration NRB or channel bandwidth. Firstly, take the current signallings about bandwidth indicating as an example. There are cell-specific carrier bandwidth within SCS-SpecificCarrier broadcast in SIB1 and UE specific channel bandwidth indicated via ServingCellConfig. The two signallings are finally indexed to IE SCS-SpecificCarrier, which is shown as follows. The bandwidths indicated by the network are all in PRB granularity. For MPR reduction feature, the extension ratios should multiply with UE max transmission bandwidth configuration NRB.

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=      SEQUENCE {

offsetToCarrier              INTEGER (0..2199),

subcarrierSpacing            SubcarrierSpacing,

carrierBandwidth             INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation  INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

Secondly, if extension bandwidth equals to the extension ratio multiplied by UE channel bandwidth, the extension bandwidth may be not a regular bandwidth whose NRB is not specified. Take an example, if UE channel bandwidth is 30MHz and extension ratio is 1/4, the extension bandwidth at each side is 7.5MHz, and the corresponding NRB is not defined in the spec, which will not facilitate to define the new inner RB region.

Proposal 1: For MPR reduction feature, the extension bandwidth equals to extension ratio multiplied by UE max transmission bandwidth configuration NRB .

Guard band for extended CBW

- Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests

To ensure RB alignment between the UE channel bandwidth and the extended CBW, we propose that RB based extension starts from the edge of the original UE maximum transmission bandwidth configuration, which overlaps with the guard band of the UE channel bandwidth (as illustrated in Figure 2.1-1). In terms of the guard band for extended CBW, we propose not to define it. If the guard band of the extended CBW is defined, the extension will be RB based & CBW based, where the applying range of IBE and OOBE requirements is depicted in Figure 2.1-2. In this approach, IBE requirement will end at the last extended RB and SEM requirement will start from the edge of the extended CBW. There is no requirement applied in the guard band of the extended CBW. Since the extended RB is a virtual bandwidth, the part or all of extended bandwidth may be allocated to another UE. To avoid interference, it is important to apply emission requirement in the guard band of the extended CBW. Hence, we propose not to define the guard band for the extended CBW like Figure 2.1-3.

Proposal 2: Not to define the guard band for extended CBW.

Figure 2.1-1: RB based extension

Figure 2.1-2: Applicable emission requirement when guard band of the extended CBW is defined

Figure 2.1-3: Applicable emission requirement when guard band of the extended CBW is not defined

Derivation of new inner RB region in extended CBW

- Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

Regarding the derivation of new inner RB region in extended CBW, the above approach was agreed as a starting point to be further discussed in #115 meeting WF. For the second formula to calculate RBStart,High_new, RBStart,Low_new should be revised to RBStart,Low. And the new inner RB region in extended CBW should be defined as follows:

- max(0, RBStart,Low – RBextension_low)  ≤  RBStart  ≤  min(NRB – LCRB, NRB + RBextension_high – RBStart,Low – LCRB),

- and LCRB  ≤  ceil((NRB+ RBextension_low + RBextension_high)/2).

Proposal 3: The new inner RB region in extended CBW should be defined as follows:

- max(0, RBStart,Low – RBextension_low)  ≤  RBStart  ≤  min(NRB – LCRB, NRB + RBextension_high – RBStart,Low – LCRB),

- and LCRB  ≤  ceil((NRB+ RBextension_low + RBextension_high)/2).

  - 2.2 Applicable requirements

Derivation of the extended IBE

- Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No

In last meeting, it was agreed that Fixed IBE used for extended CBW at each side of UE CBW as the baseline. The remaining issue is the applying range of the extended IBE, and whether to apply the extended IBE in the guard band of extended CBW. As shown in Figure 2.1-3, we propose that extended IBE will be applied ending at the last extended RB and SEM requirement will be applied starting from the last extended RB.

- Proposal 4: It is proposed that extended IBE will be applied ending at the last extended RB and SEM requirement will be applied starting from the last extended RB.

  - 2.3 Signalling

- Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side - Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

For signalling, we reached an agreement in last meeting that two UE capabilities will be introduced. One UE capability is only supporting 1/2 extension ratio, and another is supporting both 1/2 and 1/4 extension ratios. Regarding the interaction between UE and NW is that firstly, UE reports the capability of supported extension ratios, and secondly, NW judges the available gap width on both sides of the UE and indicates the extension ratio.

For NW signalling, we can use 3 bits to indicate the extension ratio at each side, which is shown in the following table.

Table 2.3-1: Extension ratio at each side

| Bits | Extension ratio at each side |
| --- | --- |
| 000 | (0, 0) |
| 001 | (0, 1/2) |
| 010 | (1/2, 0) |
| 011 | (1/2, 1/2) |
| 100 | (0, 1/4) |
| 101 | (1/4, 0) |
| 110 | (1/4, 1/4) |
| 111 | reserved |

Proposal 5: It is proposed to use 3 bits for NW signalling to indicate the extension ratio at each side.

For UE capabilities supporting MPR reduction, the features are based on the scenarios that no adjacent in-band/out-of- band co-existence issue or narrower UE channel bandwidth within wider BS channel bandwidth, which are related to actual deployment in a band. Hence, the UE capabilities supporting MPR reduction should be per band.

Proposal 6: The UE capabilities supporting MPR reduction should be per band.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: For MPR reduction feature, the extension bandwidth equals to extension ratio multiplied by UE max transmission bandwidth configuration NRB .

Proposal 2: Not to define the guard band for extended CBW.

Proposal 3: The new inner RB region in extended CBW should be defined as follows:

- max(0, RBStart,Low – RBextension_low)  ≤  RBStart  ≤  min(NRB – LCRB, NRB + RBextension_high – RBStart,Low – LCRB),

- and LCRB  ≤  ceil((NRB+ RBextension_low + RBextension_high)/2).

Proposal 4: It is proposed that extended IBE will be applied ending at the last extended RB and SEM requirement will be applied starting from the last extended RB.

Proposal 5: It is proposed to use 3 bits for NW signalling to indicate the extension ratio at each side.

Proposal 6: The UE capabilities supporting MPR reduction should be per band.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2507930, WF on NR_ENDC_RF_Ph4_part1, Huawei, HiSilicon