3GPP TSG RAN WG4 Meeting #116	R4-2510494

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.25.3.3

Source:	vivo

Title:	Further discussion on Conditional Intra-CU LTM

Document for:	Discussion

- Introduction

After the discussion of the past three meetings, the general aspect on defining delay requirements on CLTM is determined. In this paper, we will further provide our thinking on the details of the definition of delay requirements for supporting conditional Intra-CU LTM based on the latest WF [1].

- Discussion

<conditional LTM>

Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation. Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 < Agreement > ad hoc - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. - Note: the limitation of 160/480 ms from Rel-18 still applies.

For CSI-RS based L1-RSRP measurement, the conditions defined for existing CSI-RS based L3-RSRP measurement requirements can be reused, i.e.,TCSI-RS_SFN_intra = 0 if the UE is indicated that the neighbor cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled).

Regarding whether to include the unknown cell case, we believe that it can follow the previous CHO definition logic. In CHO, it doesn’t include the unknown cell case. The main reason is that once the CHO evaluation conditions are configured via RRC, the UE continuously evaluates whether the conditions are met. Therefore, before executing CHO, the UE has already measured the cell, so in CHO, it is always treated as a known cell. Following this logic, CLTM should also only consider the known cell case.

Proposal 1：For CSI-RS based L1-RSRP measurement:

- For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- TCSI-RS_SFN_intra = 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled)

- Similar to CHO, RAN4 not to include unknown cell case for CLTM

For the conditions Tfirst-RS = 0 and TRS-proc = 0, there was a preliminary discussion in the last meeting. However, there was no clear conclusion on how the Rel-18 limitation of 160/480 ms applies in detail. For CLTM, since there is no cell switch command (unlike in legacy LTM), the conditional cell switch is autonomously evaluated and triggered by the UE. Therefore, the ending point for the 160/480 ms requirement needs to be adapted for CLTM. Based on our understanding, this time point can be defined as when the UE starts cell switch execution to the target cell.

Proposal 2: For applying the 160/480 ms limitation from Rel-18 to Tfirst-RS = 0 and TRS-proc = 0 in CLTM, the existing condition where the timer starts upon receiving a cell switch command should be updated to instead start when the UE starts executing the cell switch to the target cell.

- Summary

Proposal 1：For CSI-RS based L1-RSRP measurement:

- For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- TCSI-RS_SFN_intra = 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled)

- Similar to CHO, RAN4 not to include unknown cell case for CLTM

Proposal 2: For applying the 160/480 ms limitation from Rel-18 to Tfirst-RS = 0 and TRS-proc = 0 in CLTM, the existing condition where the timer starts upon receiving a cell switch command should be updated to instead start when the UE starts executing the cell switch to the target cell.

- References

- R4-2508283 WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom