3GPP TSG-RAN4 Meeting #116	R4-2510626

# Bengaluru, India, 25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5893 | rev | - | Current version: | 17.18.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_MG_enh-Core) CR on core requirements for R17 MGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on current requirements for CSSF for NCSG, SCell in dormancy is measured within NCSG regardless of the UE reported measurement capability. However, SCell in dormancy is an activated SCell with active BWP. If the dormant BWP does not contain SSB and UE reports ‘gap’ for the SCell, the SCell should be measured with MG rather than NCSG, so measurement in NCSG only apply when dormant BWP contains SSB. On the other hand, if UE reports ‘ncsg’, the SCell should be measured with NCSG even it is in dormancy. However, this case is precluded by the wording “(in non-dormancy)” in the bullet for activated SCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the wording “(in non-dormancy)” in the bullet for activated SCell to lift restriction on measurement of activated SCell with NCSG. - Add condition that dormant BWP contains SSB for measurement of SCell in dormancy with NCSG regardless of the UE reported measurement capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement requirements for SCell in dormancy are incorrect in terms of whether to use MG or NCSG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.1.5.3	Monitoring of multiple layers within NCSG

The measurement requirements derived from CSSFwithin_ncsg,i defined in this clause are applicable provided that network provides NCSG pattern for measurement.

The carrier-specific scaling factor CSSFwithin_ncsg,i for a measurement object i derived in this clause is applied to following measurement types:

-	SSB-based intra-frequency measurement object without measurement gap as defined in clause 9.2.1 corresponding to an activated serving cell, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG;

-	SSB-based intra-frequency measurement object with NCSG as defined in clause 9.2.1 corresponding to an activated serving cell, when all or part of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG;

-	SSB-based intra-frequency measurement object corresponding to a deactivated Serving cell, or to an activated Serving cell in dormancy with SSB within the dormant BWP, when all or part of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG regardless of the UE capability reporting of intraFreq-needForNCSG;

-	SSB-based inter-frequency measurement object without measurement gap as defined in clause 9.3.1, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the NCSG;

-	SSB-based inter-frequency measurement object with NCSG as defined in clause 9.3.1;

-	E-UTRA inter-RAT measurement object, when the measurement can be performed with no measurement gap but NCSG as defined in clause 9.4;

UE is expected to conduct the measurement of this measurement object i only within the NCSG.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present for an intra-frequency measurement object, and smtc1 is fully overlapping with NCSG and smtc2 is partially overlapping with NCSG, requirements derived from CSSFwithin_ncsg,i and CSSFoutside_gap,i are not applicable.

<End of Change 1>