3GPP TSG-RAN WG4 Meeting #116	R4-2510677

# Bangalore, India, 25 Aug - 29 Aug, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rx beam factor optimization of R19 RRM enhancements

Agenda Item:	7.14.3.1

Document for:	Discussion

- Introduction

During last RAN4 meeting, around the multi-Rx based Rx beam sweeping factor optimization, the following progress were achieved and summarized in [1] as below:

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Issue 1-1-2: Status report of fast beam sweeping Agreement: - No need to define UE status report of fast beam sweeping. Issue 1-1-3: UE capability for Rx BSF optimization Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

In this paper, we provide our further views on the remaining issues of the reduced Rx beam sweeping factor on L3 measurements.

- Discussion

- Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Due to the concern on UE power consumption, some companies proposed to consider some quit mechanism to allow UE to quit the fast L3 measurement mode even the condition of fast L3 measurement activation is still valid. In last meeting, after warm discussion, some good progress were achieved around the quitting condition of FBS. There are still two suspending points, one is the exact length of T1, the other is how to design the re-evaluation mechanism to allow UE return to normal behavior, i.e. evaluate the activation condition.

Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

For the candidate value of T1, we believe it should make sure UE has enough SSB samples to finish the measurement or cell identification on the target frequency. So the solution used in how to specify the length of fast measurement window in on-demand SSB of R19 NES[2] could be a good reference to define the length of T1 here. Similarly, the candidate of T1 could be min(Treport , Tidentify), wherein Treport is until the time point of reporting the first L3 measurement report for the target frequency layer. Tidentify equals the minimum requirements of measurement period or cell identification.

Proposal 1: Regarding the candidate of T1, it should guarantee UE has enough SSB samples to finish the measurement or cell identification on the target frequency.

- it could be min(Treport, Tidentify), wherein Treport is until the time point of reporting the first L3 measurement report for the target frequency layer. Tidentify equals the minimum requirements of measurement period or cell identification.

For re-evaluation mechanism for scenario 1/2/3/4, it is necessary since anyway UE needs to return to the normal activation/deactivation condition evaluation to proceed the subsequent measurement. Since in last meeting, RAN4 approved that no need to define UE status report of fast beam sweeping. It seems we need to specify how long UE would hold on the quit status. In last meeting, the timer controlled re-evaluation mechanism was provided as one choice. The timer starts when UE quits from multi-Rx mode. While considering the candidate value of this timer, currently we can not find any technical reason to determine the candidate value. So it is better to define a fixed value instead of letting the NW to configure the value. The fixed value could be determined and take UE power consumption into account.

Proposal 2: UE re-evaluates the activation condition after T2, the starting point of T2 is when UE quits from Multi-Rx mode.

Revision on the previous approved applicable scenario for FBS

During the initial meetings, RAN4 firstly achieved consensus on the applicable scenario for FBS as below:

Applicability requirement: (112 meeting) Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Based on the above agreements, one understanding is that within all the intra-frequency and/or inter-frequency carriers, only one target carrier is configured for L3 SSB measurement, so the target carrier is either serving carrier or inter-frequency carrier. But if we look at the activation/deactivation condition of FBS, shown as below, it could be found that no matter the target carrier is intra-frequency or inter-frequency, the RSRP/RSRQ measured in serving cell(PCell in FR2-1) is always used to evaluate whether the condition meets.

Agreement in online session: (112bis meeting) For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. Agreement:(113 meeting) - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

As a result, no matter the target carrier is in the same frequency of serving cell or not, the L3 measurement in serving cell is always needed as the condition evaluation. The MO configuration is per frequency level, as long as the target frequency is different from the frequency of serving cell, actually two MOs should be configured, one is for the target carrier, the other is for the serving cell(PCell in FR2-1). So we believe some revision is needed on top of above applicability requirement for the sake of preciseness. We provide a revision as below:

Applicability requirement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. When they are different, NW can configure L3 SSB measurement on both target carrier and serving carrier. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

If the target frequency is inter-frequency, actually NW would configure two MOs, one is for PCC, the other is for this target frequency, based on RAN2 specification in TS 38.331 as below, the MO configuration in PCC is mandatory.

When two MOs are configured, some solution is needed to inform UE which MO is the target frequency to be accelerated by FBS.

Proposal 3: Since no matter the target frequency is same or not with the frequency of serving cell, the L3 SSB measurement on serving cell is always needed to be as the FBS activation/deactivation condition evaluation, the previous applicability requirement needs further revision as below:

Applicability requirement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. When they are different, NW can configure L3 SSB measurement on both target carrier and serving carrier. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Proposal 4: When two MOs are configured by NW, one is for PCC, the other is for the target frequency in which FBS is applicable, NW and UE should be aligned on which MO is the FBS target.

UE capability for Rx BSF optimization

Regarding the UE capability, the only suspending point is whether to separate UE capability into intra-frequency and intra-frequency+inter-frequency in same FG. We understand the request of such UE capability separation lies in the concern on searcher. If the target carrier is inter-frequency, which implies actually UE needs to perform both intra-frequency measurement at the serving cell(PCell in FR2-1) and inter-frequency measurement at the target carrier. At the beginning of FBS discussion, companies had some discussion on the searcher assumption but no consensus achieved. Considering different UE vendors may have different searcher implementation, so we are fine to separate this UE capability into intra-frequency and intra-frequency+inter-frequency in same FG.

Proposal 5: Fine to us to separate the UE capability for Rx BSF optimization into intra-frequency and intra-frequency+inter-frequency in same FG.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: Regarding the candidate of T1, it should guarantee UE has enough SSB samples to finish the measurement or cell identification on the target frequency.

- it could be min(Treport, Tidentify), wherein Treport is until the time point of reporting the first L3 measurement report for the target frequency layer. Tidentify equals the minimum requirements of measurement period or cell identification.

Proposal 2: UE re-evaluates the activation condition after T2, the starting point of T2 is when UE quits from Multi-Rx mode.

Proposal 3: Since no matter the target frequency is same or not with the frequency of serving cell, the L3 SSB measurement on serving cell is always needed to be as the FBS activation/deactivation condition evaluation, the previous applicability requirement needs further revision as below:

Applicability requirement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. When they are different, NW can configures L3 SSB measurement on both target carrier and serving carrier. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Proposal 4: When two MOs are configured by NW, one is for PCC, the other is for the target frequency in which FBS is applicable, NW and UE should be aligned on which MO is the FBS target.

Proposal 5: Fine to us to separate the UE capability for Rx BSF optimization into intra-frequency and intra-frequency+inter-frequency in same FG.

- Reference

- R4-2504900 “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT

- R4-2502598 WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114.