3GPP TSG-RAN4 Meeting # 116	R4-2510941

Bangalore, IN. 25th – 29th, 2025

# Source:	ZTE Corporation, CMCC

# Title:	TP for TR 38.746 HPUE CA_n8-n41

# Agenda Item:	6.8.5

# Document for:	Approval

- Introduction

In this contribution, a TP to TR38.746 is provided to introduce the following configurations, where total 2Tx with 1Tx one each band for inter-band UL CA or 2Tx for PC1.5 intra-band UL CA.

| Band combination configuration | UpLink configuration | Power class |
| --- | --- | --- |
| CA_n8A-n41A | n41 CA_n8A-n41A | PC2 |
| CA_n8A-n41A | n41 | PC1.5 |
| CA_n8A-n41C | n41 CA_n8A-n41A | PC2 |
| CA_n8A-n41C | n41 | PC1.5 |
| CA_n8A-n41C | n8 | PC2 |
| CA_n8A-n41C | n8 | PC2 |

- Reference

- [1] TR 38.746, High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19), v0.4.0

- Text Proposal

----- Start of TP -----

## 5.x	CA_8-n41

### 5.x.1	Operating bands for CA

Table 5.x.1-1:   NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n41A | n88 n418,9 CA_n8A-n41A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41C | n88 n418,9 CA_n41C CA_n8A-n41A8 CA_n8A-n41C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

Note: The above foot-script note number is same as clause 5.5A.3.1 in TS38.101-1

Note: On top of the existing specification, the new added configurations are highlighted in yellow for convenience.

### 5.x.2	Maximum output power for inter-band CA

The UE Power class 2 for uplink Configuration for CA_n8-n41 is shown in Table 5.6.2-1.

Table 5.12.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n8-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.x.3	MSD scenario studies

For harmonic MSD by HPUE n8 or n41:

- According to the PC3 co-existence study, UL3 harmonic of n8 falls into DL1 of n41, and there is no ULx harmonic of n41 falls into DL1 of n8.

For harmonic mixing MSD caused by HPUE UL n8 or n41:

- According to the PC3 co-existence study, there are no harmonic mixing issues when the UL is n8 or n41.

For cross-band isolation MSD by HPUE UL n8 or n41:

- According to the PC3 co-existence study, there are no cross-band isolation issues when the UL is n8 or n41.

For IMD MSD caused by HPUE UL CA_n8A-n41A

- According to the PC3 co-existence study, there is IMD3 issue (fn41-2*fn8 fall into n8 DL) and IMD5  (4*fn8 - fn41 fall into n8 DL) fall into n8 DL)caused by CA_n8A-n41A.

### 5.x.3	REFSENS requirements

#### 5.x.3.1	REFSENS requirements for total power class 2

For PC2 UL n8, the MSD is already defined in the specification. Comparing the existing MSD in TS38.101-1, there are no additional harmonic MSD requirement needed to be specified.

For HPUE UL CA_n8A-n41A, the current PC3 IMD MSD is defined as 12.1dB, and no IMD5 MSD is defined.

By using the LUT approach in R4-2507935, the IMD3 MSD caused by PC2 UL CA_n8A-n41A is 21.1dB (=12.1+9dB), which looks like in table 5.x.3.1-1.

Table 5.x.3.1-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 21.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

However, due to the introduction of LUT approach for HPUE MSD simplification, it is suggested not to add the above specific new HPUE MSD in the specification.

#### 5.x.3.2	REFSENS requirements for total power class 1.5

Comparing the existing MSD in TS38.101-1, there are no additional harmonic MSD requirement needed to be specified.

----- End of TP -----