3GPP TSG-RAN WG4 Meeting #116	R4-2510961

Bengaluru, IN, August 25th – 29th 2025

Agenda Item:	7.21.2.1

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding SBFD sub-band configuration. Analysis of SBFD BS RF TX/RX requirements and new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

## 2.1	SBFD subband configuration

### Sub-topic 1-1: Declaration and flexibility for SBFD operation ##### Issue 1-1-1: Declaration and flexibility for SBFD operation Agreement: - For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting. ### Sub-topic 1-2: DL/UL subband configurations support for SBFD Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. ##### Issue 1-2-2: Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

The left-over issue for the two sub-topics/three issues above are the same and focused on FR1 MR SBFD-capable BSs, and it is agreed that a power level threshold should be established: Above this threshold, analogue filters are expected to be used, and the restrictions above are applicable as FR1 WA BS; Below this threshold, analogue filters are not expected to be used, and the restrictions above are not applicable.

Considering the BS rated carrier TRP output power for BS type 1-O defined in table 9.3.1-1 in TS 38.104 [4], we propose using 33 dBm either as TRP or total conducted power as the power level threshold.

While the introduced power level threshold may appear to limit the maximum number of subband configurations for FR1 MR SBFD-capable BS, it is important to note that above this threshold, an analog filter is required for each subband configuration. As a result, there is a practical limit to the number of analog filters that can be supported by a single base station. Therefore, the selection of the power level threshold should be based on practical considerations.

While the introduced power level threshold may appear to limit the maximum number of subband configurations for FR1 MR SBFD-capable BS, it is important to note that above this threshold, an analog filter is required for each subband configuration. As a result, there is a practical limit to the number of analog filters that can be supported by a single base station. Therefore, the selection of the power level threshold should be based on practical considerations.

Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

## 2.2	Conformance testing for SBFD-capable BS

Declarations

With the introduction of SBFD feature in 3GPP, the specific SBFD-related declarations need to be incorporated in TS 38.141-1/-2 for perform conformance testing of SBFD-capable BS. Below is a summary of the declarations that have been agreed upon in past discussions during the Rel-19 meetings.

- Declaration of TX power for SBFD.

- Declaration of bands supported for SBFD.

- Declaration of channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104.

- Declaration of SBFD subband configuration(s) per supported channel bandwidth, including

  - SBFD pattern (DUD or DU/UD),

  - size of DL subband(s),

  - size of UL subband,

  - size of guardband(s) between subbands.

Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	While the introduced power level threshold may appear to limit the maximum number of subband configurations for FR1 MR SBFD-capable BS, it is important to note that above this threshold, an analog filter is required for each subband configuration. As a result, there is a practical limit to the number of analog filters that can be supported by a single base station. Therefore, the selection of the power level threshold should be based on practical considerations.

Observation 2	Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

Proposal 1	Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

# References

R4-2510963, Impact on SBFD BS RF TX requirements, Ericsson

R4-2510964, Impact on SBFD BS RF RX requirements, Ericsson

R4-2510962, New SBFD BS RF requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP