# 3GPP TSG-RAN WG4 Meeting #116                                R4-2510974

# Bangalore, India, 25th – 29th August, 2025

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.13.4

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], the objectives related to NR BS MMSE-IRC receiver are listed :

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial discussion on the NR BS MMSE-IRC receiver.

# 2 Discussion

Based on the conclusion in RAN4 #115 meeting [2], only one open issue is still undecided for BS demodulation performance requirements MMSE-IRC.

Applicability for different BS classes and test scenarios

- Online agreement: - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

In LTE requirements, RAN4 defined applicability rule to distinguish different BS types based on deployment assumption, which is common understanding that the HomNet requirements are not applicable for local area BS and HetNet requirements are not applicable for wide and medium ares BS. Hence, we believe that this also can be used for NR requirements.

As illustrated before, HomNet requirements is applicable for wide and medium area BS, and HetNet is applicable for local area BS. If a BS declared to support wide and medium area BS, which means that the HetNet requirements are not applicable wide and medium ares BS, why we need to skip the test for HetNet. No HetNet requirements are defined for wide and medium area BS. Hence, we think option 1 needs to be further clarified. We understand that the motivation of introducing more applicable rules to reduce the test effort. However, more rules bring more complexity for RAN4 specification. Hence, we propose to do not define additional applicability rule for homogenous and heterogenous network scenarios

Proposal 1. Propose not to define additional applicability rule for homogenous and heterogenous network scenarios.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of MMSE-IRC receiver for NR BS, The conclusions are:

Proposal 1. Propose not to define additional applicability rule for homogenous and heterogenous network scenarios.

# 4	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5.

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS.