3GPP TSG-RAN WG4 Meeting #116	R4-2510994

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia

Title:	On Demodulation and CSI Reporting Requirements for CSI Prediction

Agenda item:	7.17.7

Document for:	Discussion

Introduction

The open issues to be discussed in RAN4#115 was documented in Topic Summary for NR_AIML_air_part1 [1]. The ones that relate to demodulation and CSI reporting requirements are summarized below:

Sub-topic 1-1 Requirements baseline One company proposed to also consider Rel-18 eType II follow PMI performance relative to Rel-15 single Tx codebook when defining the requirements Issue 1-1: Requirement considerations - Proposals - Option 1: consider Rel-18 eType II follow PMI performance relative to Rel-15 single Tx codebook - Performance delate should be higher? - Option 2: no need to consider performance of other schemes when defining the requirements - Recommended WF Option 1 Sub-topic 1-2 PMI report and scheduled PDSCH separation The time distance between the report with the predicted PMI and the actual scheduled PDSCH should be discussed Issue 1-2: PMI Report and Scheduled PDSCH separation - Proposals - Option 1: Reduce the time separation between PMI report and scheduled PDSCH relative to the values assumed in Rel-18 prediction tests - RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)” - Option 2: Reuse the same values as in Rel-18 prediction tests - Option 3: others - Recommended WF - To be discussed Sub-topic 1-3 100Hz Doppler handling Several companies are proposing to de-prioritize or eliminate 100Hz Doppler from the set of considered values Issue 1-3: Doppler handling - Proposals - Option 1: De-prioritize 100Hz Doppler - Consider 100Hz Doppler in the future if needed - Option 2: Eliminate 100Hz Doppler from the set of considered values - Option 3: Keep 100Hz Doppler, further study the performance and consider defining a test - Recommended WF - Option 1 Sub-topic 1-4 SNR handling Some companies raised the issue of what SNR to be used Issue 1-4: SNR handling - Proposals - Option 1: RAN4 should discuss how to choose the SNR range for AI/ML based prediction. Following is the suggestion from R4-2505974: - Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration - Option 2: other proposals - Option 3: there is no need to fix the SNR - Recommended WF - TBD Sub-topic 1-5 SNR levels for training data and generalization The issue of whether the SNR for the training data should be discussed agreed was brought up. This issue could also be related with generalization if different there would be different tests with different SNR levels. Issue 1-5: SNR levels for training data and generalization - Proposals - Option 1: Agree in RAN4 different SNR levels for training data - Option 2: Training data generation should be left to implementation - Option 3: No need to agree anything explicit in RAN4 about training data, UE performance at different SNR levels should be tested - Option 4: No need for any discussion on training data or such generalization tests - Recommended WF - To be discussed Sub-topic 1-6 Simulation results and next steps Several companies submitted simulation results and proposals for next steps (refinement of parameters, etc) Issue 1-6: Simulation results and next stepts - Proposals - Discuss the simulation results and next steps - Simulation results - Parameter refining if needed - Next steps - Recommended WF - To be discussed Discussion on refinement of simulation parameters and next stepsto be done mainly offline Sub-topic 1-7 Generalization Several companies proposed to further study generalization issues and how to ensure that the UE performance does not degrade under different conditions. Issue 1-7: Generalization - Proposals - Option 1: Mix several static conditions to endure the generality of the prediction model - Option 2: Introduce tests with different channel models - Option 3: Use different antenna parameters - Option 3: Test under different SNRs - Option 4: further investigate the granularity of Doppler spread for which a new AI/ML model would be needed - Option 5: others - Recommended WF - Postpone discussion to next meeting It was agreed in a previous meeting that generalization would be discussed later on when the requirements and tests details (e.g. conditions, etc) would become more clear. Companies can still consider the above proposals when discussing the definitions of the requirements and tests.

During RAN4#115 discussions, several agreements on some of the above-mentioned open issues were made. They are documented in the way forward of Rel-19 Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface in [2] and summarized below:

CSI reporting requirement framework for CSI prediction Agreements in main session: Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

As shown above, there were 3 open issues from [1] that have been discussed and agreed online in RAN4#115. Hence, the remaining open issues need to be further revisited and discussed in RAN4#116.

This contribution provides our views on the remaining issues in demodulation and/or CSI reporting requirements for CSI prediction for discussions in RAN4#116.

Discussion

PMI Report and Scheduled PDSCH separation

The first open issue is about PMI Report and Scheduled PDSCH separation as below [1]:

Issue 1-2: PMI Report and Scheduled PDSCH separation - Proposals - Option 1: Reduce the time separation between PMI report and scheduled PDSCH relative to the values assumed in Rel-18 prediction tests - RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)” - Option 2: Reuse the same values as in Rel-18 prediction tests - Option 3: others - Recommended WF - To be discussed

Rel-19 NR AIML air part1 is the first release for CSI reporting requirements with AIML. Its benefits over Rel-18 non-AIML should be assessed carefully under the same setups. Reducing the time separation between PMI report and scheduled PDSCH relative to the values assumed in Rel-18 prediction tests will make the UE with AIML CSI prediction experiencing a lesser channel aging effect, and, hence, it performs better in the test.

As documented in [3] in the simulation assumption, for FDD it is stated that: “…this reported PMI cannot be applied at the gNB downlink before slot#(n+4)”. Hence, RAN4 should reuse the same value of time separation between PMI report and scheduled PDSCH (i.e., slot#(n+4)) for FDD as in Rel-18 prediction tests and not to reduce it (to slot#(n+3)).

Concerning the time separation between PMI report and scheduled PDSCH, RAN4 to keep the same value as in the previously agreed simulation assumption (i.e., slot#(n+4)) and not to reduce it (to slot#(n+3)).

SNR handling

The next open issue is about SNR handling as below [1]:

Sub-topic 1-4 SNR handling Some companies raised the issue of what SNR to be used Issue 1-4: SNR handling - Proposals - Option 1: RAN4 should discuss how to choose the SNR range for AI/ML based prediction. Following is the suggestion from R4-2505974: - Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration - Option 2: other proposals - Option 3: there is no need to fix the SNR - Recommended WF - TBD

In the real world, the UE with AIML-based predictions is expected to deal with different SNR values. Hence, the UE with AIML-based prediction should be able to make reliable predictions in different SNR values as in the field, and not only in a specific SNR range targeted for the testing. Fixing the range of SNR for data collection for AI/ML training would ensure the AIML-based prediction performs well when it is experiencing the SNR within the range of SNR for the training. Nonetheless, its performance in SNR values outside the SNR range for training might be degraded due to overfitting.

If RAN4 needs to fix the SNR range for data collection for AIML training, the SNR range should cover a certain range of throughput. For CSI reporting requirements, the common throughput considered is the 90% throughput, while for demodulation performance requirements the considered throughput is the 70% throughput. Hence, the ±X dB range (from the SNR of 90% throughput) should cover also the SNR of 70% throughput.

Concerning the SNR handling for data collection for AI/ML training, RAN4 to first discuss whether fixing the SNR within only a certain range would not result in overfitting.

If RAN4 agrees to fix the SNR range, the SNR range should cover both the SNRs of 90% and 70% throughput.

Simulation results and next steps

The next open issue is about Simulation results and next steps as below [1]:

Sub-topic 1-6 Simulation results and next steps Several companies submitted simulation results and proposals for next steps (refinement of parameters, etc) Issue 1-6: Simulation results and next stepts - Proposals - Discuss the simulation results and next steps - Simulation results - Parameter refining if needed - Next steps - Recommended WF - To be discussed Discussion on refinement of simulation parameters and next stepsto be done mainly offline

As mentioned above, the discussion on refinement of simulation parameters and next steps was done offline during RAN4#115. The outcome of the discussions is documented in [3].

In [3], two different SGCSs for CSI prediction step2 were considered, namely, SGCS2 and SGCS3. SGCS2 is calculated by comparing the feedbacked SVD of model output with the SVD of the ground truth Raw channel; and SGCS3 is calculated by comparing the feedbacked SVD of model output with the feedbacked SVD of the ground truth Raw channel. Figure 1 shows how the SGCS is calculated [3][4].

Figure 1 Calculation of SGCS after CSI Feedback [3][4]

It can be observed that SGCS2 and SGCS3 are different. As SGCS3 considers the codebook for both the model and the ground truth, its performance is better than SGCS2.

From the simulation results collection in RAN4#115, for CSI prediction step2, SGCS2 and SGCS3 show different results.

As SGCS3 considers the same codebook for both the model and the ground truth, it is a more appropriate measure of similarity compared to SGCS2 which only considers the codebook only for the model.

RAN4 to prioritize SGCS3 over SGCS2 in the evaluation.

Generalization

The next open issue is about Generalization as below [1]:

Sub-topic 1-7 Generalization Several companies proposed to further study generalization issues and how to ensure that the UE performance does not degrade under different conditions. Issue 1-7: Generalization - Proposals - Option 1: Mix several static conditions to endure the generality of the prediction model - Option 2: Introduce tests with different channel models - Option 3: Use different antenna parameters - Option 3: Test under different SNRs - Option 4: further investigate the granularity of Doppler spread for which a new AI/ML model would be needed - Option 5: others - Recommended WF - Postpone discussion to next meeting It was agreed in a previous meeting that generalization would be discussed later on when the requirements and tests details (e.g. conditions, etc) would become more clear. Companies can still consider the above proposals when discussing the definitions of the requirements and tests.

As mentioned above, the generalization will be discussed and refined once the requirements and test details become clearer. It is expected that in RAN4#116 companies will further discuss the refinement of the test parameters (possibly offline and together with the discussions of simulation results in Issue 1-6).

Conclusion

In this contribution, we provide our views on open issues in Rel-19 NR AIML air part1. The following Observations and Proposals were made:

Proposal 1: Concerning the time separation between PMI report and scheduled PDSCH, RAN4 to keep the same value as in the previously agreed simulation assumption (i.e., slot#(n+4)) and not to reduce it (to slot#(n+3)).

Proposal 2: Concerning the SNR handling for data collection for AI/ML training, RAN4 to first discuss whether fixing the SNR within only a certain range would not result in overfitting.

Proposal 3: If RAN4 agrees to fix the SNR range, the SNR range should cover both the SNRs of 90% and 70% throughput.

Observation 1: From the simulation results collection in RAN4#115, for CSI prediction step2, SGCS2 and SGCS3 show different results.

Observation 2: As SGCS3 considers the same codebook for both the model and the ground truth, it is a more appropriate measure of similarity compared to SGCS2 which only considers the codebook only for the model.

Proposal 4: RAN4 to prioritize SGCS3 over SGCS2 in the evaluation.

- References

- R4-2507570, Topic summary for [115][131] NR_AIML_air_part1, 3GPP RAN4#115, Qualcomm, Malta, May 19-23, 2025

- R4-2508080, WF on NR_AIML_air_part1, 3GPP RAN4#115, Qualcomm, Malta, May 19-23, 2025

- R4-2507940, The summary for CSI prediction simulation results, 3GPP RAN4#115, Oppo, et. al, Malta, May 19-23, 2025

- R4-2505513, CSI Prediction: Simulation Results, 3GPP RAN4#115, Qualcomm, Malta, May 19-23, 2025