33GPP TSG-WG4 Meeting #116	R4-2511091

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on interruption requirements for Type 4 EN-DCNR-CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025/8/4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RRM interruption requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated scenario should be specified based on the agreements we achieved in #112~#114-bis separately - RAN2 agreements: RAN2#130 - BS signaling For BS signaling, introduce Rel-19 nonCollocatedTypeNR-CA/MRDC-r19 with two code points {Type 1, Type 4} into CellGroupConfig, applicable when MaxMIMO-layers is configured as 4. If Rel-19 nonCollocatedTypeNR-CA/MRDC-r19 is absent, Rel-19 UE(s) apply Rel-18 principle accordingly. - UE capability: Introduce per-BC UE capability interBandMRDC-WithOverlapDL-Bands-r19 into MRDC-Parameters, and per-BC UE capability intraBandNR-CA-non-collocated-r19 into CA-ParametersNR. Capture that if a UE supports Type 4 UE capabilities, it also reports support on corresponding Type 2 UE capabilities.. - RAN4 RF LS (R4-2505161): |
| Summary of change: |  | Define RRM interruption requirements for Type 4 UE supporting intra-band non-collocated NR-CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no RRM interruption requirements to support intra-band non-collocated ENDC/NR-CA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.3, 8.2.1.2.4, 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ...38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slots, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to X1 slots, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is not provided with nonCollocatedTypeMRDC-r18, or

-	of up to max{Y1 slots + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE capable of requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

-	if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if UE supporting requirementTypeIndication-r18 is capable of [interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

3> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

4>	of up to X1 slots

3>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

4> of up to max{Y1 slots + TSMTC_duration, 5ms}

3>else if [nonCollocatedTypeMRDC-v19xy] is not provided

4> of up to max{Y1 slots + TSMTC_duration, 5ms}

2> else if UE not supporting requirementTypeIndication-r18 indicates it is capable of [interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

3> of up to X1 slots

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if [nonCollocatedTypeMRDC] is not provided

3>	of up to X1 slots

2> else

3> of up to max{Y1 slots + TSMTC_duration, 5ms}

or

-	if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of[ interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

1>	of up to max{Y1 slots + TSMTC_duration, 5ms}

Where X1 and Y1 are specified in table 8.2.1.2.3-1.

When one SCell in SCG is added or released, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slots + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot,

-	if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band, and UE is capable of [intraBandNR-CA-non-collocated-r19],

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if [nonCollocatedTypeNR-CA-v19xy] is presented with type 4

3>	of up to X1 slots

2>else if [nonCollocatedTypeNR-CA-v19xy] is presented with type 1

3>	of up to Y1 slots + TSMTC_duration

2>else if [nonCollocatedTypeNR-CA-v19xy] is not provided

3> of up to Y1 slots + TSMTC_duration

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA is not provided

3>	of up to X1 slots

2> else

3> of up to Y1 slots + TSMTC_duration

or

-	if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of [intraBandNR-CA-non-collocated-r19]

1>	of up to Y1 slots + TSMTC_duration

,where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

-	of up to Y1 slots + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in table 8.2.1.2.3-2.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | N/A | - N/A |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell in MCG is activated from deactivated or dormant state, or deactivated from activated or dormant state, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slots, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to X2 slots, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being activated or deactivated and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and not provided with nonCollocatedTypeMRDC-r18, or

-	of up to max{Y2 slots + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG.

-	if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if UE supporting requirementTypeIndication-r18 is capable of [interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

3> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

4>	of up to X2 slots

3>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

4> of up to max{Y2 slots + TSMTC_duration, 5ms}

3>else if [nonCollocatedTypeMRDC-v19xy] is not provided

4> of up to max{Y2 slots + TSMTC_duration, 5ms}

2> else if UE not supporting requirementTypeIndication-r18 indicates it is capable of [interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

3> of up to X2 slots

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if nonCollocatedTypeMRDC is not provided

3>	of up to X2 slots

2> else

3> of up to max{Y2 slots + TSMTC_duration, 5ms}

or

-	if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicate it is capable of [interBandMRDC-WithOverlapDL-Bands-r19] on this band pair

1>	of up to max{Y2 slots + TSMTC_duration, 5ms}

Where X2 and Y2 are specified in table 8.2.1.2.4-1.

When one SCell in SCG is activated or deactivated, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cells and the SCells being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slots + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot,

-	if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band, and UE is capable of [intraBandNR-CA-non-collocated-r19],

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

3>	of up to X1 slots

2>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

3>	of up to Y1 slots + TSMTC_duration

2>else if [nonCollocatedTypeMRDC-v19xy] is not provided

3> of up to Y1 slots + TSMTC_duration

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA is not provided

3>	of up to X1 slots

2> else

3> of up to Y1 slots + TSMTC_duration

or

-	if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of [intraBandNR-CA-non-collocated-r19]

1>	of up to Y1 slots + TSMTC_duration

,where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

-	of up to Y2 slots + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

Where X2 and Y2 are specified in table 8.2.1.2.4-2.

Table 8.2.1.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length | nterruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | N/A | N/A |

Table 8.2.1.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and 7 is added or released using the same RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any active serving cell during the RRC reconfiguration procedure as follows:

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

Where X1 is specified in table 8.2.2.2.1-1.

or

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

-	if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band, and UE is capable of [intraBandNR-CA-non-collocated-r19],

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

3>	of up to X1 slots

2>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

3>	of up to the duration shown in table 8.2.2.2.1-2

2>else if [nonCollocatedTypeMRDC-v19xy] is not provided

3> of up to the duration shown in table 8.2.2.2.1-2

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA is not provided

3>	of up to X1 slots

2> else

3> of up to the duration shown in table 8.2.2.2.1-2

or

-	if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of [intraBandNR-CA-non-collocated-r19]

1>	of up to the duration shown in table 8.2.2.2.1-2

Table 8.2.2.2.1-1: Interruption length X1 for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0ms; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

8.2.2.2.2	Interruptions at SCell activation/deactivation

When an SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed an interruption on any active serving cell:

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slots, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18] and [nonCollocatedTypeNR-CA-r18 is not provided.

Where X2 is specified in table 8.2.2.2.2-1.

or

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

-	if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band, and UE is capable of [intraBandNR-CA-non-collocated-r19],

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

3>	of up to X2 slots

2>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

3>	of up to the duration shown in table 8.2.2.2.2-2

2>else if [nonCollocatedTypeMRDC-v19xy] is not provided

3> of up to the duration shown in table 8.2.2.2.2-2

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA is not provided

3>	of up to X2 slots

2> else

3> of up to the duration shown in table 8.2.2.2.2-2

or

-	if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of [intraBandNR-CA-non-collocated-r19],

1> of up to the duration shown in table 8.2.2.2.2-2

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

8.2.2.2.3	Interruptions during measurements on deactivated SCC

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

-	If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 on this FR1 band and nonCollocatedTypeNR-CA-r18 is not provided, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band, and UE is capable of [intraBandNR-CA-non-collocated-r19],

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if [nonCollocatedTypeMRDC-v19xy] is presented with type 4

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2>else if [nonCollocatedTypeMRDC-v19xy] is presented with type 1

3>	the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA is not provided

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2> else

3> the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

or

-	if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of [intraBandNR-CA-non-collocated-r19]

1> the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is in the same FR2 band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

The interruption requirements in table 8.2.2.2.3-1 are not applicable when a UE is configured with NCSG in the same frequency range as the deactivated SCell unless the SMTC on the deactivated SCC is fully non-overlapped with NCSG.

Table 8.2.2.2.3-1: Interruption duration for measurement on deactivated SCC for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

End of Change 1