3GPP TSG-RAN WG4 Meeting#116                                R4-2511129

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.2

Source: 	ZTE Corporation, Sanechips

Title: 	TP to TS38.191 FRC for device 1 REFSENS

Document for:	Approval

Introduction

- In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to provide the TP to TS38.191 on FRC for device 1 REFSENS requirements.

References

- RP-250796, Core part: Solutions for Ambient IoT (Internet of Things) in NR, Approved.

- TS 38.191 NR; Ambient IoT device radio transmission and reception, approved.

Annex A (normative):
Reference measurement channels

A.1	Fixed Reference Channels for reference sensitivity level (OOK)

| Reference channel | Configuration | A-FR1-B1-1 | A-FR1-B1-2 | A-FR1-B1-3 | A-FR1-B1-4 |
| --- | --- | --- | --- | --- | --- |
| SIP | SCS | 15 | 15 | 15 | 15 |
|  | PRB | 1 | 2 | 3 | 4 |
|  | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
|  | chip number except for SIP,padding | 228 | 228 | 228 | 228 |
|  | Padding | 6 | 12 | 2 | 2 |