3GPP TSG RAN WG4 Meeting #116	R4-2511265

Bangalore, India 25th Aug. 2025 – 29th Aug. 2025

Agenda Item:	7.8.5

Source: 	Ericsson

Title:	Discussion on Rel-19 ATG CA RRM Core Issues

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on all general issues regarding the enhancement of the ATG CA, and one issue is still open before finalizing the Core part. The approved WF of RAN4#115 can be found in [1]. This contribution discusses the remaining core issue related to ATG CA enhancement.

# Discussion

## Sharing Factor between L1 measurement and RRM measurement

In RAN4#114-bis, one of the companies proposed adding one extra symbol both before and after each consecutive SSB symbol, as well as the RSSI symbols of the serving cells, when inter-band carrier aggregation within FR1 is configured and the UE supports Case 4. The proposal captured in the WF [2] is as follows:

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - Proposal 1: (LG) - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

We believe the assumption that a 33μs MRTD results in the loss of an entire data symbol is conservative, especially when the ATG DL UE is co-located and in line of sight, where the usual RTD is often no more than 5 μs corresponding to only ~0.14% of a symbol at 30 kHz SCS. Even under the maximum MRTD of 33 µs, the impact is ~0.93% of a symbol at 30 kHz SCS. UEs with a common Rx beam should support sufficient beam-switching timing and accuracy, while advanced receiver tracking and signal processing can further minimize any effective loss. The existing ±1-symbol guard around SSB/SS-RSSI occasions already accounts for beam-switch transients in generic cases. Moreover, there is no study motivating significant measurement inaccuracies or radio link degradation due to a 33 μs MRTD. Hence, we believe the baseline specification is valid, and no extra relaxation is needed in this case.

RAN4 should agree to reuse the legacy specification, i.e.

- 	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations and proposals.

No table of figures entries found.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should agree to reuse the legacy specification, i.e.

	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

# References

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC