3GPP TSG RAN WG4 Meeting #116	R4-2511266

Bangalore, India, 25 August 2025 – 29 August 2025

Agenda Item:	7.8.6

Source: 	Ericsson

Title:	Discussions on Rel-19 ATG CA RRM Performance Enhancement remaining issues

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on the test method and scaling factor for UE with antenna array. The approved WF of RAN4#115 can be found in [1]. This contribution addresses the remaining issues related to the ATG CA enhancement performance, specifically the test applicability rule, test setup, and test scope.

# Discussion

In this contribution, we discuss our initial views on the performance requirements based on the agreements regarding core requirements and the remaining core requirements' open issues.

## Test applicability rule

Issue 2-1-1: Test applicability rule - Proposal 1: - UE is only required to be tested in one of the supported test configurations - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration - Proposal 2: - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

To minimize test cases, test applicability rules are commonly applied. While configurations for NR PCell and SCell can be selected independently for flexibility, this may lead to excessive combinations and increased testing efforts. To align with existing RRM test principles for 3GPP and reduce the testing burden, we propose defining the test applicability rule for Rel-19 ATG test cases as follows:

RAN4 should discuss and agree on the following test applicability rule

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

## Test Setups

In RAN4#114-bis, we agreed on the following test setups.

Issue 2-2-3: Test setups Agreement: - Following test setups agreed in R18 can be reused: - TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell - Channel model: AWGN with residual doppler channel model - Number of neighbour cells: one neighbour cell in the test if needed - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system. - BS location: The specific gNB reference location is emulated by test system - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. - OCNG: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

For TDD pattern, RAN4 agreed that the legacy TDD pattern can be used for PCell, but there was still FFS for SCell. While legacy TDD configurations such as TDDConf.1.1 and TDDConf.2.1 can be reused for testing purposes, these patterns include UL slots that are not utilized in ATG DL CA scenarios. Therefore, reusing legacy patterns leads to underutilization of the available TDD frame resources.

For intra-band CA (e.g. n79), synchronization requirements necessitate that SCell and PCell use identical TDD configurations to avoid scheduling conflicts and ensure proper timing operation.

For intra-band CA involving TDD bands, The SCell should reuse the same TDD configuration as the PCell, as defined in clause A.3.1.4 of 38.133, to ensure synchronization and scheduling integrity.

For inter-band CA (e.g., n3_n79), such synchronization constraints do not apply between FDD PCell and TDD SCell. Thus, in real network, a new TDD pattern that maximizes downlink transmission by using all SCell slots for DL can improve spectral efficiency. Hence, RAN4 should define new TDD pattern that captures real networks characteristics. TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2 proposed below should be discussed and captured for 15kHz and 30kHz, respectively.

RAN4 should discuss and add TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2.

Table 1: TDD UL/DL configuration for SCS=15 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 | TDDConf.1.3 |
| referenceSubcarrierSpacing | kHz | 15 | 15 | 15 |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ | ‘D’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 | 1 |
| nrofDownlinkSlots |  | 1 | 1 | 1 |
| nrofDownlinkSymbols |  | 10 | 6 | 0 (full slot DL) |
| nrofUplinkSlot |  | 2 | 2 | 0 |
| nrofUplinkSymbols |  | 2 | 6 | 0 |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ | ‘D’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 | 1 |
| nrofDownlinkSlots |  | 1 | 1 | 1 |
| nrofDownlinkSymbols |  | 0 | 0 | 0 |
| nrofUplinkSlot |  | 0 | 0 | 0 |
| nrofUplinkSymbols |  | 0 | 0 | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information. |  |  |  |  |

Table 2: TDD UL/DL configuration for SCS=30 kHz

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 | 1 |
| nrofDownlinkSlots |  | 3 | 1 | 3 | 2 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 | 0 |
| nrofUplinkSlot |  | 4 | 2 | 4 | 0 |
| nrofUplinkSymbols |  | 4 | 2 | 6 | 0 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 | 1 |
| nrofDownlinkSlots |  | 2 | Not configured | 2 | 2 |
| nrofDownlinkSymbols |  | 0 | Not configured | 0 | 0 |
| nrofUplinkSlot |  | 0 | Not configured | 0 | 0 |
| nrofUplinkSymbols |  | 0 | Not configured | 0 | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |  |

## Test Scope

In previous meeting, a proposal for test cases was merged into a table in issues 2-2-5 [1]. We agreed to discuss which test cases should be considered. Below, one can find comments on the scope of each test case.

Table 3: Test case Lists for ATG CA in Rel-19

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.3 |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.4 |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case for SCell known based on A.6.5.3.1 |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | Modify Test case for SCell known based on A.6.5.3.2 |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | Modify Test case for SCell unknown based on A.6.5.3.3 |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | Modify Test case for Direct SCell known based on A.6.5.3.4 |
|  | Direct SCell activation at handover with known SCell in FR1 | Modify Test case for Direct SCell known based on A.6.5.3.5 |
|  | Direct SCell Activation at RRC Resume | The motivation to introduce this new test case is not clear. No test case required. |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case for Fast SCell known based on A.6.5.3.10 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | Modify Test case for Fast SCell known based on A.6.5.3.11 |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | Modify Test case based on A.6.5.3.14 |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | Modify Test case based on A.6.5.3.15 |
|  | Inter-band SSB-less SCell Activation based on A-TRS | Modify Test case based on A.6.5.3.16 |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Modify test case based on A.6.5.5 |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | Modify test case based on A.6.6.3 |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | Modify test case based on A.6.6.3 |
|  | non-simultaneous timer-based BWP switch delay on multiple CCs | No test case required. |
| SCell dormancy switch | SCell dormancy switch | Motivation for this test case is not clear. Further discussion needed. |
| Deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | No test case required. |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | New test case required for Pre-MG activation/deactivation delay |
| Others | Intra-frequency measurement (with scheduling restriction verified as well) | FFS for Case 4 UEs and different subcarrier spacing. |
|  | Inter-frequency measurement (with scheduling restriction verified as well) | FFS for Case 4 UEs and different subcarrier spacing. |
|  | L1-RSRP measurement | L1-RSRP measurements for beam reporting for ATG PCell are already defined in A.19.5.3. Since the SCell and PCell are co-located, we do not see any need to introduce this for SCell. |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | New test case required based on A.6.6.10 and A.6.6.11 for L3 measurement |
|  | The interruption of SCell addition/release | Motivation for this test case is not clear. Further discussion needed. |
|  | The interruption of SCell activation/deactivation | Motivation for this test case is not clear. Further discussion needed. |

RAN4 should discuss and finalize the test cases based on Table 3 and decide on the work split during this meeting.

RAN4 should discuss and agree on the test cases based on Table 3.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations.

Observation 1	For intra-band CA involving TDD bands, The SCell should reuse the same TDD configuration as the PCell, as defined in clause A.3.1.4 of 38.133, to ensure synchronization and scheduling integrity.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should discuss and agree on the following test applicability rule

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

Proposal 2	RAN4 should discuss and add TDDConf.1.3 in Table 1 and TDDConf.2.4 in Table 2.

Proposal 3	RAN4 should discuss and agree on the test cases based on Table 3.

# References

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC