# 3GPP TSG-RAN WG4 Meeting #116		        R4-2511418

# Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	7.27.3.1

Source: 	Huawei, HiSilicon

Title:	On remaining open issues in NR NTN Ph3

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in the last meeting, listing a number of open issues under the topics of:

- Output power enhancement for NR NTN PC3

- UL capacity enhancements

- DL coverage enhancements.

In the sequel, we share our views on how to conclude the remaining open issues.

# 2	Discussion

## 2.1 Output power enhancement for NR NTN PC3

The agreements in the WF are as follows.

RAN4#115 Agreements:

- Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting

  - FFS for details, capabilities will be needed for feature list

- When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased

  - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations]

  - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost

- Boost is 0.5 dB

  - FFS on duplex modes

  - For information only: HD-FDD/FD-FDD support is indicated per band

    - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting

- Rel-18 and Rel-19 power boosting schemes can be simultaneously supported

  - FFS for details

- PC3 ACLR requirements apply for power boosted transmissions

- Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

In Rel-18, power boosting was introduced for pi/2-BPSK and QPSK TN UEs under the following conditions as specified in Clause 6.2.4.

ΔPPowerBoost is defined as 1dB for power class 3 and 0.5dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

-	0 dB otherwise;

- The Rel-18 TN UE power boosting feature is only applicable for DFT-s-OFDM with pi/2 BPSK or QPSK. Once enabled, the MOP for PC3 is increased by 1dB for inner RB allocations.

Subject to the signalling, different MPR requirements may apply for different RB allocations within the inner region, as specified in Clause 6.2.2.

When ΔPPowerBoost is a positive value and the UE supports powerBoosting-pi2BPSK-QPSK-r18

-	An enhanced power inner allocation region within the inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low + P1  ≤  RBStart  ≤  RBStart,High - P1

Where P1 = MIN{12,CEIL(2+NRB/25)}

-	When a UE that supports powerBoosting-pi2BPSK-QPSK-r18 but does not support powerBoosting-pi2BPSK-QPSK-Modified-r18 an RB allocation that belongs to the inner region but is outside the enhanced power inner region, the applicable MPR from Table 6.2.2-x is increased by the value of ΔPPowerBoost.

- For a TN UE supporting powerBoosting-pi2BPSK-QPSK-r18, it’s mandatory to boost its power for the enhanced power inner allocation region. For the rest of the inner region, it’s optional to boost its power if the UE does not support powerBoosting-pi2BPSK-QPSK-Modified-r18.

Based on the discussions in previous meetings, the consensus seems to be that the Rel-18 TN power boosting feature can be ported to NTN.

- When porting Rel-18 TN power boosting feature to NTN, the same signalling scheme and the related transmitter power requirements can be reused. For Rel-19 NTN power boosting, new UE capabilities are needed.

Except for the Rel-18 TN power boosting scheme, 0.5dB power boost is considered for all modulations and waveforms in Rel-19, owing to the potential reduction of insertion loss for HD-FDD UEs. The MPR for edge RB allocations is typically limited by SEM, which is unlikely benefited from the reduction of insertion loss.

- Consider to increase the MPR by the amount of the boost for edge RB allocations for DFT-s-OFDM and QPSK and pi/2 BPSK, in addition to CP-OFDM and DFT-s-OFDM 16QAM and 64QAM. New UE capability (e.g. powerBoosting-Modified-r19) can be introduced to indicate whether the increase is not needed, similar to the function of powerBoosting-pi2BPSK-QPSK-Modified-r18.

The Rel-18 and Rel-19 power boosting capabilities may be indicated by the UE simultaneously. However, the feasibility of 1.5dB combined boost has not been properly studied. Hence, the total amount of boost for the same RB allocation should not exceed 1dB. Otherwise, it’s questionable whether PC3 ACLR requirement can still apply.

- Both Rel-18 and Rel-19 power boosting capabilities may be indicated by the UE simultaneously. However, the max amount of boost for the same RB allocation should not exceed 1dB.

As per the Rel-18 TN power boosting scheme, the SAR issue for a UE is addressed by limiting the UL duty cycle. Similar side condition should be defined for the Rel-19 scheme.

- For the Rel-19 NTN power boosting scheme, the amount of boost is 0.5dB, applicable for PC3, and the percentage of uplink symbols transmitted in a certain evaluation period is less than 90%.

Considering the very limited study on feasibility, it’s premature to decide whether the Rel-19 power boosting scheme is applicable for FD-FDD.

- The Rel-19 NTN power boosting scheme is applicable for HD-FDD. FD-FDD is FFS.

## 2.2 UL capacity enhancements

RAN4#115 Agreements:

- As a starting point, re-use DMRS-bundling requirement values and side conditions

  - Add “No UE specific and common TA updates” to side conditions

  - FFS for further updates based on inputs in next meeting

- UE needs to be able to indicate OCC2/4 feature and/or OCC duration support for GSO deployment scenario only, for NGSO deployment scenario only, or for both GSO and NGSO deployment scenario

  - Further check and optimize alignment with RAN1 capabilities

In RAN1’s LS to RAN4 [2], the agreement in RAN1 is as follows:

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

- FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.

- As per RAN1’s LS, it’s assumed that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

The OCC length is either 2 or 4 slots, which is shorter than the max duration (8 or 16 slots) considered for DMRS bundling.

- The duration of one OCC group is either 2 or 4 slots, which is shorter than the max duration for DMRS bundling. Reusing the same requirement for DMRS bundling will not tighten the phase continuity requirement for a UE.

Consider the OCC length, the phase continuity requirement can be simplified.

- Based on the requirement for DMRS bundling, the phase continuity requirement for OCC can be defined as follows and the same side conditions apply.

Table 2.2-1: Maximum allowable phase difference for OCC2 and OCC4

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p |
| --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK, 16QAM, 64QAM | [25] degrees |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. |  |  |

## 2.3 DL coverage enhancements

RAN4#115 Agreements:

- Discuss updates to sync raster due to impact of cell search delay with 160ms SSB periodicity in next meeting

- Encourage companies to provide detailed proposal on sync raster update

  - Note: also 3 MHz is being introduced in rel-19

If the density of the sync raster is reduced, it might impose some restrictions on network deployment on certain frequency bands. Alternatively, the frequencies for sync raster can be divided into a primary group and a secondary group. The network is encouraged to use the frequency in the primary group to transmit SSB and the UE will first search on the primary group, too.

- Consider to divide the SS raster entries for a band into multiple groups such as: a primary group and a secondary group. The network prioritises the entries in the primary group for transmitting SSB, and the UE will first search the primary group.

# 3	Conclusion

We have studied the remaining open issues and made the following observations and proposals:

Observation 1:The Rel-18 TN UE power boosting feature is only applicable for DFT-s-OFDM with pi/2 BPSK or QPSK. Once enabled, the MOP for PC3 is increased by 1dB for inner RB allocations.

Observation 2:For a TN UE supporting powerBoosting-pi2BPSK-QPSK-r18, it’s mandatory to boost its power for the enhanced power inner allocation region. For the rest of the inner region, it’s optional to boost its power if the UE does not support powerBoosting-pi2BPSK-QPSK-Modified-r18.

Proposal 1:When porting Rel-18 TN power boosting feature to NTN, the same signalling scheme and the related transmitter power requirements can be reused. For Rel-19 NTN power boosting, new UE capabilities are needed.

Proposal 2:Consider to increase the MPR by the amount of the boost for edge RB allocations for DFT-s-OFDM and QPSK and pi/2 BPSK, in addition to CP-OFDM and DFT-s-OFDM 16QAM and 64QAM. New UE capability (e.g. powerBoosting-Modified-r19) can be introduced to indicate whether the increase is not needed, similar to the function of powerBoosting-pi2BPSK-QPSK-Modified-r18.

Proposal 3:Both Rel-18 and Rel-19 power boosting capabilities may be indicated by the UE simultaneously. However, the max amount of boost for the same RB allocation should not exceed 1dB.

Proposal 4:For the Rel-19 NTN power boosting scheme, the amount of boost is 0.5dB, applicable for PC3, and the percentage of uplink symbols transmitted in a certain evaluation period is less than 90%.

Proposal 5:The Rel-19 NTN power boosting scheme is applicable for HD-FDD. FD-FDD is FFS.

Observation 3:As per RAN1’s LS, it’s assumed that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Observation 4:The duration of one OCC group is either 2 or 4 slots, which is shorter than the max duration for DMRS bundling. Reusing the same requirement for DMRS bundling will not tighten the phase continuity requirement for a UE.

Proposal 6:Based on the requirement for DMRS bundling, the phase continuity requirement for OCC can be defined as follows and the same side conditions apply.

Table 2.2-1: Maximum allowable phase difference for OCC2 and OCC4

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p |
| --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK, 16QAM, 64QAM | [25] degrees |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. |  |  |

Proposal 7:Consider to divide the SS raster entries for a band into multiple groups such as: a primary group and a secondary group. The network prioritises the entries in the primary group for transmitting SSB, and the UE will first search the primary group.

# References

- R4-2508764 Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm, RAN4#115

- R4-2505305 LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, RAN1, RAN4#115