3GPP RAN WG4 Meeting #116																	R4-2511499

Bangalore, India, 25th – 29th August 2025

Agenda item:	3

Source:	Apple

Title:	On nominated channel bandwidth concept in the 3GPP specifications

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding satellite bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. After that more satellite bands were added, such as bands n254, n252. In addition to that there are several ongoing spectrum WIs that will add even more satellite bands.

Most of the satellite bands have additional local requirements, which RAN WG4 captures accordingly as additional band specific requirements. In particular, ETSI has additional requirements for most of the low- and mid-frequency satellite bands, such as L- and S-bands. While capturing and implementing ETSI standards several concerns were raised that some of them are more stringent compared to the existing 3GPP core requirements, especially in-band emission requirements. And while this is typically not a big issue for the NR-based NTN solutions that have large guard bands, some proponents claimed that IOT-based NTN might suffer from high A-MPR in some allocations.

# 2	Nominated channel bandwidth

As mentioned in the Introduction part, existing satellite L- and S-band have additional requirements from ETSI whereupon in some cases ETSI requirements are more stringent than 3GPP core requirements. In turn, RAN WG4 usually has no option but to define A-MPR, which according to some proponents might have quite high values, especially for very specific edge allocations in NB-IOT based NTN deployments. As one of the potential solutions, there were proposals to adopt a concept known as "nominated" channel bandwidth in the ETSI specifications. However, this concept does not exist in the current 3GPP specifications.

There is an incoming LS from ETSI asking 3GPP [2] to investigate the concept of "nominated" channel bandwidth, for which we would like to provide our initial views:

-	Since we are already at the end of Rel-19 and it is not likely for 3GPP to agree on big features that would change the entire paradigm of the 3GPP system. Thus, the "nominated" channel bandwidth, if adopted, should leverage as much as possible existing design and existing features.

-	3GPP has already been working on the feature called extended channel bandwidth, which is effectively same approach as the ETSI "nominated" channel bandwidth.

-	While the "extended" channel bandwidth feature has been developed for NR implicitly aiming at the TN deployments, it does not preclude adoption of the same feature for NTN, at least the NR NTN.

-	As for the IOT-based NTN, we anticipate leveraging the basic approach from the extended channel bandwidth feature, which can be simplified a lot because IOT NTN has only one channel bandwidth of 200kHz.

-	It is also worth noting that while ETSI "nominated" channel bandwidth effectively allows a manufacturer to declare any bandwidth, it is not likely that 3GPP can follow the same approach. Instead, we anticipate that 3GPP would follow the principle similar to the extended channel bandwidth where we agree on a set of extended channel sizes, which a UE can be tested for ensuring certain performance from the network perspective.

-	Last but not least, this concept should be implemented in such a way that it can be applied to the existing and upcoming satellite bands.

# 3	Conclusions

In this discussion paper we have presented our initial views on the concept of nominated channel bandwidth. As explained in the paper, we do not anticipate RAN WG4 to have enough room for a big feature meaning that the nominated channel bandwidth concept, if agreed, should leverage as much as possible existing features and solutions, such as Rel-19 extended channel bandwidth. When and if possible, the corresponding simplifications can be made, which could be the case for the IOT-based NTN technology.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

R4-2509023, "LS on Harmonised Standard for NTN capable UE", ETSI TC SES