# 3GPP TSG-RAN WG4 Meeting # 116													R4-2511434

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson, Sony

Title:			TP for 38.191 backscatter power

Agenda item:		7.22.3.2

Document for:		Approval

# Introduction

In this paper, the TP for backscatter power loss is proposed.

# TP proposal

## 6.2	Transmitter power

### 6.2.1	Device backscatter power

The backscatter power is defined as mean filtered power measured over the duration of the D2R signal, excluding the power around the carrier frequency. The backscatter loss is defined as the difference between the input CW power at the device antenna in dB scale to the backscatter power at the device antenna in dB scale.

The backscatter power is measured with the test metric of EIRP as specified in clause 8.2.1. The minimum requirement on basckscatter loss in Table 6.2.1.1-1 shall be met with the test parameters defined in Annex X, at the peak antenna gain direction declared by the device vendor.

Table 6.2.1.1-1: Maximum allowable backscatter loss

| Operating band | Maximum backscatter loss (dB) |  | CW power at the device antenna (dBm) |
| --- | --- | --- | --- |
| n8 | OOK | 10 | -27 |
|  | BPSK | 6 |  |
|  | OOK | 15 | -10 |
|  | BPSK | 11 |  |

Annex X (normative):

The test configuration for backscattering loss

The test configuration in Table X.1-1 is defined to be used for testing the backscattering loss requirement defined in section 6.2.1