3GPP TSG-RAN WG4 Meeting #116															           R4-2511778

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			5.8.1

Source:	Sony

Title:	WF on A-MPR value of b254

Document for:	Approval

# Topic #1: A-MPR values for b254

#### Issue 1-1: A-MPR values of NS_04N and NS_05N for b254 with 15 kHz SCS

It is agreed to adopt the following A-MPR values:

Table 1-1-1 A-MPR for NS_04N with 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤12 |  | ≤8.5 | ≤6.5 | ≤1 | ≤0.5 | ≤0.5 | ≤1 | ≤6.5 | ≤8.5 | ≤12 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  | ≤ 2 |  |  |  | ≤ 8.5 |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 6 |  |  |  |  |  | ≤ 6 |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 2 |  |  |  |  |  |  |  |  |  |  |  |

Table 1-1-2 A-MPR for NS_05N with 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤15 |  | ≤10.5 | ≤8 | ≤2 | ≤1.5 | ≤1.5 | ≤2 | ≤8 | ≤10.5 | ≤15 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR (dB) | ≤ 11 |  |  |  | ≤ 3.5 |  |  |  | ≤ 11 |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  |  |  | ≤ 8.5 |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 4.5 |  |  |  |  |  |  |  |  |  |  |  |

#### Issue 1-2: A-MPR values of NS_04N and NS_05N for b254 with 3.75 kHz SCS

It is agreed to adopt the following A-MPR values:

Table	1-2-1 A-MPR for NS_04N with 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-9 | 10-38 | 39-41 | 42-44 | 45-47 |
| A-MPR (dB) | ≤ 10.5 | ≤ 5.5 | ≤ 3 | ≤ 0 | ≤ 3 | ≤ 5.5 | ≤ 10.5 |

Table	1-2-2 A-MPR for NS_05N with 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-10 | 11-37 | 38-41 | 42-44 | 45-47 |
| A-MPR (dB) | ≤13.5 | ≤8 | ≤5 | 0 | ≤ 3 | ≤ 5.5 | ≤ 13.5 |

#### Issue 1-3: A-MPR values of NS_11N and NS_12N for b254 with 15 kHz and 3.75 kHz SCS

It is agreed to adopt the following A-MPR values:

Table 1-3-1 A-MPR for NS_11N and NS_12N with 15 kHz and 3.75 kHz SCS

| SCS and number of tones | 3.75kHz | 15 kHz 1-tone | 15 kHz 3-tone | 15 kHz 6-tone | 15 kHz 12-tone |
| --- | --- | --- | --- | --- | --- |
| A-MPR (dB) | 18.0 | 15 | 11 | 8.5 | 4.5 |

#### Issue 2: Measurement bandwidth

It is agreed to allow smaller measurement bandwidth for A-MPR of NS_04N/05N/11N/12N for b254, with adding the following note under the table of requirement of NS_04N/05N/11N/12N for b254.

NOTE:	 As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.