3GPP TSG-RAN4 Meeting #116	R4-2511853

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 on REFSENS requirements for LP-WUS/LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Vivo, Huawei, Murata, ZTE Corporation, Samsung, CATT, Spreadtrum, Sony, CMCC, LG Electronics, OPPO, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding the REFSENS requirements for LP-WUS/LP-WUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the previously agreed table with numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This draft CR is on top of the previously endorsed big draft CR R4-2506882 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ START OF CHANGES ************************************

## <<< START OF CHANGES >>>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRLP-WUS	Reference sensitivity adjustment for specific bands with FDL_low higher than 2400 MHz

ΔRXR,2R	Reference sensitivity adjustment for two antenna ports XR UEs on bands defined in Table 7.3.2-2b

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(GBChannel,low, GBChannel,high)

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF, shift	RF reference frequency for Supplementary Uplink (SUL) bands, the uplink of all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

Gn100post connector	Declared value of the post chipset unit antenna connector gain for band n100, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

Gn101post connector 	Declared value of the post chipset unit antenna connector gain for band n101, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

GBChannel(i)	Minimum guard band defined in clause 5.3.3 of carrier i

GBChannel,low	Minimum guard band defined in clause 5.3.3 for the lowest assigned component carrier in clause 5.3A.3

GBChannel,high	Minimum guard band defined in clause 5.3.3 for the highest assigned component carrier in clause 5.3A.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=i=1jNRBi*2μi for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,LP-WUS	Number of RBs for LP-WUS

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

Pmax,c,AC	Maximum output power defined as the sum of measurement of all antenna connectors

Pmax,c,TABC	Maximum carrier output power defined as the sum of measurement of all TAB connectors

Prated,c,AC	Rated maximum output power defined as the sum of power over all antenna connectors

Prated,c,TABC	Rated maximum output power defined as the sum of power over all TAB connectors

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PREFSENS_SL 	The REFSENS power for Sidelink

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	The lowest RB index of transmitted resource blocks

RBstart_CA	The lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

************************************ END OF CHANGES ************************************

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

************************************ START OF CHANGES ************************************

## 7.3M	Reference sensitivity for LP-WUS/WUR

### 7.3M.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna port, at which the MDR criterion with the side conditions specified in 7.1M shall be met.

### 7.3M.2	Reference sensitivity power level for LP-WUS/WUR

The REFSENS for the LP-WUS/WUR shall be as specified in Table 7.3M.2-1. The reference sensitivity in Table 7.3M.2-1 shall be modified by the amount given in ΔRLP-WUS in Table 7.3M.2-2 for the applicable operating bands.

Table 7.3M.2-1: Reference sensitivity for LP-WUS/LP-WUR

| RX parameter | SCS kHz | Value (dBm) |
| --- | --- | --- |
| REFSENS for UEs supporting [Type1 LR] | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENS for UEs supporting [Type2 LR] | 15 | -102.0 |
|  | 30 | -99.0 |

Table 7.3M.2-2: Reference sensitivity allowance ΔRLP-WUS

| Operating band | ΔRLP-WUS (dB) |
| --- | --- |
| n41, n77, n78, n79 | 1.0 |
| n104 | 3.0 |

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

************************************ END OF CHANGES ************************************