3GPP TSG-RAN WG4 Meeting #116	R4-2512823

# Bangalore, India,  25th August – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0041 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the S-band NTN HPUE power back-off simulations to TR 38.863 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, MediaTek, Qualcomm, Oppo, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the NTN high-power UE feature will enable power classes higher than 23dBm for the NTN bands. However, since some of the NTN bands have additional regulatiry emission requirements, additional analysis is required to check required power back-off values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Annex is added to capture power back-off simulations results based on the input from the following contributions: R4-2509854 (MediaTek) R4-2511400 (Qualcomm) R4-2510521 (OPPO) R4-2511269 (Nokia) R4-2509537 (Apple) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.863 will not have technical analysis regarding power back-off needed to meet existing regulatory emission requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex F:
Power back-off simulation results for the S-band

## F.1	PC3 power class

### F.1.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

## F.2	PC2 power class

### F.2.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### F.2.1.1	NS_24N

##### F.2.1.1.1	Company A (MediaTek/R4-2509854)

###### F.2.1.1.1.1	Simulation assumptions

###### F.2.1.1.1.2	Simulation results

Table below summarises proposed A-MPR values for the existing A-MPR regions, i.e., same regions that already exist for the PC3 power class.

Table F.2.1.1.1.2-1: Proposed A-MPR regions and values

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer | Outer/ Inner | Outer/ Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.0 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 13.3 | ≤ 7.1 |  | ≤ 8.5 | ≤ 21 | ≤ 13.5 |  |
| CP-OFDM QPSK | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.1 | ≤ 15.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13.9 | ≤ 7.3 |  | ≤ 8.5 | ≤ 22.6 | ≤ 16.1 |  |

##### F.2.1.1.2	Company B (Qualcomm/R4-2511400)

###### F.2.1.1.2.1	Simulation assumptions

###### F.2.1.1.2.2	Simulation results

In the table below, A-MPR regions A to C are the existing regions from the PC3 power, and region D is a new A-MPR reion.

Table F.2.1.1.2.2-1: Proposed A-MPR regions and values

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  | Region D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  | ≤ 3.24 | A8 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 10MHz | Fc = 1985 | >5.4 |  | A4 |  | ≥ 4.5 | A8 |  |  |  |  |  |  |
| 10MHz | 1985 ≤ Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 | ≥3.06 <5.76 | ≤1.44 | A8 |
| 15MHz | Fc = 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.04 |  | A5 | ≥4.04 <8.64 | >1.44 | A6 | ≥4.04 <8.64 | ≤1.44 | A8 |
| 20MHz | Fc = 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 | ≥ 4.68 < 12.96 | ≤2.16 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 | ≥ 5.58 < 11.52 | ≤1.44 | A8 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 14 | ≤ 8 | ≤ 6 | ≤ 8.5 | ≤ 22 | ≤ 13 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 5 |

##### F.2.1.1.3	Company C (R4-2510521/OPPO)

###### F.2.1.1.3.1	Simulation assumptions

The simulation assumptions are as follows:

-	PC2 power class with 1Tx

-	5,10, 15 and 20MHz channels residing at the higher edge of the uplink operating band n256 and keeping 5MHz protecting guard gap between the higher edge of the channel bandwidth and the highest edge of the uplink operating band.

-	CP-OFDM and DFT-s-OFDM

###### F.2.1.1.3.2	Simulation results

To assess PC2 power back-off values and potential A-MPR regions, PC2 CP-OFDM QPSK power back-off values are compared to the corresponding results obtained for the PC3 power class under the existing A-MPR regions. Based on that the following observations are made:

-	The PC3 power back-off simulations results have more margins compared the current region frame and related AMPR values in existing specification.

-	Some regions for PC2 are extended compared with PC3 due to higher output power, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz.

-	The power back-off value of A6 region in F.2.1.1.3.2-1 is 5dB smaller than the value of A6 region in F.2.1.1.3.2-2.

Figure F.2.1.1.3.2-1: A-MPR regions for the for 5MHz channel at 1997.5 < Fc ≤ 2002.5 and 10MHz channel at 1985 < Fc ≤ 1995

Figure F.2.1.1.3.2-2: A-MPR regions for the for 10MHz at 1995 < Fc ≤ 2000, 15MHz and 20MHz.

According to the simulation results, A-MPR regions for NS_24N can reuse same regions as the existing PC3 regions. However, some region ranges should be extended compared to PC3, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz. It is also necessary to a new region name for the A6 region in Figure F.2.1.1.3.2-2 for 10MHz, 15MHz, and 20MHz to distinguish with the A6 region in F.2.1.1.3.2-1 for 5MHz and 10MHz, i.e., A8 replace A6.

In the figures below further results are presented for NS_24N with PC2 10MHz at Fc=2000MHz under different modulations with CP-OFDM and DFT-s-OFDM waveforms, whereupon the simulation results are based on fixed bias PA.

Figure F.2.1.1.3.2-3:	AMPR for PC2 10MHz CP-OFDM under different modulations.

Figure F.2.1.1.3.2-4:	AMPR for PC2 10MHz DFT-s-OFDM under different modulations.

Table F.2.1.1.3.2-1 proposes A-MPR regions and power back-off values, which are based on the existing PC3 regions.

Table F.2.1.1.3.2-1: Proposed A-MPR values and regions based on existing PC3 A-MPR regions

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <3.78 |  | A5 | ≥3.78 <8.64 | >1.44 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| CP-OFDM QPSK | 16 | 12 | 11 | 9.5 | 26 | 21 | 8.5 |
| CP-OFDM 16QAM | 16 | 12 | 11 | 9.5 | 26 | 21 | 8.5 |
| CP-OFDM 64QAM | 16 | 12 | 11 | 9.5 | 26 | 21 | 8.5 |
| DFT-s-OFDM pi/2 BPSK | 15 | 11 | 10 | 8.5 | 26 | 20 | 7.5 |
| DFT-s-OFDM QPSK | 15 | 11 | 10 | 8.5 | 26 | 20 | 7.5 |
| DFT-s-OFDM 16QAM | 15 | 11 | 10 | 8.5 | 26 | 20 | 7.5 |
| DFT-s-OFDM 64QAM | 15 | 11 | 10 | 8.5 | 26 | 20 | 7.5 |

If we use the modified regions as shown in Table F.2.1.1.3.2-2 for PC2 AMPR, we can see that the regions of A1, A4 and A5 are extended, but regions A2, A3 and A8 are reduced.

Table F.2.1.1.3.2-2: Proposed values based on modified AMPR regions

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | > 2.16 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | > 1.44 | A1 | > 2.7 | >1.08 ≤ 1.44 | A2 | ≤2.7 | ≤ 1.44 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | > 3.24 | A1 | ≥ 5.4 | > 2.16 ≤ 3.24 | A2 | < 5.4 | ≤ 3.24 | A3 |
|  |  |  |  |  |  | ≤ 2.16 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥ 5.4 |  | A5 | < 3.42 |  | A5 | ≥ 3.42 <5.4 | >1.08 | A8 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | < 5.94 |  | A5 | ≥ 5.94 <8.64 | >1.44 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <7.74 |  | A5 | ≥7.74 <11.52 | >1.44 | A8 |

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A8 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner |  | Outer |
| CP-OFDM QPSK /16QAM/64QAM | ≤ 16 | ≤ 10 | ≤ 10 | ≤ 9.5 | ≤ 26 | ≤ 12 | ≤ 17 | ≤ 8.5 |
| DFT-s-OFDM pi/2 BPSK /QPSK/16QAM/64QAM | ≤ 15 | ≤ 9 | ≤ 9 | ≤ 8.5 | ≤ 26 | ≤ 11 | ≤ 16 | ≤ 7.5 |

##### F.2.1.1.4	Company D (R4-2511269/Nokia)

###### F.2.1.1.4.1	Simulation assumptions

The following simulation assumptions are made:

-	Operating band n256 (UL 1980 MHz – 2010 MHz)

-	5MHz, 10 MHz, 15 MHz, and 20 MHz channel bandwidth

-	Power class PC2

-	WOLA processing

-	I/Q image -28 dBc (default according to 38.101-1, Table 6.4.2.3-1) for other modulations than 256QAM

-	For 256QAM the I/Q image is -33.7 dBc as in MPR simulations.

-	Carrier leakage -28 dBc (default according to 38.101-1, Table 6.4.2.3-1)

-	CIM3 -60 dBc

-	CIM5 -70 dBc

-	Channel noise 38.5 dBc

-	PA noise -75 dBm/MHz

-	Only additional spurious emission mask from NS_24 is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

-	Phase noise and channel noise are only added for QAM256, following LTE 256-QAM MPR simulations.

###### F.2.1.1.4.2	Simulation results

The following figures provide simulation results for different channel sizes and different locations for PI/2 BPSK, DFT-s-OFDM QPSK and CP-OFDM QPSK.

Figure F.2.1.1.4.2-1: 5MHz results at Fc=1997.5MHz

Figure F.2.1.1.4.2-2: 5MHz results at Fc=2002.5MHz

Figure F.2.1.1.4.2-3: 10MHz results at Fc=1995MHz

Figure F.2.1.1.4.2-4: 10MHz results at Fc=2000MHz

Figure F.2.1.1.4.2-5: 15MHz results at Fc=2000MHz

Figure F.2.1.1.4.2-6: 20MHz results at Fc=1995MHz

Based on conducted simulations the follow A-MPR values and regions can be defined. The simulation results based on the A-MPR table of PC3, but some regions are different from PC3. Empty cells indicate that MPR is already enough according to our results. In some cases, region B needs to be slightly extended to cover all the allocations that need A-MPR, which also affects region C. This extension is likely needed due to the tail of the CIM3 peak. An alternative way to cover the A-MPR allocations is to decrease the upper LCRB limit of region C or create a new region. We also found that in case of PC2 the region A at 5MHz needs to be extended a bit due to regrowth of the allocation. and regions B and C need to be modified for 15MH. The reason we think why 5MHz regions for PC1 can be the same as PC3’s is because the linearity of the PC1 PAs is better due to the difference of form factors from PC2 and PC2 PAs.

Table F.2.1.1.4.2-1: Proposed A-MPR regions and values for PC2

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.06 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 9 | 5 | 0.5 | 2.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM QPSK | 10 | 5 | 2 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 16 QAM | 11 | 5 | 2.5 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 64 QAM | 11 | 5 |  | 4.5 | 17 | 11.5 |  |
| DFT-s-OFDM 256 QAM | 11 | 5 |  |  | 17 | 11.5 |  |
| CP-OFDM QPSK | 11.5 | 5.5 | 3.5 | 5 | 17.5 | 12.5 |  |
| CP-OFDM 16 QAM | 11.5 | 5.5 | 3.5 | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 64 QAM | 11.5 | 5.5 |  | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 256 QAM | 11.5 |  |  |  | 17.5 | 12.5 |  |

##### F.2.1.1.5	Company E (R4-2509537/Apple)

###### F.2.1.1.5.1	Simulation assumptions

PC2 power class simulations are conducted with the common assumptions as presented below:

-	Tx power: PC2 (+26dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_24

For the sake of completeness, the channel bandwidts and the corresponding center frequencies are summarised below:

-	5MHz: Fc=2002.5MHz, Fc=1997.5MHz, Fc=1992.5MHz

-	10MHz: Fc=2000MHz, Fc=1995MHz, Fc=1985MHz

-	15MHz: Fc=1997.5MHz, Fc=1987.5MHz

-	20MHz: Fc=1995MHz, Fc=1990MHz

###### F.2.1.1.5.2	Simulation results

Figure F.2.1.1.5.2-1, Figure F.2.1.1.5.2-2 and Figure F.2.1.1.5.2-3 present results for the 5MHz channel with the PC2 power class. The results are in line with the existing 5MHz channel A-MPR values for PC3 as defined by NS_24. For instance, while the large allocation with CP-OFDM and PC3 would need up to 13dB A-MPR, in case of PC2 the maximum power baack-off increases up to 15dB. Nevertheless, it is worth noting that large power back-off is needed only for the full channel allocations, which are not likely to be used with the NTN deployments.  Once the 5MHz channel is shifted father away from the upper band edge, one can notice an improvement in the required power back-off values. For instance, for the 5MHz channel residing at 1992.5MHz A-MPR would be need only for the large allocations, whereas small allocations do not even need MPR.

Figure F.2.1.1.5.2-1: Power back-off results for 5MHz at Fc=2002.5MHz.

Figure F.2.1.1.5.2-2: Power back-off results for 5MHz at Fc=1997.5MHz.

Figure F.2.1.1.5.2-3: Power back-off results for 5MHz at Fc=1992.5MHz.

Next, Figure F.2.1.1.5.2-4 to F.2.1.1.5.2-6 present the 10MHz simulation results, from which as anticipated we can see even higher power back-off required for the full channel and upper band edge allocations. In fact, the same trend can be observed further for the 15MHz and 20MHz channel bandwidths presented at figures below.

Figure F.2.1.1.5.2-4: Power back-off results for 10MHz at Fc=2000MHz

Figure F.2.1.1.5.2-4: Power back-off results for 10MHz at Fc=1995MHz

Figure F.2.1.1.5.2-5: Power back-off results for 10MHz at Fc=1985MHz

Figure F.2.1.1.5.2-6: Power back-off results for 15MHz at Fc=1997.5MHz

Figure F.2.1.1.5.2-7: Power back-off results for 15MHz at Fc=1987.5MHz

Figure F.2.1.1.5.2-8: Power back-off results for 20MHz at Fc=1995MHz

Figure F.2.1.1.5.2-9: Power back-off results for 20MHz at Fc=1990MHz

## F.3	PC1.5 power class

### F.3.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

## F.4	PC1 power class

### F.4.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### F.4.1.1	NS_24N

##### F.4.1.1.1	Company A (R4-2511269/Nokia)

###### F.4.1.1.1.1	Simulation assumptions

The following simulation assumptions are made:

-	Operating band n256 (UL 1980 MHz – 2010 MHz)

-	5MHz, 10 MHz, 15 MHz, and 20 MHz channel bandwidth

-	Power class PC1

-	WOLA processing

-	I/Q image -28 dBc (default according to 38.101-1, Table 6.4.2.3-1) for other modulations than 256QAM

-	For 256QAM the I/Q image is -33.7 dBc as in MPR simulations.

-	Carrier leakage -28 dBc (default according to 38.101-1, Table 6.4.2.3-1)

-	CIM3 -60 dBc

-	CIM5 -70 dBc

-	Channel noise 38.5 dBc

-	PA noise -75 dBm/MHz

-	Only additional spurious emission mask from NS_24 is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

-	Phase noise and channel noise are only added for QAM256, following LTE 256-QAM MPR simulations.

###### F.4.1.1.1.2	Simulation results

The following figures provide simulation results for different channel sizes and different locations for PI/2 BPSK, DFT-s-OFDM QPSK and CP-OFDM QPSK. It should be noted that power back-off is presented only for those cases when the corresponding value exceeds existing MPR margins.

Figure F.4.1.1.1.2-1: 5MHz results at Fc=1997.5MHz

Figure F.4.1.1.1.2-2: 5MHz results at Fc=2002.5MHz

Figure F.4.1.1.1.2-3: 10MHz results at Fc=1995MHz

Figure F.4.1.1.1.2-4: 10MHz results at Fc=2000MHz

Figure F.4.1.1.1.2-5: 15MHz results at Fc=2000MHz

Figure F.4.1.1.1.2-6: 20MHz results at Fc=1995MHz

Based on the presented results PC1 A-MPR regions and the corresponding A-MPR values can be as summarised below in Table F.4.1.1.1.2-1.

Table F.4.1.1.1.2-1: Proposed A-MPR regions and values for PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.86 |  | A5 | ≥4.86 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.76 |  | A5 | ≥5.76 <11.52 | >1.44 | A6 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 8 | 6.5 |  | 1.5 | 21.5 | 16.5 |  |
| DFT-s-OFDM QPSK | 9 | 8 |  | 3 | 21.5 | 16 |  |
| DFT-s-OFDM 16 QAM | 9.5 | 8 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 64 QAM | 9.5 | 8.5 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 256 QAM | 9.5 | 8.5 |  |  | 21.5 | 16 |  |
| CP-OFDM QPSK | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 16 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 64 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 256 QAM | 10 | 10 |  |  | 21.5 | 16.5 |  |

##### F.4.1.1.2	Company B (R4-2510521/OPPO)

###### F.4.1.1.2.1	Simulation assumptions

Simulation assumptions are as follows:

-	PC1 power class with 4Tx

-	5,10, 15 and 20MHz channels residing at the higher edge of the uplink operating band n256 and keeping 5MHz protecting guard gap between the higher edge of the channel bandwidth and the highest edge of the uplink operating band.

-	CP-OFDM and DFT-s-OFDM

###### F.4.1.1.2.2	Simulation results

Figure F.4.1.1.2.2-1 and Figure F.4.1.1.2.2-2 present a comparison of the NS_24 power back-off results for 5MHz (Fc=2002.5) and 10MHz (Fc=2000MHz) under PC2 1Tx, PC1.5 2Tx and PC1 4Tx with CP-OFDM QPSK. The simulation results are based on fixed bias PA.

Figure F.4.1.1.2.2-1 AMPR for 5MHz CP-OFDM QPSK under different power classes

Figure F.4.1.1.2.2-2 AMPR for 10MHz CP-OFDM QPSK under different power classes

In the AMPR simulation for PC2 1Tx, PC1.5 2Tx and PC1 4Tx, the same PC2 PA with fixed bias is assumed for these three power classes. From Figure F.4.1.1.2.2-1 and Figure F.4.1.1.2.2-2 it can be seen that:

-	The PC2 and PC1.5 have the similar AMPR region frame and values due to the maximum output power of each PA for PC2 and PC1.5 is 26dBm;

-	The AMPR region frame for PC1 has more margin than PC2 and PC1, since the maximum output power of each PA for PC1 is 25dBm.

Therefore, PC1 can use same AMPR regions as PC2. Table F.4.1.1.2.2-1 proposes power back-off values, which are based on the existing PC3 regions.

Table F.4.1.1.2.2-1: Proposed PC1 4Tx AMPR values based on PC3 A-MPR regions

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| CP-OFDM QPSK | ≤ 20.5 | ≤ 16 | ≤ 16 | ≤ 15 | ≤ 31 | ≤ 23 | ≤ 10 |
| CP-OFDM 16QAM | ≤ 20.5 | ≤ 16 | ≤ 16 | ≤ 15 | ≤ 31 | ≤ 23 | ≤ 10 |
| CP-OFDM 64QAM | ≤ 20.5 | ≤ 16 | ≤ 16 | ≤ 15 | ≤ 31 | ≤ 23 | ≤ 10 |
| DFT-s-OFDM pi/2 BPSK | ≤ 18.5 | ≤ 15 | ≤ 15 | ≤ 13.5 | ≤ 31 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM QPSK | ≤ 18.5 | ≤ 15 | ≤ 15 | ≤ 13.5 | ≤ 31 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM 16QAM | ≤ 18.5 | ≤ 15 | ≤ 15 | ≤ 13.5 | ≤ 31 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM 64QAM | ≤ 18.5 | ≤ 15 | ≤ 15 | ≤ 13.5 | ≤ 31 | ≤ 22 | ≤ 9 |

If the modified regions are used as shown in Table F.2.1.1.3.2-2, regions A1, A4 and A5 are extended, but regions A2, A3 and A8 are reduced as shown in Table F.4.1.1.2.2-2 below

Table F.4.1.1.2.2-2: PC1 4Tx AMPR values based on modified A-MPR regions as PC2

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A8 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner |  | Outer |
| CP-OFDM QPSK | ≤ 20.5 | ≤ 15 | ≤ 15 | ≤ 10 | ≤ 31 | ≤ 15.5 | ≤ 23 | ≤ 10 |
| CP-OFDM 16QAM | ≤ 20.5 | ≤ 15 | ≤ 15 | ≤ 10 | ≤ 31 | ≤ 15.5 | ≤ 23 | ≤ 10 |
| CP-OFDM 64QAM | ≤ 20.5 | ≤ 15 | ≤ 15 | ≤ 10 | ≤ 31 | ≤ 15.5 | ≤ 23 | ≤ 10 |
| DFT-s-OFDM pi/2 BPSK | ≤ 18.5 | ≤ 14 | ≤ 14 | ≤ 9 | ≤ 31 | ≤ 14 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM QPSK | ≤ 18.5 | ≤ 14 | ≤ 14 | ≤ 9 | ≤ 31 | ≤ 14 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM 16QAM | ≤ 18.5 | ≤ 14 | ≤ 14 | ≤ 9 | ≤ 31 | ≤ 14 | ≤ 22 | ≤ 9 |
| DFT-s-OFDM 64QAM | ≤ 18.5 | ≤ 14 | ≤ 14 | ≤ 9 | ≤ 31 | ≤ 14 | ≤ 22 | ≤ 9 |